RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Control Systems | Max. Marks | : | 100 |
| Subject Code | : | EE52/EE502 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Obtain i) F-V and ii) F-I analogous circuit of the following mechanical system. Fig.1(a) | CO1 | (12) |
|  | b) | What are the advantages of having feedback in a control system? Explain with at least 2 examples with respective block diagrams. | CO1 | (08) |
| 2. | a) | Obtain the transfer function of the following circuit diagram. Fig.2(a) | CO1 | (10) |
|  | b) | Obtain i) F-V and ii) F-I analogous circuit of the following mechanical system. Fig.2(b) | CO1 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | Define: i) Peak Overshoot ii) Peak Time with respect to an under damped system. Hence derive the expression for the same. | CO2 | (10) |
|  | b) | Consider a unity feedback system with a closed t.f. Determine the open-loop transfer function. G(s). Show that the steady-state error with unit ramp input is given as . | CO2 | (10) |
| 4. | a) | Obtain the transfer function from following circuit diagram. | CO2 | (12) |
|  | b) | Find step, ramp and parabolic error coefficients and their corresponding steady-state error for unity feedback system having transfer function. | CO2 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | State and explain RH criterion of stability. What are its limitations? | CO3 | (08) |
|  | b) | Plot the root locus of the system with OLTF , Show that part of root locus of system is a circle. | CO3 | (12) |
| 6. | a) | How the roots of the characteristic equation are related to stability? | CO3 | (06) |
|  | b) | Sketch the root locus plot and find stability for a unity feedback system whose OLTF is given as G(s)=. Find the range of K for which the system is stable. | CO3 | (14) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Define: - Resonant peak - Resonant frequency - Bandwidth - Cut off rate. | CO4 | (08) |
|  | b) | A unity negative feedback system is characterized by, . Find the values of K and so that and | CO4 | (12) |
| 8. | a) | Sketch the Nyquist plot and determine the stability of the following open loop transfer function of unity feedback control system . | CO3 | (10) |
|  | b) | For , sketch Nyquist plot and determine the stability of the system . Calculate GM of the system. | CO3 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Obtain the transfer function of the system whose Bode plot is as shown below. Fig.9(a) | CO4 | (10) |
|  | b) | Obtain the transfer function of a lag compensator for m = - 400 and fm = 5KHz. Assume C = 10nF. Also find the corner frequencies. | CO5 | (10) |
| 10. | a) | Obtain the Bode plot of a unity negative feedback system, whose OLTF is . Hence, find the stability of the system. | CO4 | (12) |
|  | b) | Obtain the transfer function of a lead compensator for m = 350 and fm = 4kHz. Assume C=20nF. | CO5 | (08) |

****************