RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

SEMESTER END EXAMINATIONS – JANUARY 2017

| Course & Branch | : | Electrical & Electronics Engineering. | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Control Systems | Max. Marks | : | 100 |
| Subject Code | : | EE502 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT-I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | What is the significance of presence of feedback in control system? Explain with at least two example. | CO1 | (06) |
|  | b) | Define transfer function? And Briefly explain the need for modeling of a system. | CO1 | (06) |
|  | c) | Obtain the transfer function of the following system f(t) | CO1 | (08) |
| 2 | a) | Obtain the i) Force - current analogy ii) Force – Voltage analogy of the following mechanical system. motor. | CO1 | (12) |
|  | b) | Derive transfer function of armature controlled separately excited DC servo motor. | CO1 | (08) |
|  |  | UNIT-II |  |  |
| 3. | a) | Obtain the transfer function using Mason’s gain reduction formula for fig shown in 3a) . | CO2 | (08) |
|  | b) | The closed loop transfer function of a second order system is given by T(s)=25/(S2+6S+25). Determine the damping ratio, natural frequency of oscillations, rise time, settling time and peak overshoot if subjected to unit step input. Also calculate expression for it’s output response. | CO2 | (08) |
|  | c) | For a system G(s)H(s)=K/[S2(S+2)(S+3)]. Find the value of K to limit steady state error to 10, when input to system is 1+10t+(40/2)t2. | CO2 | (04) |
| 4. | a) | Find the C/R using block diagram reduction technique of the system shown in fig 4a) also verify using signal flow graph. | CO2 | (10) |
|  | b) | Open loop transfer function of unity feedback system is G(S)=K/[(1+TS)S] ,where K and T are constants. Determine factor by which gain ‘K’ should be multiplied so that overshoot of unit step response be reduced from 75% to 25%. | CO2 | (10) |
|  |  | UNIT-III |  |  |
| 5. | a) | A negative unity feedback system is having OLTF. Find the range of K from RH criterion for which system stable. | CO3 | (08) |
|  | b) | A system having OLTF , oscillates with frequency 2 rad/sec. Find values of ‘’and ‘p’ if system is stable. | CO3 | (12) |
| 6. | a) | Plot the root locus of the system with OLTF . | CO3 | (10) |
|  | b) | The closed loop TF of positional control system is given by. Determine, i) The range of K for which system is stable. ii) What should be the upper limit and lower limit of K if all the closed loop poles are required to be to the left of line | CO3 | (10) |
|  |  | UNIT-IV |  |  |
| 7. | a) | Using Nyquist stability criterion, investigate the closed loop stability of a negative feedback control system whose open loop transfer function is given by G(S)H(S)=100(1+5s)/[s4(s+1)]. | CO4 | (12) |
|  | b) | Consider G(S)H(S)=1/[s(1+T1s)(1+T2s)]. Draw the polar plot. | CO4 | (08) |
| 8. | a) | Using Nyquist stability criterion, investigate the closed loop stability of a negative feedback control system whose open loop transfer function is given by G(S)H(S)=10/[s(s+1)(s+2)], hence determine the range of value of K for stability. | CO4 | (12) |
|  | b) | Consider G(S)H(S)=1/[s2(1+T1s)]. Draw the polar plot. | CO4 | (08) |
|  |  | UNIT-V |  |  |
| 9. | a) | Obtain the transfer function of lag compensator, for and. Assume. Also find the corner frequencies. | CO5 | (08) |
|  | b) | Obtain the transfer function of the system whose Bode plot is as shown below. | CO4 | (12) |
| 10. | a) | Obtain the Bode plot of unity negative feedback system, whose OLTF is . Hence find the stability of the system. | CO4 | (14) |
|  | b) | Explain the need for designing of a compensator with one practical example. | CO4 | (06) |

****************