RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS –  AUGUST 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Control Systems | Max. Marks | : | 100 |
| Subject Code | : | EE52/EE502 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT-I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Obtain the transfer function of the following mechanical system. Fig.1(a) | CO1 | (10) |
|  | b) | What are the advantages of having feedback over an open loop, in a control system?. Explain with at least 2 examples with respective block diagrams. | CO1 | (10) |
| 2. | a) | Obtain the transfer function of the following system. If the inductance 'L' is replaced with another resistor, whether output leads or lags? Justify your answer. Fig. 2(a) | CO1 | (08) |
|  | b) | Obtain the transfer function for the following system. Also draw, i) F-V and ii) F-I analogous circuit of the following mechanical system given in Fig. 2(b). Fig. 2(b) | CO1 | (12) |
|  |  | UNIT- II |  |  |
| 3. | a) | For a unity feedback system find (i) type of the system (ii) error coefficients (iii) steady state error for input . | CO1 | (08) |
|  | b) | What is meant by steady-state error? Derive an expression for steady-state error of a unity negative feedback system. | CO2 | (04) |
|  | c) | A second order system is represented by the transfer function, . A step input of 10Nm is applied to system and results are, - % - The steady state value of output is 0.5 radians. Find the values of J,B and K. | CO2 | (08) |
| 4. | a) | Reduce the following block diagram to a single transfer function. Fig. 4(a) i) Fig. 4(a) | CO2 | (08) |
|  | b) | Derive an expression for time response 'c(t)' of a standard 2nd order under damped system. | CO2 | (08) |
|  | c) | Define the Stable System? If a closed loop TF is having zero at -2 and poles at -3, +5, whether system is Stable? Justify with its time response behavior. | CO3 | (04) |
|  |  | UNIT- III |  |  |
| 5. | a) | State and explain RH criterion of stability and its limitations? Hence Examine the stability of the given equations using Routh’s stability criterion: - ii) | CO3 | (08) |
|  | b) | Plot the root locus of the system with OLTF , Show that part of root locus of system is a circle. | CO3 | (12) |
| 6. | a) | How the change in system gain affects stability of the system? Briefly explain relevant mathematics with an example. | CO3 | (08) |
|  | b) | A system having OLTF , oscillates with frequency 2 rad/sec. Find values of ‘’and ‘p’ if system is stable. | CO3 | (12) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Can we interpret the time domain behavior of the system based on frequency response? Justify your answer with relevant relations. | CO4 | (06) |
|  | b) | Sketch the Nyquist plot and determine the stability of the following open loop transfer function of unity feedback control system . Comment on stability. | CO4 | (14) |
| 8. | a) | Check the closed loop stability of the following open loop system on GH plane using conformal mapping technique by considering entire right half of s-plane as reference. | CO4 | (10) |
|  | b) | Using Nyquist stability criterion, investigate the stability of closed loop system whose OLTF is . | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | For a unity feedback system, the open loop transfer function is G(s)=0.25(1+0.5s)s(1+2s)(1+4s). Draw the log magnitude plot and the phase plot and determine: - the gain cross over frequency - the phase margin - the phase cross over frequency - the gain margin. | CO4 | (12) |
|  | b) | Explain the procedure for lag compensation. | CO5 | (08) |
| 10. | a) | Design a lead compensator for a unity feedback system with open loop transfer function G(s)=ks(s+1)(s+5) to satisfy the following specifications : - Kv >50 (ii) Phase Margin is >20 | CO5 | (12) |
|  | b) | Explain the concept of phase margin & gain margin? | CO4 | (08) |

****************