| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

M S RAMAIAH INSTITUTE OF TECHNOLOGY

(AUTONOMOUS INSTITUTE, AFFILIATED TO VTU)

BANGALORE – 560 054

MAKEUP EXAMINATIONS – JULY 2016

| Course & Branch | : | B.E. – Electrical & Electronics Engg. | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Linear Integrated Circuits | Max. Marks | : | 100 |
| Subject Code | : | EE604 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Typical standard values of Resistors and Capacitors sheet allowed. - Suitably assume the missing data if any.

|  |  |  | UNIT – I |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) |  | Two signals which each ranging from 0.1 to 1 V are to be summed and amplified by a factor of 5. Using LF353 op-amp, design a suitable non inverting summing circuit. Draw the circuit diagram with component values mentioned on it | C01 | (08) |
|  | b) |  | For a inverting amplifier as shown in Fig. 1(b) determine: i) AV ii) V0 iii) IL iv) I0 v) Input impedance vi) Output impedance vii) What will be the output voltage of the inverting amplifier if the input voltage changes from 2V to 5V? (choose open loop input impedance = 0.3 MΩ; open loop output impedance = 75Ω; open loop gain = 50,000, IB = 500 nA). Fig. 1(b) | C01 | (08) |
|  | c) |  | With a neat sketch explain the technique adopted in 741 for nulling the input offset voltage and current. | C01 | (04) |
| 2 | a) |  | A LF353 BIFET Op-Amp is used in a non inverting amplifier with a voltage gain of 100. Calculate the typical output voltage that would result from common mode input with a peak level of 100mV. (From LF353 data sheet CMRRTyp = 100 dB). | C01 | (04) |
|  | b) |  | Determine the output voltage for the open loop difference amplifier amplifier circuit given in fig. 2.(b), if i) Vin1 = 5 µV d.c. and Vin2 = -7 µV d.c. (both are d.c. input voltage) ii) Vin1 = 7.07 mV (rms) and Vin2 = 14.14 mV (rms) (sine wave a.c. input). Draw the input and output waveform for both the cases. Op-amp used is 741 op-amp with a supply voltage of + 15V (open loop gain of op-amp is 2,00,000). 2.(b) | C01 | (08) |
|  | c) |  | With a neat sketch explain the working of non inverting summing amplifier. Derive an expression for its output voltage. Also discuss the design steps involved in designing the circuit using 741 op-amp and BIFET LF353 op-amp . | C02 | (08) |
|  |  |  | UNIT – II |  |  |
| 3. | a) |  | Using a 741 op-amp, design a high Zin non inverting amplifier to operate with a +36V power supply. The load resistance is 12 K, the lower cut off frequency is to be 150 Hz and the voltage gain is to be 7. Draw the circuit with component values mentioned on it. | C03 | (10) |
|  | b) |  | The non saturating precision half wave rectifier has to rectify (pass) only positive half cycle of the input voltage. using bipolar Op-amp design a suitable circuit which produces a 5V peak output from a sine wave input with a peak value of 0.5V and the frequency of 1 MHz. Use supply voltage of + 15V. Draw the circuit with component values mentioned on it. | C03 | (06) |
|  | c) |  | With a neat sketch explain the working of precision rectifier peak detector. | C02 | (04) |
| 4. | a) |  | With a neat sketch explain the working for capacitor coupled voltage follower. Write the steps involved in designing the circuit. | C02 | (05) |
|  | b) |  | An op-amp dead zone circuit with a 5V peak input is to have unity gain and is to pass the upper half of the positive half cycle of input waveform. i) Design a suitable circuit using bipolar op-amp. Specify the op-amp and the diodes if the signal frequency ranges from 1 KHz to 33 KHz ii) If a + 15V supply is used with the circuit, modify the circuit to derive the reference voltage from the supply iii) Make the dead zone circuit adjustable between 1/3rd and 2/3rd of the peak output voltage by suitable modification. Draw the circuit diagrams for (i), (ii) and (iii) separately with component values mentioned on it. | C03 | (15) |
|  |  |  | UNIT – III |  |  |
| 5. | a) |  | Design a second order high pass active filter with cut-off frequency of 1KHz using 741 op-amp. | CO2 | (06) |
|  | b) |  | With a circuit diagram and waveform explain the principle of operation of triangular generator using op-amp and derive the expression for frequency of oscillation. | CO3 | (10) |
|  | c) |  | Mention the advantages of active filters over passive filters. | CO2 | (04) |
| 6. | a) |  | Design an astable multivibrator to have 4KHz output with an amplitude of ±14V with ±15V supply using 741 opamp. | CO2 | (06) |
|  | b) |  | Design a second order low pass active filter using 741 op-amp to have cut-off frequency of 5KHz. Draw the circuit. | CO2 | (06) |
|  | c) |  | With a circuit diagram and waveform explain the principle of operation of RC phase oscillator using op-amp and derive the expression for frequency of oscillation. | CO2 | (08) |
|  |  |  | UNIT – IV |  |  |
| 7. |  | a) | Design a non inverting voltage level detector to have VUT = 12 and VLT = 8V. Assume that + Vsat = + 15V. Sketch the circuit with component values mentioned on it. | C03 | (05) |
|  |  | b) | With a neat sketch explain the working of a window detector. | C02 | (05) |
|  |  | c) | Discuss the effect of stray capacitance on op-amp circuit stability. Write the equation to determine the value of stray capacitance that might produce circuit instability. Also discuss the methods for compensating stray capacitance effects. | C02 | (10) |
| 8. |  | a) | With a neat sketch show how a voltage level detector with independent adjustment of hysteresis and center voltage can be used to design a battery voltage level monitoring system. Explain the operation of battery voltage level monitoring system. | C02 | (10) |
|  |  | b) | Using a 709 op-amp, design an inverting amplifier to have AV = 50 and VS = 50 mV. Select suitable compensating components. Refer fig. 8(b)(i) and 8(b)(ii) for selecting components for the circuit designed (From 709 data sheet IB(max) = 200 nA) Fig 8(b)(i) fig. 8(b)(ii) | C02 | (06) |
|  |  | c) | List at least four precaution that should be observed for operational amplifier circuit stability. | C02 | (04) |
|  |  |  | UNIT – V |  |  |
| 9. |  | a) | Design an integrator having gain limiting frequency fa = 200 Hz and highest input signal frequency fb = 2000 Hz using 741 op-amp. Draw the circuit diagram with component values mentioned on it. | C03 | (10) |
|  |  | b) | Briefly explain the typical performance parameters for a voltage regulator. | C02 | (05) |
|  |  | c) | With a neat sketch explain how a pulse stretcher can be designed with a monostable multivibrator using 555 timer. | C02 | (05) |
| 10. |  | a) | With a neat sketch explain the working of low voltage DC voltmeter and low voltage AC voltmeter. | C02 | (12) |
|  |  | b) | Design an astable multivibrator using 555 timer for a free running frequency of 3 KHz. The multivibrator is to have a duty cycle of 50%. Draw the circuit diagram with component values mentioned on it. | C03 | (08) |

********************