RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering. | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Linear Integrated Circuits | Max. Marks | : | 100 |
| Subject Code | : | EE54/EE604 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Use of standard Resistor and Capacitor value list is permitted. - Suitably assume the missing data, if any

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Explain how slew rate of an op-amp can introduce distortion in the output waveform. | CO1 | (05) |
|  | b) | Briefly explain the following: (i) Output voltage range (ii) Input voltage range (iii) CMRR (iv) SVRR (v) output offset voltage. | CO1 | (10) |
|  | c) | An operational amplifier circuit with a closed loop gain of 100 has a common mode output of 5µV when the common input is 5mV. Determine the common mode rejection ratio. | CO1 | (05) |
| 2. | a) | With a neat sketch explain the working of an inverting amplifier. Derive an expression for gain of the inverting amplifier. Write equations for determining suitable values for each resistor (i) using bipolar op-amp (ii) using BIFET op-amp. | CO2 | (10) |
|  | b) | The difference of two input signals is to be amplified by a factor of 37. Each input has amplitude of approximately 50 mV. Using an LF353 op-amp, design a suitable circuit and calculate the differential and common mode input resistance. Draw the circuit diagram mentioning the component value on it. | CO2 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | With a neat sketch explain the working of high input impedance capacitor coupled non inverting amplifier. Derive an expression for the input impedance of the circuit. Write equations for calculating component values using 741 and LF353 op-amp. | CO2 | (10) |
|  | b) | A + 1.5V signal is to be sampled by a sample and hold circuit. The holding time is to be 700µs, and the output accuracy is to be approximately 0.3%. LM108 op-amps are to be used, and the available FET has a 25Ω channel resistance and a 300nA gate – source reverse leakage current. Design the circuit and determine the minimum acquisition time. Draw the circuit with component values mentioned on it. (For LM108 data sheet IB (max) = 2 nA) | CO2 | (10) |
| 4. | a) | With a neat circuit and waveforms explain the working of voltage follower peak detector. Write the equations for calculating the capacitor value and the op-amp minimum slew rate. How is it different from precision rectifier peak detector. | CO2 | (12) |
|  | b) | A capacitor coupled difference amplifier is to have two input voltage with each range from 200 mV to 2V. The lowest signal frequency is 300 Hz, the load resistance is 3.3 kΩ and the voltage gain is to be 5. Design a suitable circuit using BIFET LF353 op-amp. Draw the circuit with component values mentioned on it. | CO2 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | Bring out the difference between active filter and passive filter. | CO2 | (03) |
|  | b) | (i) Design a Butterworth second order High pass filter for a lower cutoff frequency of 5 kHz and a pass band gain of 1.586. Draw the circuit with component values mentioned on it. (ii) Draw the theoretical frequency response curve on a semilog sheet for the filter designed in (i). (iii) Modify the filter designed in (i) for a new cutoff frequency of 4 kHz. | CO2 | (12) |
|  | c) | Design a Wein bridge oscillator for an oscillating frequency of 2 kHz. Draw the circuit with component values mentioned on it. | CO2 | (05) |
| 6. | a) | With a neat sketch and waveforms, explain the working of a triangular wave generator. Discuss the steps required to design the circuit. Write the modified circuit diagram and waveforms, show how the triangular wave generator can be modified to produce a saw tooth waveform. | CO2 | (10) |
|  | b) | Design an active notch filter for a frequency of 60 Hz. Sketch the circuit with component values mentioned on it. | CO2 | (05) |
|  | c) | What do you mean by frequency stability in an oscillator? Explain its significance. | CO2 | (05) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Design an inverting Schmitt trigger with VUT = 4V and VLT = 2V. Use 741 Op-amp with a supply voltage of + 15V. Draw the circuit with component values mentioned on it and also draw the expected input and output waveform and the hysteresis curve for the circuit designed. | CO2 | (10) |
|  | b) | Sketch a circuit to show Zin mod method of frequency compensation and explain its operation. State the application of the circuit and write equation for the feedback factor. | CO3 | (10) |
| 8. | a) | With a neat circuit diagram, input and output waveform and hysteresis curve, explain the working of window detector. Mention its applications. | CO2 | (10) |
|  | b) | i) Calculate the slew rate limited cutoff frequency for a voltage follower circuit using a 741 op-amp if a peak of sine wave output is to be 5V. ii) Determine the maximum peak value of the sinusoidal output voltage that will allow the 741 voltage follower circuit to operate at the 800 KHz unity gain cutoff frequency. iii) Calculate the maximum peak value of sine wave output voltage that can be produced by the amplifier which is having an upper cutoff frequency of 8 KHz. (From 741 data sheet: typical slew rate = 0.5 V/µs, unity gain bandwidth = 800 KHz) | CO3 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Design a differentiator to differentiate an input signal that varies in frequency from 10 Hz to about 1 KHz. Sketch the circuit with component values mentioned on it | CO2 | (10) |
|  | b) | (i) Briefly explain the typical performance parameters for a voltage regulator. (ii) With a neat sketch, explain the operation of adjustable positive voltage regulator. | CO5 | (10) |
| 10. | a) | With a neat sketch explain the working of DAC using current to voltage converter op-amp. | CO4 | (10) |
|  | b) | Design an astable multivibrator for a duty cycle of 50%. The free running frequency is 1 kHz. Draw the circuit with component values mentioned on it. | CO5 | (10) |

****************