RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

SEMESTER END EXAMINATIONS –  MAY/JUNE 2017

| Course & Branch | : | B.E: Electrical & Electronics Engineering. | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Linear Integrated Circuits | Max. Marks | : | 100 |
| Subject Code | : | EE604 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Use of standard resistor and capacitor values list is permitted. - Suitable assume the missing data if any..

|  |  | UNIT-I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Briefly explain the ideal electrical characteristics of an op-amp. | CO1 | (05) |
|  | b) | i) Two signals that each range from 0.1V to 1V are to be summed and amplified by a factor of 5. Using an LF353 op-amp, design a suitable non inverting summing circuit. Sketch the circuit with component values mentioned on the circuit. Use + 15V supply. ii) If the circuit designed in section (i) has to work as an averaging circuit with an amplification factor of 5, what should be the value of the resistor connecting inverting terminal to ground? (keep all other parameters and component values same.) | CO5 | (07) |
|  | c) | Explain the following terms: - Input bias current ii) Power supply rejection ratio iii) Common mode rejection ratio iv) Slew rate. | CO1 | (08) |
| 2. | a) | With a neat sketch explain the working of a non-inverting amplifier. Derive an expression for its voltage gain. Discuss the design procedure to design a non-inverting amplifier using: (i) bipolar op-amp and (ii) BIFET op-amp. | CO2 | (08) |
|  | b) | A 1V signal source with a source resistance of 47 KΩ has to be connected to a 10 KΩ load resistor. Calculate the load voltage i) when the load is directly connected to the source [refer fig. 2(b)(i)] ii) when the voltage follower using 741 Op-amp is connected in between the load and the source. [refer fig. 2(b) (ii)] (From 741 data sheet: typical open loop gain = 200000; typical open loop input impedance = 2 MΩ; typical open loop output impedance = 75Ω) Fig. 2(b) (i) 2(b)(ii) | CO5 | (12) |
|  |  | UNIT- II |  |  |
| 3. | a) | Show how a half wave non saturating precision rectifier can be combined with a summing circuit to produce a full wave precision rectifier. Draw the voltage waveform throughout the circuit and derive an expression for its output voltage to show that full wave rectification is performed. | CO2 | (10) |
|  | b) | Using a LF353 BIFET op-amp, design a high Zin capacitor coupled non inverting amplifier to have a lower cutoff frequency of 200 Hz. The input and output voltage are to be 15mV and 3V respectively and the minimum load resistance is 12 KΩ. Use a supply voltage of +15V. Draw the circuit diagram with component values mentioned on it. | CO5 | (10) |
| 4. | a) | A capacitor coupled difference amplifier is to have two input voltage with each range from 200 mV to 2V. The lowest signal frequency is 300 Hz, the load resistance is 3.3 kΩ and the voltage gain is to be 5. Design a suitable circuit using BIFET LF353 op-amp. | CO5 | (06) |
|  | b) | With a neat sketch and waveforms, explain the working of voltage follower peak detector. Write the equations for calculating the capacitor values for the peak detector circuit. | CO2 | (08) |
|  | c) | With a neat sketch explain the working of non-saturating precision half wave rectifier. Draw the input voltage waveform, output voltage waveform and waveform at the output pin of op-amp. | CO2 | (06) |
|  |  | UNIT-III |  |  |
| 5. | a) | Design a second order Butter worth low pass filter having upper cutoff frequency of 2KHz. Plot the frequency response of the designed filter. | CO2 | (10) |
|  | b) | Draw the circuit of a square wave generator and explain. Draw the various waveforms and derive an expression for period T. | CO3 | (10) |
| 6. | a) | Design a wide band pass filter with fL= 100Hz and fH= 2KHz and a pass band gain of 4. Draw the frequency response plot of this filter. Also calculate the value of Q for the filter. | CO2 | (10) |
|  | b) | i) With a circuit diagram explain the working of anopamp phase shift oscillator. ii) Design a phase shift oscillator to have a cutoff frequency of 500Hz. | CO3 | (10) |
|  |  | UNIT-IV |  |  |
| 7. | a) | Design an non inverting Schmitt trigger with VUT = 5V and VLT = 2V. Use 741 Op-amp with a Supply voltage of + 15V. Draw the circuit with component values mentioned on it. Also draw the expected input, output waveform and Plot the hysteresis curve for the circuit designed. | CO3 | (10) |
|  | b) | Show how feedback in an non inverting amplifier can produce instability. | CO3 | (05) |
|  | c) | Explain why manufacturer's recommended compensating components are normally suitable for both inverting and non inverting amplifiers. State the precautions that should be taken with low gain inverting amplifier circuits. | CO3 | (05) |
| 8. | a) | i) Calculate the slew rate limited cutoff frequency for a voltage follower circuit using a 741 op-amp if a peak of sine wave output is to be 5V ii) Determine the maximum peak value of the sinusoidal output voltage that will allow the 741 voltage follower circuit to operate at the 800 KHz unity gain cutoff frequency. iii) Calculate the maximum peak value of sine wave output voltage that can be produced by the amplifier which is having an upper cutoff frequency of 8 KHz. (From 741 data sheet: typical slew rate = 0.5 V/µs, unity gain bandwidth = 800 KHz) | CO1 | (10) |
|  | b) | Design an non inverting Schmitt trigger with VUT = 1V and VLT = 1V. Use 741 op-amp with a supply voltage of + 15V. Draw the circuit with component values mentioned on it. | CO3 | (05) |
|  | c) | With the help of a neat sketch and waveforms, demonstrate the effect of noise on the performance of an open loop zero crossing detector. | CO3 | (05) |
|  |  | UNIT- V |  |  |
| 9. | a) | With a neat circuit diagram explain the working of a low voltage DC voltmeter. | CO3 | (10) |
|  | b) | With a neat sketch waveforms and frequency response curve, explain the working of an integrator. Derive an expression for its output voltage. Discuss the procedure for designing the circuit. | CO2 | (10) |
| 10. |  | Write a note on the following: i) Phase locked loop ii) Universal active filter iii) Current to voltage converter iv) voltage to current converter with grounded load. | CO4 | (20) |

****************