RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS –  JANUARY 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering. | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Linear Integrated Circuits | Max. Marks | : | 100 |
| Subject Code | : | EE54/EE604 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Use of standard Resistor and Capacitor value list is permitted. - Suitably assume the missing data, if any.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Draw and explain the block diagram of an op-amp. Also explain the function of each block. | CO1 | (07) |
|  | b) | A 741 operational amplifier has a + 15 V supply. Calculate typical time required for the op-amp output to move from its negative extreme to its positive extreme. | CO1 | (05) |
|  | c) | Draw and explain the equivalent circuit and ideal transfer curve of an op-amp. | CO1 | (08) |
| 2. | a) | With a neat sketch explain the working of an non inverting amplifier. Derive an expression for gain of the inverting amplifier. Write equations for determining suitable values for each resistor (i) using bipolar op-amp and (ii) using BIFET op-amp. | CO2 | (10) |
|  | b) | The difference of two input signals is to be amplified by a factor of 50. Each input has amplitude of approximately 50 mV. Using 741 Op-amp design a suitable circuit such that it gives approximately equal input resistance at the two input terminal and provides output voltage nulling. Sketch the circuit indicating the component values Use + 15V supply. | CO2 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | A capacitor coupled non inverting amplifier is to have a +30V supply, a voltage gain of 50, an output amplitude of 3V, a lower cut off frequency of 50 Hz, upper cutoff frequency of 10KHz and a load resistance of 10 KΩ. Design a suitable circuit using a 741 op-amp. Draw the circuit diagram with component values mentioned on it. Also draw the input waveform, output of the op-amp waveform (usually termed as V0¦) and output of the circuit waveform (V0) when (i) The bottom most resistor of the circuit is directly connected to ground (ii) A capacitor is connected in between the bottom most resistor and ground. | CO2 | (12) |
|  | b) | With a neat circuit and waveforms, explain the working of op-amp sample and hold circuit. Discuss the holding time and acquisition time for the same and write the equations for determining capacitor size and minimum acquisition time. | CO2 | (08) |
| 4. | a) | With a neat sketch explain the working of capacitor coupled voltage follower. Write the steps involved in designing the circuit. Also explain why the capacitor coupled voltage follower's input impedance is much smaller than that of direct coupled voltage follower. | CO2 | (08) |
|  | b) | Using bipolar op-amp with Vcc = + 15V, design the high input impedance precision full wave rectifier circuit. The input peak voltage is to be 1V and no amplification is to occur. Sketch the circuit diagram indicating component values . | CO2 | (08) |
|  | c) | Discuss the advantages of a precision rectifier over an ordinary diode rectifier circuit. | CO2 | (04) |
|  |  | UNIT- III |  |  |
| 5. | a) | (i) Design a First order low pass filter to have a cut off frequency of 1 KHz with a pass band gain of 2. Draw the circuit diagram with component values on it. (ii) On a semi log sheet, plot the frequency response curve of the low pass filter designed in (i). (iii) Using frequency scaling technique, modify low pass filter designed in (i) to have a new cutoff frequency of 1.6 KHz. | CO2 | (12) |
|  | b) | With a neat sketch explain the working of Wien bridge oscillator. | CO2 | (08) |
| 6. | a) | Show how a high pass filter and a low pass filter be combined with a summing circuit to produce wide band reject filter. Explain its operation. Draw its frequency response plot. Also discuss the design procedure for designing the circuit. | CO2 | (08) |
|  | b) | Design the triangular wave generator so that frequency of oscillation f0 = 10 KHz and V0(P-P) = 10V. Use 741 op-Amp with a supply voltage of + 15V. Draw the circuit diagram with component values mentioned on the circuit. Also sketch the expected waveform. | CO2 | (12) |
|  |  | UNIT-IV |  |  |
| 7. | a) | With the help of suitable sketch, briefly explain the following compensation methods: (i) Phase lag compensation (ii) Phase lead compensation. | CO3 | (10) |
|  | b) | Design a non inverting voltage level detector to have VUT = 12 and VLT = 8V. Assume that + Vsat = + 15V. Draw the circuit diagram with component values mentioned on it. Draw the expected input and output waveform for the circuit designed. Also draw the hysteresis curve. | CO2 | (10) |
| 8. | a) | Explain the meaning of op-amp circuit stability. . | CO3 | (05) |
|  | b) | Using an LM108 op-amp, design an inverting amplifier to amplify a 100 mV signal by a factor of 3. Select suitable frequency compensation. (For LM108 IB = 2 nA). | CO3 | (05) |
|  | c) | With a neat sketch show how a voltage level detector with independent adjustment of hysteresis and center voltage can be used to design a battery voltage level monitoring system. Explain the operation of battery voltage level monitoring system. | CO2 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | With a neat sketch, waveforms and frequency response curve, explain the working of a differentiator. Derive an expression for its output voltage. Also mention the steps required to design the differentiator. | CO2 | (12) |
|  | b) | Briefly explain the following terms with respect to voltage regulators: - Dropout voltage ii) Temperature stability iii) Ripple rejection iv) line regulations. | CO5 | (08) |
| 10. | a) | i) With a neat sketch and waveforms, explain the working of 555 timer as a monostable multivibrator. ii) Suggest suitable modifications need to be made to the monostable multivibrator such that it works as a frequency divider circuit. iii) Monostable mutivibrator is to be used as a divided by 2 network (frequency divider). The frequency of the input trigger signal is 2KHz. If the value of C = 0.01F, what should be the value of RA? | CO5 | (12) |
|  | b) | With a neat sketch explain the working of low voltage d.c. voltmeter. | CO4 | (08) |

****************