RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS –  JANUARY 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Linear Integrated Circuits | Max. Marks | : | 100 |
| Subject Code | : | EE54 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | With a neat sketch explain potential divider biasing applied to a direct coupled voltage follower. Write equations to calculate the bias resistor values. Also explain why a resistor is required to be placed in the feedback path. | CO2 | (08) |
|  | b) | Determine the output voltage for the open loop difference amplifier circuit given in fig. 1.(b), if - Vin1 = 5 µV dc and Vin2 = -7 µV dc (both are dc input voltage) - Vin1 = 7.07 mV (rms) and Vin2 = 14.14 mV (r.m.s) (sine wave ac input) Draw the input and output waveform for both the cases. Op-amp used is 741 op-amp with a supply voltage of + 15V (open loop gain of op-amp is 2,00,000) Fig.1.(b) | CO2 | (07) |
|  | c) | A LF353 BIFET Op-Amp is used in a non-inverting amplifier with a voltage gain of 100. Calculate the typical output voltage that would result from common mode input with a peak level of 100mV. (From LF353 data sheet CMRRTyp = 100 dB). | CO1 | (05) |
| 2. | a) | A direct coupled non inverting amplifier with a + 15V supply is to produce maximum possible output voltage and is to have a voltage gain of 60dB. The amplitude of the input voltage is to be 5 mV AC. Using 741 Op-Amp, design a suitable circuit. Ensure that the circuit is suitably compensated for IBRB drop. Draw the circuit diagram with the component values mentioned on it. Also calculate the input and the output impedance for the circuit designed (From 741 data sheet: typical open loop gain = 200000; typical open loop input impedance = 2MΩ; typical open loop output impedance = 75Ω). Also draw the input and output waveforms for the circuit designed. | CO2 | (10) |
|  | b) | With a neat circuit diagram explain the operation of non-inverting summing amplifier. Derive an expression for its output voltage and explain how it can be used as an averaging circuit. Also discuss the design steps involved in designing the non-inverting summing amplifier using i) 741 op-amp and ii) BIFET op-amp. | CO2 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | Design a high input impedance capacitor coupled voltage follower using a 741 op-amp. The lower cutoff frequency for the circuit is to be 50 Hz and the load resistance is RL = 3.9 K ohms. Also determine the minimum theoretical input impedance of the circuit. Draw the circuit diagram with component values mentioned on it. | CO2 | (10) |
|  | b) | A capacitor coupled non-inverting amplifier is to have a +28V supply, a voltage gain of 10, an output amplitude of 5V, a lower cut off frequency of 60 Hz, upper cutoff frequency of 10KHz and a load resistance of 3.3 KΩ. Design a suitable circuit using a 741 op-amp. Draw the circuit diagram with component values mentioned on it. Also draw the input waveform, output of the op-amp waveform (usually termed as V0¦) and output of the circuit waveform (V0) when - The bottom most resistor of the circuit connected to ground directly - A capacitor is connected in between the bottom most resistor and ground. | CO2 | (10) |
| 4. | a) | A 300 KHz signal with peak amplitude of 500 mV is to be full wave rectified and amplified to 5V. Design a suitable circuit using BIFET LF353 op-amp with a supply voltage of + 15V. Sketch the circuit with component values mentioned on it. | CO2 | (07) |
|  | b) | The non-saturating precision half wave rectifier has to rectify (pass) only positive half cycle of the input voltage. Using bipolar Op-amp, design a suitable circuit which produces a 5V peak output from a sine wave input with a peak value of 0.5V and the frequency of 1MHz. Use supply voltage of + 15V. Draw the circuit with component values mentioned on it. | CO2 | (05) |
|  | c) | With neat circuit diagram and waveform, explain the working of high input impedance precision full wave rectifier. Derive an expression for its output voltage. | CO2 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | Using a bipolar op-amp, design a plus and minus precision clipping circuit to clip the output at a level adjustable from 2V to 5V. The reference voltage is to be derived from the op-amp + 12 V power supply. Draw the circuit with component values mentioned on it. | CO2 | (10) |
|  | b) | With a neat sketch explain the working of Wein bridge oscillator. Discuss the steps involved in designing the oscillator. | CO2 | (05) |
|  | c) | Design the phase shift oscillator for an oscillating frequency of 200 Hz | CO2 | (05) |
| 6. | a) | An op-amp dead zone circuit with a 5V peak input is to have unity gain and is to pass the upper half of the positive half cycle of input waveform. i) Design a suitable circuit using bipolar op-amp. Specify the op-amp and the diodes if the signal frequency ranges from 1 KHz to 33 KHz. ii) If a + 15V supply is used with the circuit, modify the circuit to derive the reference voltage from the supply. iii) Make the dead zone circuit adjustable between 1/3rd and 2/3rd of the peak output voltage by suitable modification. iv)Draw the circuit diagrams for (i), (ii) and (iii) separately with component values mentioned on it. | CO2 | (12) |
|  | b) | Show how a high pass filter and a low pass filter can be combined with a summing circuit to produce wide band reject filter. Explain its operation. Draw its frequency response plot. Also discuss the design procedure for designing the circuit. | CO2 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | With a neat circuit diagram and waveforms explain the working inverting voltage level detector with different UTP and LTP. Plot the hysteresis curve. Also discuss the design steps involved in designing the circuit. | CO2 | (10) |
|  | b) | A battery charger control circuit is to be designed to monitor the voltage level of a 12V battery. When the battery’s voltage drops below 10.5V, the battery should be connected to the charger. When the battery voltage reaches 13.5V, it should be disconnected from the charger. The reference voltage is to be -15V. Assume +Vsat = + 13V. Design a suitable circuit. Sketch the complete circuit with component values mentioned on it. | CO2 | (10) |
| 8. | a) | - Using a 709 Op-amp, design an inverting amplifier as shown in Fig.8(a) to have AV =300 and Vs =1mV (IB(max) for 709=200nA) - Determine the load capacitance that might create instability in the inverting amplifier in the inverting amplifier circuit designed in (i) - Calculate the feedback capacitance required to compensate for a load capacitance of 0.1F for the circuit designed in (i) - If an additional resistor R = 470 ohms is included in the circuit between output of the op-amp and the junction of load capacitor and R2, calculate the feedback capacitance required to compensate for CL = 0.1F. Fig. 8(a) | CO3 | (10) |
|  | b) | Define gain bandwidth product and discuss its application to inverting and non-inverting amplifiers. | CO3 | (05) |
|  | c) | With the help of suitable sketch, briefly explain the following compensation methods: - Phase lag compensation ii. Phase lead compensation. | CO3 | (05) |
|  |  | UNIT- V |  |  |
| 9. | a) | With a neat sketch explain the working of DAC using op-amp current to voltage converter. | CO4 | (10) |
|  | b) | i) With a neat sketch and waveforms, explain the working of 555 timer as a monostable multivibrator. ii) Suggest suitable modifications that need to be made to the monostable multivibrator such that it works as a frequency divider circuit. iii) Monostable mutivibrator is to be used as a divided by 2 network (frequency divider). The frequency of the input trigger signal is 2KHz. If the value of C = 0.01F, what should be the value of RA? | CO5 | (10) |
| 10. | a) | With a neat sketch explain how the current to voltage converter with floating load can be used as a diode match finder, zener diode tester and light emitting diode tester. | CO4 | (08) |
|  | b) | Design an astable multivibrator for a duty cycle of 50%. The free running frequency is 1 kHz. Draw the circuit with component values mentioned on it. | CO5 | (08) |
|  | c) | Briefly explain the typical performance parameters for a voltage regulator. | CO5 | (04) |

****************