RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS  JANUARY – FEBRUARY 2021

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Linear Integrated Circuits | Max. Marks | : | 100 |
| Course Code | : | EE54(O) | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Explain the following terms with respect to 741 op-amp: - Output Offset range - Input Voltage range - CMRR - SVRR. | CO1 | (08) |
|  | b) | The difference of two input signals is to be amplified by a factor of 37. Each input has amplitude of approximately 50 mV. Using an LF353 op-amp, design a suitable circuit and calculate the differential and common mode input resistance. | CO2 | (08) |
|  | c) | Calculate the minimum input impedance and typical output impedance of a 741 operational amplifier employed as a voltage follower. (From 741 data sheet: minimum open loop gain = 50,000 and typical open loop gain = 2,00,000; minimum open loop input impedance = 0.3MΩ; open loop output impedance = 75Ω) | CO1 | (04) |
| 2. | a) | With a neat circuit diagram of inverting summing amplifier, derive an expression for its output voltage. Also, explain how it can be modified as an averaging circuit. | CO2 | (08) |
|  | b) | Design a non-inverting amplifier using LF353 BIFET op-amp for a voltage gain of approximately 66. The signal amplitude is to be 15mV. Sketch the circuit with component values mentioned on it. | CO2 | (07) |
|  | c) | List the characteristics of an ideal op-amp. | CO1 | (05) |
|  |  | UNIT – II |  |  |
| 3. | a) | With a neat sketch explain the working for capacitor coupled voltage follower. Write the steps involved in designing the circuit. Also explain why the capacitor coupled voltage follower's input impedance is much smaller than that of direct coupled voltage follower. | CO2 | (08) |
|  | b) | With a neat sketch show how non saturating precision half wave rectifier can be modified to obtain two independent half-wave rectified output. Draw the waveform and explain the circuit operation. | CO2 | (08) |
|  | c) | Discuss the advantages of a precision rectifier over an ordinary diode rectifier circuit. | CO2 | (04) |
| 4 | a) | With a neat sketch, explain the working of capacitor coupled difference amplifier. Mention the design step using i) 741 op-amp and ii) BIFET LF353 Op-amp. | CO2 | (10) |
|  | b) | Using bipolar op-amp with Vcc = + 15V, design the high input impedance precision full wave rectifier circuit. The input peak voltage is to be 1V and no amplification is to occur. Sketch the circuit diagram with component values mentioned on it. | CO2 | (08) |
|  | c) | What do you mean by precision diode? | CO2 | (02) |
|  |  | UNIT – III |  |  |
| 5. | a) | - Design a First order low pass filter to have a cut off frequency of 1kHz with a pass band gain of 2. Draw the circuit diagram with component values on it. - On a semi log sheet, plot the frequency response curve of the low pass filter designed in (i). Using frequency scaling technique, modify low pass filter designed in (i) to have a new cutoff frequency of 1.6 kHz. | CO2 | (12) |
|  | b) | With neat sketch and waveforms explain the working of a triangular wave generator. Also derive an expression for its frequency of oscillation. | CO2 | (08) |
| 6. | a) | A 5V, 10 kHz square wave from a signal source with a resistance of 100 Ω is to have its positive peak clamped precisely at ground level. Tilt on the output is not to exceed 1% of the peak amplitude of the wave. Design a suitable op-amp circuit using a supply of +12V. Draw the circuit with component value mentioned on it. | CO2 | (08) |
|  | b) | With a neat sketch explain the working of RC phase shift oscillator. Discuss the steps required to design the oscillator. | CO2 | (08) |
|  | c) | Bring out the difference between active filter and passive filter. | CO2 | (04) |
|  |  | UNIT – IV |  |  |
| 7 | a) | Design an inverting Schmitt trigger with VUT = 5V and VLT = 3V. Use 741 Op-amp with a Supply voltage of + 15V. Draw the circuit with component values mentioned on it and also draw the expected input and output waveform and the hysteresis curve for the circuit designed. | CO2 | (10) |
|  | b) | Sketch a circuit to show Zin mod method of frequency compensation and explain its operation. State the application of the circuit and write equation for the feedback factor. | CO3 | (10) |
| 8. | a) | With a neat sketch and waveforms demonstrate the effect of noise on a open loop zero crossing detector. Suggest the method to reduce the impact of noise on the output voltage of ZCD. | CO2 | (10) |
|  | b) | With suitable sketches show how feedback in an non inverting and inverting amplifiers can produce instability. | CO3 | (10) |
|  |  | UNIT – V |  |  |
| 9. | a) | With a neat sketch and waveforms explain the working of a differentiator. Also derive an expression for its output voltage. | CO2 | (08) |
|  | b) | Design an astable multivibrator using 555 timer for a free running frequency fo = 1.45kHz, the positive pulse width tc= 0.421ms and negative pulse width td = 0.269ms. Sketch the circuit with component values mentioned on it. | CO5 | (06) |
|  | c) | With a neat sketch explain the working of current to voltage converter. | CO4 | (06) |
| 10. | a) | With a neat sketch, explain the working of voltage to current converter with grounded load. Also, derive an expression for the load current IL. | CO4 | (08) |
|  | b) | Design an adjustable voltage regulator to satisfy the following specifications: Output voltage V0 = 3 to 6V Output current I0 = 1.0A Use LM317 voltage regulator. Sketch the circuit diagram with component values mentioned on it.(for LM317 Iadj = 100A) | CO5 | (08) |
|  | c) | Define the following terms with respect to voltage regulators: - Dropout voltage - Temperature stability - Ripple rejection - Line regulation. | CO5 | (04) |

****************