RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS – JULY/AUGUST 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering | Semester | : | V / VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Linear Integrated Circuits | Max. Marks | : | 100 |
| Subject Code | : | EE54/EE604 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | A 1V signal source with a source resistance of 47 KΩ has to be connected to a 10 KΩ load resistor. Calculate the load voltage. - When the load is directly connected to the source [refer fig. 1(a)(i)] - When the voltage follower using 741 Op-amp is connected in between the load and the source. [refer fig. 1(a) (ii)] (From 741 data sheet: typical open loop gain = 200000; typical open loop input impedance = 2 MΩ; typical open loop output impedance = 75Ω) Fig. 1(a) (i) 1(a)(ii) | CO2 | (09) |
|  | b) | List the characteristics of an ideal op-amp. | CO1 | (06) |
|  | c) | Briefly explain the following: (i) Output voltage range (ii) Input voltage range (iii) CMRR (iv) SVRR (v) output offset voltage. | CO1 | (05) |
| 2. | a) | Draw the input and output waveform for an inverting amplifier with - an input voltage of 0.5 V (rms value) and gain of 10 - an input voltage of 2 V D.C and gain of 10 - an input voltage of 1 V (rms value) and gain of 10 - an input voltage of 2 V peak A.C. value and gain of 10 Consider a 12 V supply voltage and 741 op-amp for the inverting amplifier. | CO2 | (08) |
|  | b) | Two signals which each ranges from 0.2 to 1V are to be directly summed. Using a + 15V supply voltage and 741 op-amp, design a suitable non-inverting summing circuit. Draw the circuit with component values mentioned on it. | CO2 | (07) |
|  | c) | With a neat sketch explain the working of an inverting amplifier. Derive an expression for gain of the inverting amplifier. Write equations for determining suitable values for each resistor using bipolar op-amp. | CO2 | (05) |
|  |  | UNIT- II |  |  |
| 3. | a) | With a neat sketch explain the working for capacitor coupled voltage follower. Write the steps involved in designing the circuit. Also explain why the capacitor coupled voltage follower's input impedance is much smaller than that of direct coupled voltage follower. | CO2 | (08) |
|  | b) | With a neat sketch explain the working of capacitor coupled difference amplifier. Write the equations for calculating each of the component values used in the circuit. | CO2 | (05) |
|  | c) | Using a LF353 BIFET op-amp, design a high Zin capacitor coupled non-inverting amplifier to have a low cutoff frequency of 200 Hz. The input and output voltages are to be 15mV and 3V respectively and the minimum load resistance is 12 KΩ. Sketch the circuit with component values mentioned on it. | CO2 | (07) |
| 4. | a) | With a neat sketch explain the working of a dead zone circuit. | CO2 | (05) |
|  | b) | Using bipolar op-amp with Vcc = + 15V, design the high input impedance precision full wave rectifier circuit. The input peak voltage is to be 1V and no amplification is to occur. Sketch the circuit diagram with component values mentioned on it. | CO2 | (08) |
|  | c) | With a neat sketch explain the working of precision clamping circuit. Discuss the design procedure and sketch the input and output waveforms. | CO2 | (07) |
|  |  | UNIT- III |  |  |
| 5. | a) | With a neat circuit diagram explain the working of wide band pass filter. Plot its frequency response curve and explain the step by step procedure involved in designing the filter. | CO2 | (10) |
|  | b) | - Design a second order low pass filter to have a cut off frequency of 2 KHz with a pass band gain of 1.586. Draw the circuit diagram with component values on it. - On a semi log sheet, plot the frequency response curve of the low pass filter designed in (i). - Using frequency scaling technique, modify low pass filter designed in (i) to have a new cutoff frequency of 3 KHz. | CO2 | (10) |
| 6. | a) | With a neat sketch explain the working of Wien bridge oscillator. | CO2 | (08) |
|  | b) | With a neat circuit diagram explain the working of triangular wave generator. Derive an expression for its frequency of oscillation. Also show how the triangular wave generator can be modified to produce a saw tooth waveform generator. | CO2 | (12) |
|  |  | UNIT- IV |  |  |
| 7. | a) | - Calculate the slew rate limited cutoff frequency for a voltage follower circuit using a 741 op-amp if a peak of sine wave output is to be 5V. - Determine the maximum peak value of the sinusoidal output voltage that will allow the 741 voltage follower circuit to operate at the 800 KHz unity gain cutoff frequency. - Calculate the maximum peak value of sine wave output voltage that can be produced by the amplifier which is having an upper cutoff frequency of 8 KHz. (From 741 data sheet: typical slew rate = 0.5 V/µs, unity gain bandwidth = 800 KHz). | CO3 | (05) |
|  | b) | A battery charger control circuit is to be designed to monitor the voltage level of a 12V battery. When the battery’s voltage drops below 11.5V, the battery should be connected to the charger. When the battery voltage reaches 13V, it should be disconnected from the charger. The reference voltage is to be -15V. Assume + Vsat = + 14V. Design a suitable circuit. Sketch the complete circuit with component values mentioned on it. | CO2 | (07) |
|  | c) | With a neat circuit diagram and waveforms explain the working non-inverting voltage level detector with different UTP and LTP. Plot the hysteresis curve. Also discuss the design steps involved in designing the circuit. | CO2 | (08) |
| 8. | a) | Using a 709 Op-amp, design an inverting amplifier as shown in Fig.8(a) to have AV =100 and Vs =2mV (IB(max) for 709=200nA) and also calculate the input stray capacitance that might make the circuit unstable when - The signal source is open circuited. - R3 is short circuited. - R3 is present and all resistors are reduced by a factor of 5. Fig. 8(a) | CO3 | (10) |
|  | b) | Explain miller effect. Derive the equation relating the input capacitance of an inverting amplifier to the capacitance connected between input and output terminals. Also with a neat sketch explain how miller’s effect capacitors can be employed for phase lag and phase lead compensation. | CO3 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Using 7805C voltage regulator, design a current source that will deliver a 0.25 A current to a 48Ω, 10W load. Also calculate the minimum input voltage. Draw the circuit with component values mentioned on it. | CO5 | (05) |
|  | b) | With a neat sketch explain the working of voltage to current converter with grounded load. Also derive an expression for the load current IL. | CO4 | (07) |
|  | c) | With a neat sketch, waveforms and frequency response curve, explain the working of a differentiator. Derive an expression for its output voltage. | CO2 | (08) |
| 10. | a) | Design an astable multivibrator using 555 timer for a free running frequency f0 = 1.45 KHz, the positive pulse width tc = 0.421ms and negative pulse width td = 0.269ms. Sketch the circuit with component values mentioned on it. | CO5 | (10) |
|  | b) | With a block diagram explain the working of Phase locked loop. Bring out the three stages in the operation of Phase locked loop. | CO5 | (05) |
|  | c) | Bring out the difference between monostable and astable mode of the 555 timer. | CO5 | (05) |

****************