RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS –  AUGUST 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Linear Integrated Circuits | Max. Marks | : | 100 |
| Subject Code | : | EE54 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  |  | UNIT – I |  |  |
| --- | --- | --- | --- | --- | --- |
| 1. | a) |  | Explain the following terms: i) Output voltage range ii) input offset current iii) CMRR iv) PSRR v) slew rate. | CO1 | (10) |
|  | b) |  | With a neat circuit diagram explain the operation of non-inverting summing amplifier. Derive an expression for its output voltage. | CO2 | (06) |
|  | c) |  | An inverting amplifier using 741 op-amp has input resistance of 18 kΩ and feedback resistance is 180 kΩ. Calculate the minimum input impedance and output impedance of the inverting amplifier. (From 741 op-amp data sheet, open loop voltage gain = 2,00,000, open loop output impedance = 75 Ω, open loop input impedance = 0.3 MΩ) | CO1 | (04) |
| 2 | a) |  | i) The difference of two input signals each ranging from 0.1 V to 1 V is to be amplified by a factor of 15. Using 741 op-amp, design a suitable difference amplifier. Determine the differential input resistance and common mode input resistance. ii) Modify the circuit designed in i) to give approximately equal input resistance at the two input terminals and to provide output voltage nulling. Draw the circuit for i) and ii) separately mentioning the component values on it. | CO1 | (10) |
|  | b) |  | Design a non-inverting amplifier using LF353 BIFET op-amp for a voltage gain of approximately 66. The signal amplitude is to be 15mV. Sketch the circuit with component values mentioned on it. | CO2 | (05) |
|  | c) |  | Explain how slew rate of an op-amp can introduce distortion in the output waveform. | CO1 | (05) |
|  |  |  | UNIT – II |  |  |
| 3. | a) |  | With a neat sketch explain the working of high input impedance capacitor coupled voltage follower amplifier. Derive an expression for the input impedance of the circuit. Write the expression for calculating component values using 741 and LF353 op-amp. | CO2 | (10) |
|  | b) |  | Design the precision full wave rectifier to produce a 2V peak output from a sine wave input with a 0.5 V peak value and 1 MHz frequency. Use bipolar op-amps with a supply voltage of ±15 V. | CO2 | (10) |
| 4. | a) |  | Design a capacitor coupled inverting amplifier using single polarity supply. The capacitor coupled inverting amplifier is to have a voltage gain of 5, the output voltage of 2.5V, lower cut off frequency of 1.5 kHz, upper cutoff frequency of 15 kHz, a load resistance of 10 kΩ and should use a +18V single polarity power supply. Use 741 op-amp. Sketch the circuit with component values mentioned on it. | CO2 | (10) |
|  | b) |  | With a neat circuit explain the working of a high input impedance precision rectifier. Draw the voltage waveforms throughout the circuit showing typical voltages at various points in the circuit. Write the expression for calculating component values using 741 op-amp. | CO2 | (10) |
|  |  |  | UNIT – III |  |  |
| 5. | a) |  | With a neat circuit and waveforms, explain the working of op-amp sample and hold circuit. Discuss the holding time and acquisition time for an op-amp sample and hold circuit and write the equations for determining capacitor size and minimum acquisition time. | CO2 | (10) |
|  | b) |  | Design a first order low pass filter to have a cutoff frequency of 15 KHz with a pass band gain of 4. Draw the circuit diagram with component values on it. On a semi log sheet, plot the frequency response curve of the low pass filter designed. Using frequency scaling technique, modify the low pass filter designed to have a new cutoff frequency of 20 KHz. | CO2 | (10) |
| 6. | a) |  | With a neat circuit diagram explain the working of triangular wave generator. Derive an expression for its frequency of oscillation. Also show how the triangular wave generator can be modified to produce a saw tooth waveform generator. | CO2 | (10) |
|  | b) |  | Design a RC phase shift oscillator for an oscillating frequency of 1 KHz. Draw the circuit with component values mentioned on it. | CO2 | (05) |
|  | c) |  | With a neat sketch explain the working of active notch filter. | CO2 | (05) |
|  |  |  | UNIT – IV |  |  |
| 7. | a) |  | Design a non-inverting Schmitt Trigger with VUT = 12 V and VLT = 8 V. Use 741 op-amp with a supply voltage of ±15 V. Draw the circuit with component values mentioned on it and also draw the expected input and output waveform and the hysteresis curve for the circuit designed. | CO2 | (10) |
|  | b) |  | Explain the effects of stray and load capacitance on op-amp circuit stability. | CO3 | (10) |
| 8. | a) |  | With the help of suitable sketches briefly explain the following compensation method: - Miller’s effect compensation - Manufacturer recommended compensation. | CO3 | (10) |
|  | b) |  | Using a bi-polar op-amp design a inverting type Schmitt Trigger circuit to trigger at ±0.5 V and to produce an output of approximately ±11V. Draw the transfer characteristics. | CO2 | (05) |
|  | c) |  | Show how the slew rate of an op-amp can produce distortion in a sinusoidal output waveform. | CO3 | (05) |
|  |  |  | UNIT – V |  |  |
| 9. |  | a) | With a neat sketch explain the working of voltage to current converter with grounded load. Also derive an expression for the load current IL. | CO4 | (10) |
|  |  | b) | Design an astable multivibrator for a duty cycle of 50%. The free running frequency is 1 KHz. Draw the circuit with component values mentioned on it. | CO5 | (10) |
| 10. |  | a) | With a neat block diagram and waveforms, explain the operation of timer IC 555 as a monostable multivibrator. | CO5 | (10) |
|  |  | b) | With the help of a neat circuit, explain the working of an integrator and draw its input voltage waveform, output voltage waveform and frequency response curve. Derive the expression for its output voltage. | CO4 | (10) |

****************