RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A+’ Grade

EXAMINATIONS SEPTEMBER /OCTOBER 2021

SUPPLEMENTARY SEMESTER / GRADE IMPROVEMENT/ RE –REGISTERED CANDIDATES

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Linear Integrated Circuits | Max. Marks | : | 100 |
| Course Code | : | EE54(O) | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer any five full questions.

| 1 | a) | An operational amplifier circuit with a closed loop gain of 100 has a common mode output of 5µV when the common input is 5mV. Determine the common mode rejection ratio. | CO1 | (05) |
| --- | --- | --- | --- | --- |
|  | b) | Using a 741 op-amp, design a non inverting amplifier to have a voltage gain of approximately 50. The signal amplitude is to be 10 mV. Calculate the input and output impedance for the non inverting amplifier designed. Also draw the circuit diagram of the non inverting amplifier. (from 741 data sheet: IB(max) = 500 nA, Zo = 75 , Zi = 2M and M = 200000) | CO2 | (10) |
|  | c) | Draw and explain the block diagram of an op-amp. Also explain the function of each block. | CO1 | (05) |
| 2. | a) | Draw and explain: i) Equivalent circuit of an op-amp ii) Ideal op-amp transfer curve. | CO1 | (06) |
|  | b) | With a neat sketch explain the working of three input inverting summing amplifier. Derive an expression for its output voltage. Write equations for determining suitable value of each resistor. Also explain how the three input summing amplifier may be used as an averaging circuit. | CO2 | (10) |
|  | c) | Bring out the difference between a voltage follower and an emitter follower. | CO2 | (04) |
| 3. | a) | With a neat sketch, explain the working of high input impedance capacitor coupled non inverting amplifier. Also, Derive an expression for the input impedance of the circuit. | CO2 | (08) |
|  | b) | Design a precision full-wave rectifier circuit to produce a 2V peak output from a sine wave input with a peak value of 0.5V and a frequency of 1MHz. Use bipolar op-amps with a supply voltage of 15V. Draw the circuit with component value mentioned on it. | CO2 | (08) |
|  | c) | Discuss the advantages of a precision rectifier over an ordinary diode rectifier circuit. | CO2 | (04) |
| 4. | a) | With a neat circuit diagram and waveforms, explain the operation of a two-output half-wave precision rectifier. | CO2 | (08) |
|  | b) | A capacitor-coupled non-inverting amplifier is to have a +24V supply, a voltage gain of 100, an output amplitude of 5V, a lower cutoff frequency of 75Hz, and a minimum load resistance of 5.6k. Using a 741 op-amp, design a suitable circuit. Draw the circuit with component value mentioned on it. | CO2 | (12) |
| 5 | a) | Using BIFET op-amps, design a voltage follower rectifier peak detector circuit to handle a peak input of 1V. The minimum rise time of the input is 30µs, the holding time is to be 300µs and the maximum output error is to be 5%. Calculate the required component values and specify the output current and slew rate of the operational amplifier. Assume diode reverse leakage current as 1µA. Sketch the circuit with component values mentioned on it. | CO2 | (08) |
|  | b) | (i) Design a First order low pass filter to have a cut off frequency of 3 KHz with a pass band gain of 4. Draw the circuit diagram with component values on it. (ii) On a semi log sheet, plot the frequency response curve of the low pass filter designed in (i). (iii) Using frequency scaling technique, modify low pass filter designed in (i) to have a new cutoff frequency of 4 KHz. | CO2 | (12) |
| 6 | a) | With a neat and waveforms, explain the working of op-amp sample and hold circuit. Discuss the holding time and acquisition time for an op-amp sample and hold circuit and write the equations for determining capacitor size and minimum acquisition time. | CO2 | (10) |
|  | b) | With a neat sketch and waveform, explain the working of square wave generator. Write expression for its frequency of oscillation. Also list out the steps for designing a square wave generator. | CO2 | (10) |
| 7. | a) | Design an inverting Schmitt trigger with VUT = 3V and VLT = -3V. Use 741 op-amp with a supply voltage of 15V. Draw the circuit with component values mentioned on it. Also, draw the input, output waveforms and hysteresis curve for the circuit designed. | CO2 | (08) |
|  | b) | - Calculate the cutoff frequency limited rise time for a voltage follower circuit using a 741 op-amp. Also determine the slew rate limited rise time if the output amplitude is to be 5V. - Determine the maximum undistorted pulse output amplitude for the 741 voltage follower if the output rise time is not to exceed 1s. - Calculate the minimum output rise time and the maximum pulse amplitude at that rise time for a 741 amplifier with an upper cutoff frequency of 100kHz (for 741 op-amp gain bandwidth product is 800kHz, slew rate is 0.5 V/µs). | CO3 | (06) |
|  | c) | Explain miller effect. Derive the equation relating the input capacitance of an inverting amplifier to the capacitance connected between input and output terminals. | CO3 | (06) |
| 8. | a) | With the help of suitable sketch, briefly explain the following compensation methods: (i) Phase lag compensation (ii) Phase lead compensation. | CO3 | (08) |
|  | b) | Design a non-inverting voltage level detector to have VUT = 12 and VLT = 8V. Assume that +Vsat = +15V. Draw the circuit diagram and expected input-output waveform for the circuit designed. Also, draw the hysteresis curve. | CO2 | (08) |
|  | c) | Discuss the necessity of positive feedback for a voltage level detector. | CO2 | (04) |
| 9 | a) | With a neat sketch, waveforms and frequency response curve, explain the working of a Integrator. Derive an expression for its output voltage. Also mention the steps required to design the integrator | CO4 | (10) |
|  | b) | i) With a neat sketch and waveforms, explain the working of 555 timer as a monostable multivibrator ii) Suggest suitable modifications need to be made to the monostable multivibrator such that it works as a frequency divider circuit. iii) Monostable mutivibrator is to be used as a divided by 2 network (frequency divider). The frequency of the input trigger signal is 2KHz. If the value of C = 0.01F, what should be the value of RA? | CO5 | (10) |
| 10 | a) | Explain the characteristic parameters of voltage regulators | CO5 | (06) |
|  | b) | With a neat sketch explain the working of low voltage a.c. voltmeter | CO4 | (08) |
|  | c) | Using 7805C voltage regulator, design a current source that will deliver a 0.25 A current to a 48Ω, 10W load. Also calculate the minimum input voltage. Draw the circuit with component values mentioned on it. | CO5 | (06) |

****************