RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

SUPPLEMENTARY SEMESTER EXAMINATIONS – AUGUST 2017

| Course & Branch | : | B.E: Electrical & Electronics Engineering | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Linear Integrated Circuits | Max. Marks | : | 100 |
| Subject Code | : | EE604 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Use of standard resistor and capacitor values list is permitted. - Suitable assume the missing data if any.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Define the following with respect to an opamp and mention the typical values with respect to 741 opamp: i) CMRR ii) PSRR iii) Slew rate iv) Input offset voltage. | CO1 | (08) |
|  | b) | An LM308 opamp circuit with a closed loop gain of 50 has a common mode voltage of 1V. Calculate the maximum output voltage this might produce. The minimum CMRR of LM308 is 80db. | CO2 | (04) |
|  | c) | The difference of 2 input signals which each range from 0.2V to 2V with an amplification factor of 5 is to be determined. Using a 741 opamp design a suitable difference amplifier. Determine the input resistance at each input, differential input resistance and the common mode input resistance. | CO2 | (08) |
| 2. | a) | If an opamp has a bias current of 200nA and two equal resistances of 22K are connected between inverting terminal and ground, as well as non inverting terminal and ground respectively. What will be the input offset voltage for a tolerance of +20% in resistor values? What will be the output voltage under open loop gain of 105? | CO2 | (05) |
|  | b) | Compare a voltage follower and an emitter follower. | CO1 | (05) |
|  | c) | Design a non- inverting amplifier to have a voltage gain of approximately 50. The input signal amplitude is to be 20mV. - Use a 741 opamp having IB(max)= 500nA. - Use a BIFET opamp LF353. | CO2 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | A + 1.5V signal is to be sampled by a sample and hold circuit. The holding time is to be 700µs, and the output accuracy is to be approximately 0.3%. LM108 op-amps are to be used, and the available FET has a 25Ω channel resistance and a 300nA gate – source reverse leakage current. Design the circuit and determine the minimum acquisition time. Draw the circuit with component values mentioned on it. (For LM108 data sheet IB (max) = 2 nA). Draw the circuit with circuit values mentioned on it. | CO5 | (12) |
|  | b) | With a neat sketch explain the working of high input impedance non inverting amplifier with single polarity supply. Write equations for calculating component values using 741 op-amp. | CO2 | (08) |
| 4. | a) | Show how two op-amp dead zone circuits can be combined with a summing circuit to produce precision limiting on both positive and negative half cycles of the output waveform. Draw the voltage waveforms throughout the circuit and explain its operation. | CO2 | (12) |
|  | b) | Using a 741 op-amp, design a high Zin capacitor coupled non inverting amplifier to operate with a +24V power supply. The minimum value of load resistance is 10 KΩ, the lower cutoff frequency is 100 Hz, upper cutoff frequency is 18 KHz and the voltage gain is to be 5. Draw the circuit with component values mentioned on it | CO5 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | (i) Design a second order low pass filter to have a cut off frequency of 10 KHz with a pass band gain of 1.586. Draw the circuit diagram with component values on it. (ii) On a semi log sheet, plot the theoretical frequency response curve of the second order low pass filter designed in (i). (iii) Using frequency scaling technique, modify low pass filter designed in (i) to have a new cutoff frequency of 20 KHz. | CO5 | (12) |
|  | b) | Design a Wien bridge oscillator for an oscillating frequency of 1 KHz. Draw the circuit with component value mentioned on it. | CO5 | (08) |
| 6. | a) | Design a square wave generator for an oscillating frequency f0 = 10 KHz using 741 op-amp with a D.C. supply voltage of + 15V. Sketch the circuit with component values mentioned on it. | CO5 | (06) |
|  | b) | Design a wide band reject filter having fH = 200 Hz and fL = 1 KHz and a pass band gain of 4. Sketch the circuit of wide band reject filter with component values mentioned on it. Also calculate the quality factor for the filter designed. | CO5 | (10) |
|  | c) | Bring out the difference between active filter and passive filter. | CO2 | (04) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain any two methods of frequency compensation techniques. | CO5 | (10) |
|  | b) | Using a 741 opamp with a supply of +12V. Design an inverting Schmitt trigger circuit to have trigger points of +1V. | CO3 | (06) |
|  | c) | Determine the upper cut off frequency for i) A voltage follower circuit using a 741 opamp and ii) A non inverting amplifier with a voltage gain of 2 using a 741 opamp. | CO5 | (04) |
| 8. | a) | Design a non inverting Schmitt trigger circuit to have UTP= +3V and LTP= -5V. Use a 741 opamp with Vcc= +15V. Draw the circuit with designed component values. | CO3 | (10) |
|  | b) | What is over compensation and what are its effects? | CO5 | (04) |
|  | c) | What precautions should be taken for opamp circuit stability? | CO5 | (06) |
|  |  | UNIT- V |  |  |
| 9 | a) | With the help of a circuit diagram, explain the working of a differentiator. Draw its input voltage waveform, output voltage waveform and frequency response curve. Derive an expression for its output voltage. Discuss the procedure for designing the differentiator. | CO2 | (12) |
|  | b) | Design a free running ramp generator using 555 timer for a free running frequency of 3 KHz. Assume Vcc = 5V and VBE = 0.7 V. Sketch the circuit with component values mentioned on it. | CO4 | (08) |
| 10. | a) | With a neat circuit diagram, explain the working of a low voltage AC voltmeters. | CO2 | (08) |
|  | b) | Design an astable multivibrator using 555 timer for a free running frequency f0 = 1.45 KHz, the positive pulse width tc,= 0.421 ms and negative pulse width td = 0.269 ms. Sketch the circuit with component values mentioned on it. | CO4 | (08) |
|  | c) | Write a brief note on voltage regulators. | CO4 | (04) |

****************