RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

MAKEUP EXAMINATIONS – MAY/JUNE2017

| Course & Branch | : | B.E: Electrical & Electronics Engineering | Semester | : | VI |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Power Systems - I | Max. Marks | : | 100 |
| Subject Code | : | EE601 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | List the advantages of per unit system. | CO2 | (04) |
|  | b) | A 300MVA, 20kVthree phase generator supplies a number of synchronous motors over a 64kmtransmission line having transformers at both ends as shown in Fig.1 (a). The motors are all rated13.2 kV. Rated inputs to the motors are 200 MVAand100 kV for M1and M2.The three-‐phase transformer T1is rated350 MVA, 230/20 kV with leakage reactance of10%. Transformer T2is composed of three single-‐phase transformers, each rated127/13.2 kV, 100 MVA with a leakage reactance of10%. Series reactance of the transmission line is 0.5Ω/km. Assume that the generator and both motors have a sub transient reactance of0.2 per unit. Draw the reactance diagram. Fig.1 (a) | CO2 | (10) |
|  | c) | Explain the concept of complex power and the direction of power flow. | CO1 | (06 ) |
| 2. | a) | A generator connected through a circuit breaker to a transformer is rated 100MVA, 18kV, with reactances of Xd”= 19% and Xd= 130%. It is operating on no load and rated voltage when a three phase short circuit occurs between the breaker and the transformer. Find i) the steady state short circuit current in the breaker ii) the initial sub-transient current in the breaker and iii) the maximum possible dc component of the short circuit current in the breaker. | CO3 | (10) |
|  | b) | Show that the pu impedance of a transformer is same whether it is referred to its LV side or HV side. | CO2 | (06) |
|  | c) | Draw the circuit models for synchronous machines, two winding transformer and transmission line. | CO1 | (04) |
|  |  | UNIT- II |  |  |
| 3. | a) | Prove that a balanced set of three-phase voltages will have only positive sequence components. | CO4 | (08) |
|  | b) | Determine the sequence components of the three voltages, <2450 and VC = 200<105OV. | CO4 | (06) |
|  | c) | Derive expression for phase voltages in terms of symmetrical components. | CO4 | (06) |
| 4. | a) | Derive the relation between sequence components of phase and line voltages in delta connected systems. | CO4 | (10) |
|  | b) | A balanced star connected load takes 30A from a balanced 3-phase, 4-wire supply. If the fuses in two lines are removed, find the symmetrical components of the line currents before and after the fuses are removed. | CO4 | (10) |
|  |  | UNIT- III |  |  |
| 5. | a) | A star connected synchronous generator has sequence reactances X0=0.09, X1=0.22, X2=0.36 all in pu. The neutral point of the machine is grounded through a reactance of 0.09 pu. The machine is running on no load with rated terminal voltage when it suffers an unbalanced fault. The fault currents out of the machine are Ia=0, Ib=3.75∟1500, and Ic=3.75∟300, all in pu with respect to phase ‘a’ line to neutral voltage. Determine i) the terminal voltages in each phase of the machine with respect to ground ii) the voltage of the neutral point of the machine with respect to ground and iii) the type of fault. | CO4 | (10) |
|  | b) | Explain the sequence networks of: i) unloaded generators ii) transmission lines. | CO4 | (10) |
| 6. | a) | Draw the zero sequence network of the power system shown in fig.5 (a). Choose a system base of 100MVA and 20 kV at G1. The ratings of the equipments are G1: 100MVA, 20kV, X=0.2pu G2: 80MVA, 20kV, X=0.2pu G3: 60MVA, 20kV, X=0.12pu TL: 100MVA, X=20Ω T1: 100MVA, 20/138kV, X=0.08pu T2: 50MVA, 20/138KV, X=0.04pu Fig.5(a) | CO4 | (06) |
|  | b) | Show that, in symmetrical circuits, currents of a given sequence produce voltage drops of the same sequence only. | CO4 | (06) |
|  | c) | Obtain the sequence networks of an unloaded three phase star connected generator grounded through a reactor. | CO4 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Draw and explain the interconnection of sequence networks in case of i) line to ground fault ii) double line to ground fault. | CO4 | (12) |
|  | b) | The reactances of a generator rated 100MVA, 20kV, are Xd”=X1=X2=20% and X0=5%. The generator is connected to a Δ-Y transformer rated 100MVA, 20Δ-230Y kV, with a reactance of 10%. The neutral of the transformer is solidly grounded. The terminal voltage of the generator is 20kV when a single line-to-ground fault occurs on the open circuited, high voltage side of the transformer. Find the initial symmetrical rms current in all phases of the generator. | CO4 | (08) |
| 8. | a) | A salient pole generator rated 20MVA, 13.8kV, with X1= 0.25pu, X2= 0.35pu and X0=0.1pu. The neutral of the generator is solidly grounded. Determine the sub-transient current in the generator and the line to line voltages for sub-transient condition when an L-G fault occurs at the generator terminal with the generator operating at rated voltage. Neglect resistance. | CO4 | (10) |
|  | b) | Derive the expression for the fault current in case of an L-L fault on an unloaded generator through a fault impedance. | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Derive the power angle equation of a salient pole synchronous machine connected to an infinite bus. Draw the power angle curve. | CO5 | (10) |
|  | b) | Find the steady state power limit of a two machine system consisting of a synchronous generator with an equivalent reactance of 0.5 pu connected to an infinite bus through a reactance of 1.0 pu. The terminal voltage of the generator is held at 1.1 pu and the voltage at the infinite bus is 1.0 pu. Use Clarke’s diagram. | CO5 | (10) |
| 10. | a) | Derive expression for the critical clearing angle for a typical system. | CO5 | (10) |
|  | b) | Power is supplied by a generator to a motor over a transmission line as shown in Fig. 10(b). To the motor bus a capacitor of 0.8 pu reactance per phase is connected through a switch. Determine the steady state power limit with and without the capacitor in the circuit. Fig. 10(b) | CO5 | (10) |

****************