RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering. | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Transmission and Distribution | Max. Marks | : | 100 |
| Subject Code | : | EE53/EE503 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit - Suitably assume the missing data, if any.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Define sag? Derive an expression for the sag of an overhead line supported at different levels. | CO2 | (10) |
|  | b) | Draw single line diagram of a typical power system. Locate various sections of it and mention there on typical voltages of generation, transmission and distribution. | CO1 | (05) |
|  | c) | Discuss the advantages of high transmission voltage in terms of volume of the conductor material required, transmission efficiency and percentage line drop. | CO1 | (05) |
| 2. | a) | Explain the need of high voltage in the transmission of power. | CO1 | (05) |
|  | b) | Discuss the limitations of AC transmission. | CO1 | (05) |
|  | c) | An overhead line is erected across a span of 250m on level supports. The conductor has a diameter of 1.42cm and has a dead weight of 1.09 kg/m. The line is subjected to a wind pressure of 37.8 kg/m2 of the projected area. The radial thickness of ice is 1.25cm. The line is carried by insulator string 1.43m long. Calculate (i) sag in an inclined direction (ii) sag in a vertical direction and (iii) the height of lowest cross arm to give a minimum ground clearance of 7.62 m under bad weather conditions. Assume one cubic metre of ice weight 913.5 kg and a maximum stress of 1050 kg/cm2. | CO2 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | Show that the inductance per unit length of an overhead line due to internal flux leakage is constant and is independent of size of conductor. | CO3 | (08) |
|  | b) | Derive the expression of inductance for a 3-phase overhead line with unsymmetrical spacing. | CO3 | (08) |
|  | c) | Find out capacitance of a single phase line 30 km long consisting of two parallel wires each 15 mm diameter and 1.5 m apart. | CO3 | (04) |
| 4. | a) | Derive an expression for capacitance of a single-phase transmission line. | CO3 | (07) |
|  | b) | A single 3-phase line operated at 50Hz is arranged as equilaterally spaced with 2m, 2m and 3m sides. The conductor diameter is 0.6 cm. Fine the inductance and capacitance per/km. The line is regularly transposed. | CO3 | (08) |
|  | c) | Why do you find line to neutral capacitance in a three phase system? Explain. | CO3 | (05) |
|  |  | UNIT- III |  |  |
| 5. | a) | A 3 phase transmission line is 400 km long and caters a load of 450 MVA, 0.8 p.f. lag at 345 kV. The ABCD constants are A = D = 0.8181  1.30 B = 172.2  84.20 C = 1.933 x 10-3 90.40 mho. Calculate sending end current and percentage voltage drop. Also calculate receiving end line to neutral voltage at no load and the sending end line to neutral voltage. | CO4 | (10) |
|  | b) | What is Ferranti effect? Explain it with the help of a phasor diagram. | CO4 | (10) |
| 6. | a) | A 3 phase 50 Hz overhead transmission line has the following constants per phase R = 28Ω, X = 63Ω, Y = 4 x 10-4 mho (per km). If the load at the receiving end is 75 MVA at 0.8 p.f. lag with 132 kV between lines, calculate the voltage, current and p.f at the sending end. Use nominal  method. Draw the circuit diagram and vector diagram. | CO4 | (12) |
|  | b) | Compare and contrast between short, medium and long transmission lines. | CO4 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Derive expression for the electric stress in a single core cable. | CO5 | (06) |
|  | b) | A single core cable for use on 11kV, 50Hz system has conductor area of 0.645 cm2 and the internal diameter of sheath is 2.18cm. The permittivity of the dielectric used in the cable is 3.5. Find the maximum and minimum electrostatic stresses in the cable. Show also the curve of stress distribution in the cable v/s distance from the centre of the conductor. | CO5 | (08) |
|  | c) | Derive an expression for capacitance of a co-axial cable. | CO5 | (06) |
| 8. | a) | What is the use of intersheaths? Explain the principle of intersheath grading of a cable. | CO5 | (08) |
|  | b) | A single core cable has a conductor of diameter 2.5 cm and a sheath of inside diameter 6 cm. Calculate the maximum stress. It is desired to reduce the maximum stress by using two intersheaths. Determine their best positions, the maximum stress and the voltages on each system. Voltage is 66 kV, 3 phase. | CO4 | (12) |
|  |  | UNIT- V |  |  |
| 9. | a) | A single phase two-wire feeder, 1500 m long, supplies a load of 60A at 0.8pf; 40A at 0.85pf; and 50A at 0.88 pf lagging at distances of 600, 1200 and 1500 metres respectively from the feeding point. The resistance and reactance of the feeder per km length are 0.06 and 0.1 ohms, respectively. If the voltage at the far end is to be maintained at 220V, calculate the voltage at the sending end, and its phase angle with respect to the receiving end voltage. Fig. 10.(a) | CO1 | (12) |
|  | b) | With necessary diagram, explain D.C distributor with uniformly distributed loads fed at both end which are maintained at equal voltages. Also derive the equation for voltage drop at different points for the chosen system. | CO1 | (08) |
| 10. | a) | A 2 wire d.c. ring main is fed at A and load is tapped at points B, C and D. The distributor length is 400 m long and point B,C and D are 150 m, 250 m and 375 m from point A. The loads are 150 A, 40 A and 200 A respectively. If the resistance per 100 m of single conductor is 0.04 ohms and voltage at feeding point A is 240 V, calculate: i) Current in each section of distributor ii) Voltage at points B, C and D. | CO1 | (10) |
|  | b) | A single phase ring distributor ABC is fed at A. The load at B and C are 40 A at 0.8 p.f. lagging and 60 A at 0.6 p.f. lagging respectively. Both power factors expressed are referred to the voltage at point A. The total impedance of section AB, BC and CA are (2+j1), (2+j3) and (1+j2) ohms respectively. Determine current in each section. | CO1 | (10) |

****************