RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

SEMESTER END EXAMINATIONS – JANUARY 2017

| Course & Branch | : | Electrical & Electronics Engineering. | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Transmission and Distribution | Max. Marks | : | 100 |
| Subject Code | : | EE503 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT-I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss about the factors affecting choice of transmission voltage level. | CO1 | (06) |
|  | b) | Discuss the effect of increase in voltage on the transmission line efficiency. | CO1 | (06) |
|  | c) | Briefly discuss HVDC transmission. | CO1 | (08) |
| 2. | a) | Derive an expression for the sag of an overhead line supported at different levels. | CO2 | (10) |
|  | b) | An overhead line with a span of 300m and in a hilly area with uniform ground slope of 1in 20 and having a conductor weight of 0.80 kg/m, is supported between two towers at equal height of 30m above the ground level. Assuming the horizontal component of tension in the line to be 1500 kg; find (i) the clearance at the point where conductor has its lowest elevation and (ii) the minimum clearance of the line from ground. | CO2 | (10) |
|  |  | UNIT-II |  |  |
| 3. | a) | Show that the inductance per unit length of an overhead line due to internal flux leakage is constant and is independent of size of conductor. | CO3 | (10) |
|  | b) | Derive the expression of inductance for a 3-phase overhead line with symmetrical spacing. | CO3 | (10) |
| 4. | a) | Derive expression for line-to-line capacitance and line to neutral capacitance of single-phase line. | CO3 | (12) |
|  | b) | A single 3-phase line operated at 50Hz is arranged as equilaterally spaced with 2m, 2m and 3m sides. The conductor diameter is 0.6 cm. Fine the inductance and capacitance per/km. The line is regularly transposed. | CO3 | (08) |
|  |  | UNIT –III |  |  |
| 5. | a) | Distinguish between short, medium and long transmission lines. | CO4 | (10) |
|  | b) | Derive an expression for voltage regulation of a short transmission line with vector diagram. | CO4 | (10) |
| 6. | a) | A 220kV, 3-phase transmission line is 40km long. The resistance per phase is 0.15Ω per km and the inductance per phase is 1.5923 mH per km. The line is supplying a 3-phase load of 381 MVA at 0.8 pf lagging. Find: (i) the voltage and power factor at the sending end of the line (ii) the voltage regulation and efficiency of the line. | CO4 | (10) |
|  | b) | Derive A, B, C, D constants of a medium length transmission line and hence prove that AD – BC = 1. | CO4 | (10) |
|  |  | UNIT-IV |  |  |
| 7. | a) | What is the purpose of metallic sheath in underground cables | CO5 | (06) |
|  | b) | A66kV single core metal sheathed cable is to be graded by means of a metallic intersheath. Calculate the diameter of the intersheath and the V at which it must be maintained in order to obtain minimum overall cable dia. The maximum V gradient at which the insulating material can be worked is 60kV/cm . | CO5 | (06) |
|  | c) | Show that in a string of suspension insulators, the disc nearest to the conductor has the highest voltage across it. | CO5 | (08) |
| 8. | a) | Explain with necessary equations and diagram capacitance grading of cables. | CO5 | (06) |
|  | b) | A single core cable 5 km long has an insulation resistance of 0·4 MΩ. The core diameter is 20 mm and the diameter of the cable over the insulation is 50 mm. Calculate the resistivity of the insulating material. | CO5 | (08) |
|  | c) | The voltage distribution across an insulator string is not uniform – comment. | CO5 | (06) |
|  |  | UNIT-V |  |  |
| 9. | a) | Discuss briefly the design considerations in distribution system. | CO1 | (06) |
|  | b) | Explain the consequences of disconnection of neutral in a 3ph -4 wire system. | CO1 | (04) |
|  |  | A 800 metres 2-wire d.c. distributor AB fed from both ends is uniformly loaded at the rate of 1·25 A/metre run. Calculate the voltage at the feeding points A and B if the minimum potential of 220 V occurs at point C at a distance of 450 metres from the end A. Resistance of each conductor is 0·05 Ω/km. | CO1 | (10) |
| 10. | a) | In a 3-phase, 4-wire, 400/230 V system, a lamp of 100 watts is connected to one phase and neutral and a lamp of 150 watts is connected to the second phase and neutral. If the neutral wire is disconnected accidentally, what will be the voltage across each lamp? | CO1 | (10) |
|  | b) | A two-wire d.c. distributor cable 1000 metres long is loaded with 0·5 amp/m. Resistance of each conductor is 0·05 Ω/km. Calculate the maximum voltage drop if the distributor is fed from both ends with equal voltages of 220 V. What is the minimum voltage and where it occurs? | CO1 | (10) |

****************