RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS – JANUARY 2018

| Course & Branch | : | B.E. : Electrical & Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Transmission and Distribution | Max. Marks | : | 100 |
| Subject Code | : | EE53/EE503 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Suitably assume the missing data, if any

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss the need of high voltage in the transmission of power. | CO1 | (05) |
|  | b) | With neat sketch explain typical transmission and distribution systems. | CO1 | (05) |
|  | c) | An overhead line with a span of 300m and in a hilly area with uniform ground slope of 1in 20 and having a conductor weight of 0.80 kg/m, is supported between two towers at equal height of 30m above the ground level. Assuming the horizontal component of tension in the line to be 1500 kg; find (i) the clearance at the point where conductor has its lowest elevation and (ii) the minimum clearance of the line from ground. | CO2 | (10) |
| 2. | a) | Discuss the advantages of high transmission voltage in terms of volume of the conductor material required, transmission efficiency and percentage line drop. | CO1 | (07) |
|  | b) | Why the basis of comparison is on the maximum voltage between conductor and ground in an overhead system? | CO1 | (05) |
|  | c) | An overhead transmission line has a span of 230m, the conductor weighing 650 kg/km. Calculate the maximum sag if the breaking stress is 4465 kg/cm2 while the area of the conductor is 1.29 cm2. Assume factor of safety = 2. | CO2 | (08) |
|  |  | UNIT- II |  |  |
| 3. | a) | Derive expression for line-to-line capacitance and line to neutral capacitance of single-phase line. | CO3 | (12) |
|  | b) | Derive the expression of inductance for a 3-phase overhead line with symmetrical spacing. | CO3 | (08) |
| 4. | a) | Derive an expression for the inductance of a conductor and total flux linkages due to external flux. | CO3 | (10) |
|  | b) | A 3 phase, 50 Hz, 132 kV overhead line has conductors placed in a horizontal plane 4 m apart. Conductor diameter is 2 cm. If the length of line is 100 km, calculate the charging current per phase assuming complete transposition. | CO3 | (10) |
|  |  | UNIT- III |  |  |
| 5. | a) | A 3-phase, 50Hz, 100km long transmission line has the following per phase parameters: r=0.1 Ω/km; x=0.2 Ω/km; Yc=0.04 x10-4 mho/km. If the line is represented by nominal – T, determine i) sending end current ii) sending end voltage iii) sending end power factor iv) Transmission efficiency when supplying a balanced load 0f 10,000 kW at 66 kV with p.f. of 0.8 lagging. | CO4 | (12) |
|  | b) | Why ABCD constants are used to evaluate the performance of a transmission line. | CO4 | (05) |
|  | c) | Why do we analyse a three phase transmission line on single phase basis? Explain. | CO4 | (03) |
| 6. | a) | Derive an expression for voltage regulation of a short transmission line with vector diagram. | CO4 | (10) |
|  | b) | A single phase overhead transmission line delivers 1100 kW at 33 kV at 0.8 p.f. lagging. The load resistance and inductive reactance of the line are 10 ohms and 15 ohms respectively. Determine sending end voltage, sending end power factor iii) transmission efficiency with circuit diagram and vector diagram. | CO4 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | A 3-phase transmission line is supported by a 3-unit suspension insulator string. The voltage across the unit nearest to the line is 20kV and that across the adjacent unit is 15kV. Determine (i) ratio of mutual to ground capacitance (ii) system line voltage. Determine the string efficiency. | CO5 | (08) |
|  | b) | Explain capacitance grading of cables and derive an expression for operating voltage. | CO5 | (08) |
|  | c) | Derive expression for the ratio of maximum stress to minimum stress in a single core cable. | CO5 | (04) |
| 8. | a) | A single core cable employing three layers of insulation with dielectric constants εr1 = 5, εr2 = 4 and εr3 = 3 respectively has a conductor of radius 1cm. Assuming that all the three insulating materials are worked at the same maximum potential gradient, workout the potential difference in kV between the core and earthed sheath. The inner radius of the sheath is 2.5cm and the maximum potential gradient is 40kV/cm. | CO5 | (10) |
|  | b) | With neat sketches explain different method for improving string efficiency. | CO5 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | A single phase a.c. distributor AB 200 metres long is fed from end A and is loaded as under : (i) 50 A at 0·8 p.f. lagging 100 m from point A (ii) 100 A at 0·707 p.f. lagging 200 m from point A The load resistance and reactance of the distributor is 0·3 Ω and 0·2 Ω per kilometer. Calculate the total voltage drop in the distributor. The load power factors refer to the voltage at the far end. | CO1 | (10) |
|  | b) | With necessary diagram, explain D.C distributor with uniformly distributed loads fed at both end which are maintained at unequal voltages. Also derive the equation for voltage drop at different points for the chosen system. | CO1 | (10) |
| 10. | a) | A 2 wire D.C distributor 300 m long is uniformly loaded with a load of 1 A/m. The resistance of single wire is 0.4 Ω/km. If the distributor is fed at one end, calculate: i) the voltage drop upto a distance of 225 m from the feeding point ii) Thee maximum voltage drop. | CO1 | (07) |
|  | b) | With the help of phasor diagram, discuss the AC distribution when power factors referred to receiving-end voltage. | CO1 | (07) |
|  | c) | Explain the use of balancer in 3 wire d.c. distribution system. | CO1 | (06) |

****************