RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS –  JANUARY 2020

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Transmission and Distribution | Max. Marks | : | 100 |
| Course Code | : | EE53 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss the need of high voltage in the transmission of power. | CO1 | (06) |
|  | b) | Discuss the effect of increase in voltage on the transmission line efficiency. | CO1 | (06) |
|  | c) | Briefly discuss HVDC transmission. | CO1 | (08) |
| 2. | a) | Briefly discuss the different types of supporting structures, which are used for overhead lines. | CO2 | (10) |
|  | b) | An overhead line is erected across a span of 250m on level supports. The conductor has a diameter of 1.42cm and has a dead weight of 1.09 kg/m. The line is subjected to a wind pressure of 37.8 kg/m2 of the projected area. The radial thickness of ice is 1.25cm. The line is carried by insulator string 1.43m long. Calculate (i) sag in an inclined direction (ii) sag in a vertical direction and (iii) the height of lowest cross arm to give a minimum ground clearance of 7.62 m under bad weather conditions. Assume one cubic metre of ice weight 913.5 kg and a maximum stress of 1050 kg/cm2. | CO2 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | Show that the inductance per unit length of an overhead line due to internal flux leakage is constant and is independent of size of conductor. | CO3 | (10) |
|  | b) | Derive an expression of inductance for a 3-phase overhead line with symmetrical spacing. | CO3 | (10) |
| 4. | a) | Derive an expression for line-to-line capacitance and line to neutral capacitance of single-phase line. | CO3 | (12) |
|  | b) | Determine the capacitance and charging current per unit length of the line for the horizontal arrangement of conductors of diameter 20mm and spacing 3m. | CO3 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | Derive an expression for sending end voltage and current for long transmission line using rigorous solution. | CO4 | (10) |
|  | b) | A 3 phase 50 Hz overhead transmission line has the following constants per phase R = 28Ω, X = 63Ω, Y = 4 x 10-4 mho (per km). If the load at the receiving end is 75 MVA at 0.8 p.f. lag with 132 kV between lines, calculate the voltage, current and p.f at the sending end. Use nominal  method. Assume length of the line = 200 km. | CO4 | (10) |
| 6. | a) | A 3 phase overhead transmission line has a total series impedance per phase of 200800 ohms and a total shunt admittance of 0.0013 900 seimen per phase. The line delivers a load of 80 MW at 0.8 p.f lagging and 220 kV between the lines. Determine the sending end voltage and current by rigorous method. | CO4 | (12) |
|  | b) | Draw the vector diagram of short transmission line and hence derive an expression for its voltage regulation. | CO4 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | What do you mean by grading of cable? Explain why and how the grading of cable is done. | CO5 | (07) |
|  | b) | What is string efficiency? Why is it necessary to have a high string efficiency? How can it be achieved? | CO5 | (06) |
|  | c) | A suspension string has 3 units. Each unit can withstand a maximum peak voltage of 11 kV. The capacitance of each joint and metal work is 20% of the capacitance of each disc. Find i) maximum line voltage for which the string can be used ii) string efficiency. | CO5 | (07) |
| 8. | a) | Derive an expression for minimum and maximum stress in an insulation of a single core cable. Hence prove (gmax/gmin) = D/d. | CO5 | (10) |
|  | b) | Each of the three insulators of a string has self capacitance C farad. Capacitance of connecting metal work to earth is 0.2 C and 0.1C to line. Calculate the voltage across each insulator as a percentage of line voltage to earth. Also find string efficiency. | CO5 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Prove that the voltage drop diagram for a uniformly loaded distributor fed at one end is parabola. | CO1 | (05) |
|  | b) | With the help of phasor diagram, discuss the AC distribution when power factors referred to receiving-end voltage. | CO1 | (05) |
|  | c) | A two conductor street main AB, 500 metres in length is fed from both ends at 250V. Loads of 50A, 60A, 40A and 30 A are tapped at distances of 100m, 250m, 350m and 400m from A, respectively. If the cross section of the conductors be 1 cm2 and specific resistance of the material of the conductors is 1.7 x 10-8 ohm-metre, determine the minimum consumer voltage. | CO1 | (10) |
| 10. | a) | A single phase a.c. distributor AB 300 metres long is fed from end A and is loaded as under : (i) 100 A at 0·707 p.f. lagging 200 m from point A (ii) 200 A at 0·8 p.f. lagging 300 m from point A The load resistance and reactance of the distributor is 0·2 Ω and 0·1 Ω per kilometer. Calculate the total voltage drop in the distributor. The load power factors refer to the voltage at the far end. | CO1 | (10) |
|  | b) | With necessary diagram, explain D.C distributor with uniformly distributed loads fed at both end which are maintained at unequal voltages. Also derive the equation for voltage drop at different points for the chosen system. | CO1 | (10) |

****************