RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS – JULY/AUGUST 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Transmission and Distribution | Max. Marks | : | 100 |
| Subject Code | : | EE53/EE503 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT – I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Transmission line efficiency is higher for higher transmission voltages. Prove it. | CO1 | (08) |
|  | b) | Compare Underground and Overhead systems of power transmission. | CO1 | (06) |
|  | c) | What are the requirements of a good distribution System? | CO1 | (06) |
| 2. | a) | Derive an expression for the sag of an overhead line supported at equal levels and also mention the effect of atmospheric conditions on the sag. | CO2 | (10) |
|  | b) | Two towers of height 95m and 70m respectively support the line conductor at a river crossing. The horizontal distance between the towers is 400 m . If the tension in the conductor is 1100kg and the weight is 8 kg/m, calculate: - Sag at lower support - Sag at upper support - Clearance of lowest point on trajectory from water level Assume bases of towers to be at water level. | CO2 | (10) |
|  |  | UNIT – II |  |  |
| 3. | a) | Derive an expression for inductance of a conductor due to external flux. | CO3 | (10) |
|  | b) | A 200km ,3-phase transmission line has its conductors placed at corners of equilateral triangle of 2.5 m side. The radius of each conductor is 1cm. Calculate: - Line to neutral capacitance of line ii) Charging current per phase for voltage of 66 KV,50 Hz. | CO3 | (10) |
| 4. | a) | Derive an expression for line to line capacitance and line to neutral capacitance of a single phase line. | CO3 | (10) |
|  | b) | The three conductors of a three phase line are arranged at the three corners of a triangle of sides 2 m, 2.5 m, and 4.5 m. Calculate the inductance per km of the line when conductors are regularly transposed. The diameter of each line conductor is 1.24 cm. | CO3 | (10) |
|  |  | UNIT – III |  |  |
| 5. | a) | Derive an expression for voltage regulation and transmission efficiency of short transmission lines. | CO4 | (10) |
|  | b) | A 3 phase line delivers 5000KW at 22KV and at a p.f. of 0.8 lagging to a load determine i)sending end voltage ii)% Regulation iii) Transmission efficiency. The resistance and reactance of each conductor is 4Ω and 6Ω respectively. | CO4 | (10) |
| 6. | a) | Derive an expression for ABCD constants of a medium transmission lines using nominal T method. Show that AD-BC=1. | CO4 | (10) |
|  | b) | A single phase medium transmission line, 100km long, has the following constants: Resistance/km/phase=0.15Ω Inductive reactance/km/phase=0.377Ω Capacitive reactance/km/phase=31.87Ω Receiving end line voltage=132kV Assuming that the total capacitance of the line is localized at the receiving end alone determine: i)Sending end current ii)Line value of sending end voltage iii) Regulation iv) Sending end power factor The line is delivering 72MW at 0.8 p.f. lagging. | CO4 | (10) |
|  |  | UNIT- IV |  |  |

| 7. | a) | What are the methods of increasing the string efficiency? Explain any one method in detail. | CO5 | (10) |
| --- | --- | --- | --- | --- |
|  | b ) | Three bus bar conductors in outdoor systems are supported by the units of the post insulators. Each unit consists of string of three pin type insulators. The voltage across the line unit is 8.5 KV while voltage across the middle unit is 7.5 KV. Find the ratio of shunt to self capacitance and the bus bar voltage of the substation. | CO5 | (10) |
| 8 | a) | Derive an expression for capacitance of a single core cable. | CO5 | (10) |
|  | b) | A single core cable of conductor diameter 1.8 cm and lead sheath of diameter 5.4 cm is to be used on a 66 kV,3 phase system. Two inter sheaths of diameter 3 cm and 4.2 cm are used in between them to obtain uniform distribution of stress. Find the voltages at which two inter sheaths are maintained if maximum stress in the layers is same. | CO5 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Discuss briefly the classification of distributor. | CO1 | (10) |
|  | b) | An electric train taking a constant current of 500A moves between the two substations 6km apart. The two substations are maintained at 580 V and 600 V respectively. The track resistance is 0.05Ω/km both go and return. Calculate: - The point of minimum potential. - The current supplied by each substation at the point of minimum potential. | CO1 | (10) |
| 10. | a) | Write the note on radial and ring main distribution system. | CO1 | (06) |
|  | b) | What are the requirements of a good distributed system? | CO1 | (04) |
|  | c) | A two-wire d.c.distributor cable 1000 meters long is loaded with 0.5 amp/m. Resistance of each conductor is 0.05 Ω/km. calculate the maximum voltage drop if the distributer is fed from both ends with equal voltages of 220 V. What is the minimum voltage and where it occurs? | CO1 | (10) |

****************