RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS –  AUGUST 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Transmission and Distribution | Max. Marks | : | 100 |
| Subject Code | : | EE53/EE503 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss the need of high voltage in the transmission of power. | CO1 | (06) |
|  | b) | With neat sketch explain a typical transmission and distribution systems. | CO1 | (06) |
|  | c) | Briefly discuss HVDC transmission. | CO1 | (08) |
| 2. | a) | Briefly discuss the different types of supporting structures, which are used for overhead lines. | CO2 | (10) |
|  | b) | An overhead line is erected across a span of 250m on level supports. The conductor has a diameter of 1.42cm and has a dead weight of 1.09 kg/m. The line is subjected to a wind pressure of 37.8 kg/m2 of the projected area. The radial thickness of ice is 1.25cm. The line is carried by insulator string 1.43m long. Calculate (i) sag in an inclined direction (ii) sag in a vertical direction and (iii) the height of lowest cross arm to give a minimum ground clearance of 7.62 m under bad weather condi913.tions. Assume one cubic metre of ice weight 913.5 kg and a maximum stress of 1050 kg/cm2. | CO2 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | Derive expression for line-to-line capacitance and line to neutral capacitance of single-phase line. | CO3 | (10) |
|  | b) | Two conductors of a single phase line, each of 1 cm diameter are arranged in a vertical plane with one conductor mounted 1 m above the other. A second identical line is mounted at the same height as the first and spaced horizontally 0.25 m apart from it. The two upper and the two lower conductors are connected in parallel. Determine the inductance per km of the resulting double circuit line. | CO3 | (10) |
| 4. | a) | What is skin effect? On what factors do it depends. | CO3 | (05) |
|  | b) | A 3 phase, 50 Hz, 132 kV overhead line has conductors placed in a horizontal plane 4 m apart. Conductor diameter is 2 cm. If the length of line is 100 km, calculate the charging current per phase assuming complete transposition. | CO3 | (10) |
|  | c) | Write a note on stranded and bundled conductors. | CO3 | (05) |
|  |  | UNIT- III |  |  |
| 5. | a) | A 3-phase, 50Hz, 300km long transmission line has the following parameters: r=0.15 Ω/km; x=0.5 Ω/km; y=3x10-6 mho/km. If the line is represented by nominal – , determine i) The ABCD constants of the line (derive the related formula); ii) The power at unity power factor that can be received if the voltage at each end is maintained at 220kV iii) The sending-end voltage required if a load of 200 MW at 0.85 lagging power factor is to be delivered with the receiving-end voltage remaining at 220kV. | CO4 | (12) |
|  | b) | Discuss why: (i) Complex power in the transmission line is given by VI* (ii) Ferranti effect is significant in long and medium lines | CO4 | (08) |
| 6. | a) | Using rigorous method, derive an expressions for sending end voltage and current for a long transmission line. | CO4 | (10) |
|  | b) | A single phase overhead transmission line delivers 1100 kW at 11 kV at 0.8 p.f. lagging. The total resistance and inductive reactance of the line are 8Ω and 16Ω respectively. Determine i) sending end voltage ii) Sending end power factor iii) Transmission efficiency iv) Percentage regulation. | CO4 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain different types of insulators used in power systems. | CO5 | (08) |
|  | b) | In a string insulator having 5 units, the self-capacitance of insulator units is 6 times to that between a unit and earth. Obtain the potential distribution across each unit in the string as percentage of potential of conductor with respect to earth. Also find the string efficiency. | CO5 | (12) |
| 8. | a) | With a neat figure, describe general construction of the cable. | CO5 | (06) |
|  | b) | Derive expression for the electric stress in a single core cable. | CO5 | (06) |
|  | c) | A single core cable for use on 11kV, 50Hz system has conductor area of 0.645 cm2 and the internal diameter of sheath is 2.18cm. The permittivity of the dielectric used in the cable is 3.5. Find the maximum and minimum electrostatic stresses in the cable. Show also the curve of stress distribution in the cable v/s distance from the centre of the conductor. | CO5 | (08) |
|  |  | UNIT- V |  |  |
| 9. | a) | With the help of phasor diagram, discuss the AC distribution when power factors referred to respective load voltages. | CO1 | (06) |
|  | b) | A dc distributor cable shown in Fig. 9(b) is 1000m long and is loaded as under: Distance from the feeding point (metres) “A”; 250; 750; 1000. Load in amperes; 100; 200; 300. The resistance of each conductor is 0.025 Ω per 1000m. Find the voltage at each load point if the voltage at the feeding point A is maintained at 250V. Fig. 9(b) | CO1 | (08) |
|  | c) | Briefly discuss classification of distribution system. | CO1 | (06) |
| 10. | a) | A single phase a.c. distributor AB 300 metres long is fed from end A and is loaded as under: (i) 100 A at 0·707 p.f. lagging 200 m from point A (ii) 200 A at 0·8 p.f. lagging 300 m from point A The load resistance and reactance of the distributor is 0·2 Ω and 0·1 Ω per kilometer. Calculate the total voltage drop in the distributor. The load power factors refer to the voltage at the far end. | CO1 | (10) |
|  | b) | Two tram cars A and B 2 kms and 6 kms away from a substation, returns 40 A and 20 A respectively to the rail. The substation voltage is 600 volts d.c. The resistance of trolly wire is 0.25 ohms/km and that of track is 0.03 ohms per km. Calculate the voltage across each tram car. | CO1 | (10) |

***************