RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

EXAMINATIONS SEPTEMBER /OCTOBER 2020

SUPPLEMENTARY SEMESTER / GRADE IMPROVEMENT/ RE –REGISTERED CANDIDATES

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | V |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Transmission and Distribution | Max. Marks | : | 100 |
| Course Code | : | EE53 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss the effect of increase in voltage on transmission line efficiency | CO1 | (04) |
|  | b) | Compare and contrast EHVAC and HVDC transmission. | CO1 | (06) |
|  | c) | An overhead line has a span of 150 m between level supports. The conductor has a cross sectional area of 2 cm2. The ultimate strength is 5000 kg/cm2 and factor of safety is 5. The specific gravity of the material is 8.9 gm/cc. The wind pressure is 1.5 kg/m. Calculate the height of the conductor above the ground level at which it should be supported if a minimum clearance of 7 m is to be left between the ground and the conductor. | CO2 | (10) |
| 2. | a) | Draw single line diagram of a typical power system. Locate various sections of it and mention there on typical voltage of generation, transmission and distribution. | CO1 | (05) |
|  | b) | Derive an expression for the sag of an overhead line supported at same levels. State also the effects of wind and ice coating on the sag. | CO2 | (10) |
|  | c) | Given that weight of conductor = 0.35 kg/m, maximum allowable strength = 800 kg, span length = 160 m and safety factor = 2, find the minimum point of catenary if supports are at 70m and 65 m. | CO2 | (05) |
|  |  | UNIT- II |  |  |
| 3. | a) | i) Distinguish between ac and dc resistances of a conductor. Why the two differ? Ii) Derive an expression for internal flux linkage due to a single current carrying conductor. | CO3 | (10) |
|  | b) | Find the capacitance of a single phase line 40 km long consisting of 2 parallel wires each 5 mm diameter and 1.5 m apart. Determine the capacitance of the line taking into account the effect of ground. The height of the conductors above the ground is 7 m. | CO3 | (10) |
| 4. | a) | What do you mean by bundled conductor? Discuss the advantages of bundled conductor over single conductor? | CO3 | (07) |
|  | b) | Why do you find line to neutral capacitance in a three phase system? Explain. | CO3 | (05) |
|  | c) | A single phase overhead line 30 km long consist of two parallel wires each 5 mm in diameter and 1.5 m apart. If the line voltage be 50 kV at 50 Hz, calculate the charging current with the line open circuited. | CO3 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | With circuit representation, explain the basis of categorization of short, medium and long transmission lines. | CO4 | (08) |
|  | b) | Obtain the expression of voltage regulation and efficiency of a short transmission line in terms of line parameters. | CO4 | (12) |
| 6. | a) | A 220kV, 3-phase transmission line is 40km long. The resistance per phase is 0.15Ω per km and the inductance per phase is 1.5923 mH per km. The line is supplying a 3-phase load of 381 MVA at 0.8 pf lagging. Find: (i) the voltage and power factor at the sending end of the line (ii) the voltage regulation and efficiency of the line. | CO4 | (10) |
|  | b) | Derive A, B, C, D constants of a medium length transmission line and hence prove that AD – BC = 1. | CO4 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain different types of insulators used in power systems. | CO5 | (08) |
|  | b) | In a string insulator having 5 units, the self-capacitance of insulator units is 6 times to that between a unit and earth. Obtain the potential distribution across each unit in the string as percentage of potential of conductor with respect to earth. Also find the string efficiency. | CO5 | (12) |
| 8. | a) | With a neat figure, describe general construction of the cable. | CO5 | (06) |
|  | b) | Derive an expression for the electric stress in a single core cable. | CO5 | (06) |
|  | c) | A single core cable for use on 11kV, 50Hz system has conductor area of 0.645 cm2 and the internal diameter of sheath is 2.18cm. The permittivity of the dielectric used in the cable is 3.5. Find the maximum and minimum electrostatic stresses in the cable. Show also the curve of stress distribution in the cable v/s distance from the centre of the conductor. | CO5 | (08) |
|  |  | UNIT- V |  |  |
| 9 | a) | Bring out the necessary requirement of a good distribution system and hence classify distribution system. | CO1 | (10) |
|  | b) | What are the additional factors which need to be kept in mind when performing a.c. distribution calculations as compared to D.C. distribution. | CO1 | (04) |
|  | c) | A two wire ring distributor is 300 m long and is fed at 240V at point A. At point B, 150 m from A, a load of 120A is taken and at C, 100 m in opposite direction, a load of 80A is taken. If the resistance per 100 m of a single conductor is 0.02 Ω, find i) current in each section of distributor ii) voltage at point B and C. | CO1 | (06) |
| 10 | a) | With suitable sketch explain the following system of distribution: i) Radial system ii) Rind main system iii) Interconnected system. | CO1 | (06) |
|  | b) | A single phase ring distributor ABC is fed at A. The load at B and C are 18A at 0.8 p.f lagging and 12A at 0.6 p.f lagging respectively, both expressed with reference to voltage at A. The total impedance of the section AB, BC and CA are (2+j2), (2 + j1) and (1+j3) ohms respectively. Find the current in each section and total current fed at A | CO1 | (10) |
|  | c) | Explain the use of balancer in 3 wire d.c. distribution system. | CO1 | (04) |

****************