RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS –  JULY 2019

| Course & Branch | : | B.E. : Electrical and Electronics Engineering | Semester | : | IV |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Signals and Systems | Max. Marks | : | 100 |
| Subject Code | : | EE43/EE403 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Sketch the signals: i) x(t)=r(t+1)-r(t)+r(t-2) ii) x(t)= -u(t+3)+2u(t+1)-2u(t-1)+u(t-3) where r(t) and u(t) represents unit ramp and unit step function respectively. | CO1 | (08) |
|  | b) | Determine whether the systems i) ii) are linear, time invariant, causal, memory less and stable. | CO1 | (08) |
|  | c) | Differentiate between power and energy signal with necessary examples. | CO1 | (04) |
| 2. | a) | Define even and odd signals. Find odd and even parts of x[t].x[t]=1+t+3t2+5t3+9t4. | CO1 | (06) |
|  | b) | Briefly explain the properties of a system. Given (i) = (ii) = . Determine the properties of the system. | CO1 | (08) |
|  | c) | i) Determine the fundamental period of the signal ii) The sinusoidal signal x(n) has fundamental period N=10 samples. Determine the smallest angular frequency Ω for which x│n│is periodic. | CO1 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | Given x(t)= e-at ;0< t < T and h(t) = 1 ; 0< t < 2T. Obtain the convolution integral graphically. | CO2 | (10) |
|  | b) | Compute the convolution of two sequences x1(n) and x2(n), given below. | CO2 | (10) |
| 4. | a) | Determine the response of the LTI system whose input x(n) and impulse response h(n)are given by x(n)=3nu(-n) and h(n)=u(n). Plot y(n). | CO2 | (10) |
|  | b) | Compute the convolution of two sequences x(t) and h(t), given below X(t)=u(t-1) – u(t-3); h(t)= u(t)-u(t-2) | CO2 | (10) |
|  |  | UNIT- III |  |  |
| 5. | a) | For the system given below, find the total response: Y(n) =5/6 Y(n-1)-1/6 Y(n-2)+x[n]-x[n-1] where x[n]=2nu[n] Y(-1)= 0, Y(-2)= 1. | CO4 | (12) |
|  | b) | Draw direct form I and direct form II structure of the following: | CO4 | (08) |
| 6. | a) | Determine the complete response of the systems: | CO4 | (12) |
|  | b) | Write and explain sampling theorem. | CO2 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Obtain DTFS coefficients of . Plot its magnitude and phase spectrum. | CO4 | (10) |
|  | b) | Explain sampling theorem. Determine the Nyquist sampling frequency and Nyquist interval for the signal | CO4 | (10) |
| 8. | a) | Find the frequency response of the system described by the difference equation | CO4 | (08) |
|  | b) | Determine the DTFT of the signal: . | CO4 | (06) |
|  | c) | Determine the Fourier series representation of the signal: | CO4 | (06) |
|  |  | UNIT- V |  |  |
| 9. | a) | Find the transfer function and impulse response of a causal LTI system if the input to the system is x(n)= (-1/3)n u(n) and the output is y(n)= 3(-1)nu(n) + (1/3)nu(n). | CO5 | (12) |
|  | b) | Determine the ROC and Z transform of the signal x(n)= (1/2)nu(n-2). Sketch the ROC, poles and zeros in the Z plane. | CO5 | (08) |
| 10. | a) | Solve the following difference equation using unilateral Z Transform y(n)-1/9 y(n-2)= x(n-1) with initial conditions y(-1)=1,and y(-2) =0, x(n)= 2u(n). | CO5 | (12) |
|  | b) | Determine the z transform and ROC of the following signals. - X(n)=u(n) ii) x(n)= (1/4)nu(-n) | CO5 | (08) |

****************