RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

MAKEUP EXAMINATIONS – MAY/JUNE 2017

| Course & Branch | : | B.E: Electrical & Electronics Engineering | Semester | : | IV |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Signals and Systems | Max. Marks | : | 100 |
| Subject Code | : | EE43 / EE403 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Given the signal x(t) as shown in Fig.1(a), sketch x(-2t+3) and x(t/2 – 2) Fig.1(a) | CO1 | (06) |
|  | b) | Determine whether the system y(n)=x(n)+3u(n+1) isLinear, time invariant, Causal and, Memoryless. | CO1 | (04) |
|  | c) | A trapezoidal pulse, x(t) defined by x(t)= { t+5, - 5 {0, otherwise. Is the signal power or energy? Find power or energy. Also find the even and odd components of x (t). | CO1 | (10) |
| 2. | a) | Define even and odd signals. Sketch x[n] and its odd and even parts where x[n]= 8(0.5)nu(n). | CO1 | (08) |
|  | b) | i) Given y(t)= x2(t), Check for periodicity. ii) Given y(t) = x(t) + 2 x (3-t), check for causality. Identify the value of t, if any, where the system will change its status from causal to non-causal. | CO1 | (06) |
|  | c) | Determine whether the signal shown in the Fig. 2(c) is a power signal or energy signal. Justify your answer and further determine its energy or power. Fig. 2(c) | CO1 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | Compute y(n)=h(n)*x(n) for h(n)= (1/2)n u[n] and x(n)= 2n u[-n]. | CO3 | (10) |
|  | b) | Convolute graphically x(t)& h(t) where x(t)=u(t-1)&h(t)=u(t-2) and plot the output . | CO3 | (10) |
| 4. | a) | Convolute the two continuous time signals x(t)= u(t) and h(t)= t u(-t) | CO3 | (10) |
|  | b) | Compute the convolution of two sequences x1(n) and x2(n), given below x1(n)={ 1,2,3} and x2(n)={ 1,2,3,4}. | CO3 | (10) |
|  |  | UNIT- III |  |  |
| 5. | a) | Find the forced response of the system given by the differential equation d2y(t)/dt2+9y(t) = x(t), where x(t)=cos4t | CO3 | (10) |
|  | b) | For the system given below, find the total response: Y(n+2) + 4Y(n+1) + 4 Y(n) = 2nu[n] Y(-1)= 0, Y(-2)= 1. | CO3 | (10) |
| 6. | a) | Define natural and force response of an LTI system. Find the forced response of the system given by the differential equation 2y(t)+d2y(t)/dt2+3dy(t)/dt=x(t)+dx(t)/dt with input x(t)=5U(t) | CO3 | (10) |
|  | b) | Draw the direct form I and II implementations of the system represented by the differential equation / difference equation - 3y(t)+2 dy(t)/dt+d3y(t)/dt3 = x(t) +3dx(t)/dt - y(n) - (1/4 )y (n-1) +(1/8 ) y(n-2) = x(n) + (1/2 ) x(n-2) | CO3 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Determine the DTFS of the signal x(n)= cos(π/3)n and draw the spectrum. | CO4 | (08) |
|  | b) | Write the modulation and convolution properties of DTFS and DTFT | CO4 | (04) |
|  | c) | Explain the time shift and linearity properties of DTFT and solve using the properties. | CO4 | (08) |
| 8. | a) | Determine the response of the LTI system whose input x(n) and impulse response h(n)are given by x(n)={1,2,3,1} and h(n)={ 1,2,1,-1} using Discrete Time Fourier Transform. | CO4 | (10) |
|  | b) | Determine the DTFT of the following signals i) ii) | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Find the inverse Z transform of: ROC : ½ <¦Z¦<∞ | CO5 | (10) |
|  | b) | Find the Z transform of x[n]=sin(Ω0n) u(n). | CO5 | (05) |
|  | c) | Write the properties of ROC. | CO5 | (05) |
| 10. | a) | Use the Z transform to perform the convolution of the following two sequences. h(n) = | CO5 | (10) |
|  | b) | Find the inverse Z transform of X(Z)= ROC ¦Z¦>1 | CO5 | (10) |

****************