RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – JUNE/JULY 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engg. | Semester | : | IV |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Signals and Systems | Max. Marks | : | 100 |
| Subject Code | : | EE43/EE403 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Let x(n) = 3n [ u(n-1) – u(n-4)], sketch the following signals. - Y1(n)= x(n-3), - Y2(n)= x(n+1) - Y3(n)= x(-n+4) - Y4(n)= x(-n-2). - Y5(n)=x(n/2-1) | CO1 | (10) |
|  | b) | Consider an LTI system with h(n)=(1/3)n u(n-1), - Is the system Causal? - Is the system stable? - Is the system memoryless? Justify the answers. | CO1 | (05) |
|  | c) | Sketch x(t) and its odd and even parts x(t)=-(t+2)u(t+2)+u(t)+4u(t)-u(t-2) | CO1 | (05) |
| 2. | a) | Determine whether each of the following signals are periodic. If periodic, find its fundamental period. - x(n)= 5 cos(0.2πn) - x(n) = sin(2n).. | CO1 | (10) |
|  | b) | A trapezoidal pulse, x(t) defined by: { 5-t, 4 x(t)= { t+5, - 5 { 0, otherwise, is the signal power or energy? Find power or energy? | CO1 | (05) |
|  | c) | Find the even and odd part of the following signals: x(n)=u(n) | CO1 | (05) |
|  |  | UNIT- II |  |  |
| 3. | a) | Convolute graphically x(t)& h(t) given in Fig.3(a) and hence plot the output signal. Fig.3(a) | CO2 | (10) |
|  | b) | Determine the output of a discrete LTI system if h(n)=u(n-3) and x(n)=u(n) | CO2 | (10) |
| 4. | a) | Given h(t)=u(t)- u(t-3) and x(t)=u(t)-u(t-1). Obtain and plot the convolution integral graphically. | CO2 | (10) |
|  | b) | Determine and plot the response of the LTI system whose input x(n) and impulse response h(n)are given by x(n)=u(n)- u(n-4) and h(n)=2n{u(n)-u(n-5)}. | CO2 | (10) |
|  |  | UNIT- III |  |  |
| 5. | a) | Define natural and force response of an LTI system. Find the forced response of the system given by the differential equation | CO3 | (10) |
|  | b) | Determine the step response of a relaxed system whose input-output relationship is given by y(n) + 4y (n-1) +4y(n-2) = x(n). | CO3 | (10) |
| 6. | a) | Find the natural response, forced response and complete response of the system given by the differential equation | CO3 | (12) |
|  | b) | Obtain the direct form-I and form-II implementation of the system y(n) - (1/2 )y (n-1) +(1/6 ) y(n-2) = x(n) + 2x(n-2). | CO3 | (08) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Obtain DTFS coefficients of. Plot its magnitude and phase spectrum. | CO4 | (10) |
|  | b) | List out the properties of DTFT. Also compute DTFT of the signal x(n)=u(n)- u(n-6). | CO4 | (10) |
| 8. | a) | The impulse response of an LTI system is given by h(n)= 0.6n u(n). Find the frequency response. | CO4 | (12) |
|  | b) | Determine the DTFT of the signal . | CO4 | (08) |
|  |  | UNIT- V |  |  |
| 9. | a) | Find the inverse Z transform of X(Z)= ROC : ½ <¦Z¦<∞ | CO5 | (10) |
|  | b) | Find the Z transform of x[n]=sin(Ω0n) u(n). | CO5 | (05) |
|  | c) | Write the properties of ROC. | CO5 | (05) |
| 10. | a) | Use the Z transform to perform the convolution of the following two sequences. h(n) = | CO5 | (10) |
|  | b) | Solve the following difference equation using unilateral Z Transform with initial conditions y(-1)=4,and y(-2) = 10. | CO5 | (10) |

****************