| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

M S RAMAIAH INSTITUTE OF TECHNOLOGY

(AUTONOMOUS INSTITUTE, AFFILIATED TO VTU)

BANGALORE – 560 054

SEMESTER END EXAMINATIONS – MAY/JUNE 2016

| Course & Branch | : | B.E. – Electrical & Electronics Engg. | Semester | : | IV |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Signals and Systems | Max. Marks | : | 100 |
| Subject Code | : | EE403 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  |  |  |  | UNIT – I |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. |  | a) |  | Let x(t) and y(t) be given as in fig. 1(a) and fig. 1(b). Sketch the following signals. - x(t)y(t-1), - x(t-1)y(-t), - z(t)=x(t)[ᵟ(t-1)+ ᵟ(t-2)]+y(t) ᵟ(t+2) Fig 1 (a) fig 1(b) |  | CO1 |  |  |  | (09) |  |
|  |  | b) |  | i) Given y(t)= x2(t), Check for periodicity. ii) Given y(t) = x(t) + 2 x(3-t), check for causality. Identify the value of t, if any, where the system will change its status from causal to non-causal. |  | CO1 |  |  |  | (06) |  |
|  |  | c) |  | Sketch x[n] and its odd and even parts for the signal given. X[n]= 8(0.5)nu(n) |  | CO1 |  |  |  | (05) |  |
| 2. |  | a) |  | Plot the signal x(t) = u(t) - r(t-1) + r(t-2) – r(t-3) – u(t-4). State whether this signal is power or energy signal? Find the corresponding energy or power as the case may be. |  | CO1 |  |  |  | (08) |  |
|  |  | b) |  | Find the even and odd part of the following signals shown in fig 2 a): fig 2 a) |  | CO1 |  |  |  | (04) |  |
|  |  | c) |  | Distinguish between i) Continuous time and Discrete time signals ii) Even and Odd signals iii) Periodic and Non-periodic signals iv) Energy and Power signals with examples. |  | CO1 |  |  |  | (08) |  |
|  |  |  |  |  | UNIT - II |  |  |  |  |  |  |
| 3. |  | a) |  | Find the convolution of the two continuous time signals given below using graphical method. X1(t) =(t+2t2)[u(t+1)-u(t-1)]. X2(t) = 2u(t+2) |  | CO2 |  |  |  | (10) |  |
|  |  | b) |  | Determine the overall impulse response of the system consisting of cascade of two systems with impulse response h1(n)=(0.5)nu(n), h2(n)=(1/4)n u(n) |  | CO2 |  |  |  | (06) |  |
|  |  | c) |  | Determine the status of the DTS characterised by the impulse response h(n)=4-n u(2-n) on stability, causality and memory. |  | CO1 |  |  |  | (04) |  |
| 4. |  | a) |  | Compute y(n)=h(n)*x(n) for h(n)= a1n u[n] and x(n)= a2nu[-n] where 0<a1<1 and -1<a2<0. |  | CO2 |  |  |  | (08) |  |
|  |  | b) |  | Plot the output response y(t) given the input and impulse response of the LTI system as x(t)=u(t+2), h(t)=u(t-3). |  | CO2 |  |  |  | (08) |  |
|  |  | c) |  | Determine the status of the DTS characterised by the impulse response h(n)=(1/2)n u(-n-1) on stability, causality and memory. |  | CO1 |  |  |  | (04) |  |
|  |  |  |  |  | UNIT -III |  |  |  |  |  |  |
| 5. |  | a) |  | For the system given below, find the total response. Y(n+2) + 4Y(n+1) + 4 Y(n) = 2nu[n] Y(-1)= 0, Y(-2)= 1. |  | CO3 |  |  |  | (08) |  |
|  |  | b) |  | State and explain sampling theorem. |  | CO4 |  |  |  | (04) |  |
|  |  | c) |  | Determine the forced response of the system described by the following difference equation for the given input. Y(n)- 2/5 y(n-1)= 2x(n); x(n)=u(n) |  | CO3 |  |  |  | (08) |  |
| 6. |  | a) |  | The continuous – time system shown in fig 6 a) consist of two integrators and two scalar multipliers. Write a differential equation that relates the output y(t) to the input x(t). Then draw direct form I and direct form II structure. Fig 6 (a) |  | CO3 |  |  |  | (12) |  |
|  |  | b) |  | Consider a discrete time system whose input x[n] and output y[n] are related by the fig 6 b), where ‘a’ is a constant. Find y[n] with the auxiliary condition y(-1)=y-1 and x[n] = k bn U[n], y[n]= yp[n] +yh[n] where yp[n] is the particular solution and yh[n] is the homogeneous solution. fig 6 b) |  | CO3 |  |  |  | (08) |  |
|  |  |  | UNIT IV |  |  |  |  |  |  |  |  |
| 7. | a) |  | Using convolution theorem , find the inverse DTFT of X(ejΩ)= 1 / (1- a e-jΩ )2, ¦a¦<1. |  |  |  | C04 |  | (10) |  |  |
|  | b) |  | State and prove the following properties of DTFS. - Frequency shift - Parseval’s property |  |  |  | C04 |  | (10) |  |  |
| 8. | a) |  | Determine the: - Fourier coefficients for the periodic sequence shown in fig 8 a) - Verify Parseval’s identity. |  |  |  | C04 |  | (10) |  |  |
|  | b) |  | State and prove the following properties of DTFT. - Duality - Time shift. |  |  |  | C04 |  | (10) |  |  |
|  |  |  | UNIT V |  |  |  |  |  |  |  |  |
| 9. | a) |  | Find the inverse Z transform of: X(Z)= ROC : ½ <¦Z¦<∞ |  |  |  | C05 |  | (10) |  |  |
|  | b) |  | Find the Z transform of x[n]=sin(Ω0n) u(n). |  |  |  | C05 |  | (05) |  |  |
|  | c) |  | Write the properties of ROC. |  |  |  | C05 |  | (05) |  |  |
| 10. | a) |  | Use the Z transform to perform the convolution of the following two sequences. h(n) = |  |  |  | C05 |  | (10) |  |  |
|  | b) |  | Find the inverse Z transform of X(Z)=, ROC ¦Z¦>1 |  |  |  | C05 |  | (10) |  |  |

**********************