RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Digital Image Processing | Max. Marks | : | 100 |
| Subject Code | : | EEPE49 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Let p and q be the pixels at co ordinates as shown in the image segment 2a) respectively. Find out which distance measure gives the minimum distance between pixels. | CO1 | (04) |
|  | b) | Explain the procedure of labeling 4 and 8 connected components. | CO1 | (04) |
|  | c) | Write short notes on sampling and quantization. | CO1 | (06) |
|  | d) | Write different data types and their description and range. |  | (06) |
| 2. | a) | For v={ 1 2 3} compute the lengths of shortest 4, 8, m paths between p and q. 3; 4; 1; 2; 0. 0; 1; 0; 4; 2 q. 4; 2; 3; 1; 4. P 3; 0; 3; 2; 1. 1; 2; 0; 3; 4. | CO1 | (06) |
|  | b) | Explain the basic relationships between pixels? | CO1 | (06) |
|  | c) | Write a note on at least eight image files and formats. | CO1 | (08) |
|  |  | UNIT- II |  |  |
| 3. | a) | With the objectives of an enhancement, interpret the following point processing techniques of image enhancement: (i)Power law transformation (ii) Gray level slicing (iii) Image negatives | CO2 | (12) |
|  | b) | What is meant by image histogram? Derive an equation for performing histogram linearization of an image | CO2 | (08) |
| 4. | a) | A 4 x 4 , 4bits/pixel image is given by: 11; 13; 5; 7. 0; 2; 8; 12. 4; 14; 1; 3. 8; 6; 3; 10. Sketch (i) its four bit planes (ii) binary image with threshold , T=7. | CO2 | (10) |
|  | b) | With suitable mathematical expressions, explain the following using Arithmetic/Logic operations:(i) Image averaging (ii) Image subtraction. | CO2 | (10) |
|  |  | UNIT- III |  |  |
| 5. | a) | Define image smoothing? Describe the low pass filtering techniques of image smoothing in spatial domain. | CO3 | (10) |
|  | b) | Discuss characteristics of high boost filter in frequency domain. Explain how high boost filtering increases the enhancement of the images. | CO3 | (10) |
| 6. | a) | With suitable sketches, relate the different high pass filters used for image sharpening in frequnecy domain. | CO3 | (10) |
|  | b) | Define homomorphic filtering. Explain the utility of this when a multiplicative noise is present in the input. | CO3 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Write the application of image segmentation. | CO4 | (10) |
|  | b) | For the given 8×8 image in fig. 7(b), implement image segmentation using region growing technique based on Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value i) th= 3 , ii) th=4 and Seed points are pixel P(4,6) and pixel q(5,4). 5; 6; 6; 6; 7; 7; 6; 6. 6; 7; 6; 7; 5; 5; 4; 7. 6; 6; 4; 4; 3; 3; 5; 6. 5; 4; 3; 2; 7; 5; 4; 3. 5; 4; 4; 3; 2; 1; 0; 2. 1; 2; 2; 3; 3; 4; 4; 5. 1; 0; 0; 4; 4; 5; 5; 6. 7; 7; 6; 6; 5; 5; 4; 4. | CO4 | (10) |
|  |  | fig: 7(b) |  |  |
| 8. | a) | Explain in detail the edge based segmentation. | CO4 | (10) |
|  | b) | For the given 8×8 image in fig 7b), implement image segmentation using region merging technique based on Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value i) th= 3 , ii) th=2. | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Let f (x, y) denote an N * N -point 2-D sequence, that has zero value outside 0 <= x, y <= N -1, where N is an integer power of 2. In implementing the 2-D Discrete Hadamard Transform of f (x, y) , we relate f (x, y) to a new N * N -point 2-D sequence H(u,v) . Define the sequence H(u,v). | CO5 | (10) |
|  | b) | Discuss the properties and applications of: - Haar transform ii) Discrete Cosine Transforms. | CO5 | (10) |
| 10. | a) | What is Image Transform? What are the applications of transform? | CO5 | (06) |
|  | b) | Write the Hadamard transform matrix Hn for n=3 | CO5 | (06) |
|  | c) | A= Find the transformed image V. Calculate Basis images. Find inverse transformed image U using inverse Transform formula and using basis images. | CO5 | (08) |

****************