RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Digital Image Processing | Max. Marks | : | 100 |
| Subject Code | : | EEE24 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | With suitable examples, explain components of an image processing system. | CO1 | (08) |
|  | b) | Discuss the following basic transformations as applied to digital images: (i) Translation (ii) Rotation. | CO1 | (08) |
|  | c) | Consider the two image subset s1 and s2 shown in the following fig.1(c). For V={1}, Find if two subsets s1 and s2 are (i) 4-adjacent (ii) 8-adjacent (iii) m-adjacent. | CO1 | (04) |
| 2. | a) | Define and illustrate with suitable example, sampling and Quantization | CO1 | (08) |
|  | b) | Write short note on: - False contouring (ii) Pseudo colouring iii. True colouring. | CO1 | (08) |
|  | c) | Let p and q be the pixels at co-ordinates (3,4) and (5,15) respectively, find out which distance measure gives the minimum distance between pixels. | CO1 | (04) |
|  |  | UNIT- II |  |  |
| 3. | a) | Explain in details any two type of image enhancement techniques. | CO2 | (08) |
|  | b) | Consider the following image shown in Fig.3(b), having grey value between 0 to 15. Stretch the grey values uniformly and linearly in the entire range. 10; 11; 12; 11; 10. 12; 12; 13; 5; 4. 13; 12; 5; 3; 5. 13; 12; 4; 3; 5. 12; 4; 3; 4; 4. | CO2 | (06) |
|  |  | fig: 3(b) |  |  |
|  | c) | Write a note on colour enhancement method. | CO2 | (06) |
| 4. | a) | Perform law transformation on the given input value for the image shown in Fig.4(c) , for γ=2.5 and γ=0.4 and comment on compression and expansion observation. | CO2 | (08) |
|  | b) | What is the importance of image enhancement in image processing? Explain in brief any one point processing techniques implemented in image processing. | CO2 | (06) |
|  | c) | Do the transformation g1(v)=v+1, g2(v)=v2 and g3(v) = negative transformation of the image f(x,y) shown in Fig. 4(c). 0; 10; 50; 100. 5; 95; 150; 200. 110; 150; 190; 210. 175; 210; 255; 100. | CO2 | (06) |
|  |  | Fig. 4(c) |  |  |
|  |  | UNIT- III |  |  |
| 5. | a) | Write the steps involved in frequency domain smoothing filtering with the help of block diagram. | CO3 | (06) |
|  | b) | For the pixel (4,4) on image segment , shown in fig.5(b) apply i) smoothing filter, ii) weighted average filter, iii) median filter, iv) min filter, v) max filter, vi) midpoint filter, of size 3×3 and 5×5 and laplacian filter only for (3x3) (4 connectivity and 8 connectivity) 0; 1; 1; 0; 0; 1; 1; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 7; 7; 2; 1; 0. 0; 1; 2; 6; 6; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 1; 1; 0; 0; 1; 1; 0. Fig.5(b) | CO3 | (14) |
| 6. | a) | With the help of block diagram, explain the Homomorphic filtering approach for image enhancement. What are the advantages of this filters. | CO3 | (10) |
|  | b) | For the image segment shown in Fig. 6(b) apply i) smoothing filter, ii) weighted average filter, iii) median filter, iv) min filter, v) max filter, vi) midpoint filter, vii) laplacian filter ( 4 connectivity & 8 connectivity) of size 3×3 on the pixel value 10. 0; 10; 5; 100. 5; 95; 150; 20. 110; 50; 190; 210. 175; 210; 255; 110. | CO3 | (10) |
|  |  | Fig. 6(b) |  |  |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain image segmentation techniques in details. | CO4 | (10) |
|  | b) | For the given 8×8 image in fig. 5(b), implement image segmentation using region splitting technique based on: Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value - th=3, ii)th=2. iii) th=4. | CO4 | (10) |
| 8. | a) | Write in details threshold based segmentation. | CO4 | (10) |
|  | b) | For the given 8×8 image in fig. 5(b), implement image segmentation using region merging technique based on Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value i) th= 4 , ii) th=2. | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | U=[1 1 1 1 1 1 1 1 1 1 1 1 1 1 1 1], find forward and inverse hadmard transform. | CO5 | (10) |
|  | b) | Find 2×2 Haar matrix H2. Then find the forward Haar transform for u=[1 1 1 1]. | CO5 | (10) |
| 10. | a) | Write the properties of DFT, DCT and DST. | CO5 | (08) |
|  | b) | Find the transform matrix & KL transform of u=[ 4 -2 -1 3] | CO5 | (12) |

**************