RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2020

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Digital Image Processing | Max. Marks | : | 100 |
| Course Code | : | EEE24 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Mention the components of the image function f(x,y). Also elaborate the role of the following steps in digital image processing system: Image acquisition, image enhancement, image segmentation and image recognition. | CO1 | (08) |
|  | b) | Discuss the elements of visual perception. | CO1 | (06) |
|  | c) | Consider the two image subset s1 and s2 shown in the following figure.1(c). For V={1}, Find if two subsets s1 and s2 are (i) 4-adjacent (ii) 8-adjacent (iii) m-adjacent. | CO1 | (06) |
| 2. | a) | Define and interpret with suitable example, sampling and Quantization. | CO1 | (10) |
|  | b) | With suitable illustrations, discuss the neighbors of a pixel and the distance measures as applied to the basic relationships between the pixels. | CO1 | (08) |
|  | c) | What is the storage space required for 456*518, 120-gray level image? | CO1 | (02) |
|  |  | UNIT- II |  |  |
| 3. | a) | What is image enhancement? Discuss the point processing techniques of image enhancement: (i) Contrast Stretching (ii) Gray level Slicing (iii) Histogram equalization. | CO2 | (12) |
|  | b) | With the principle of Pseudo colour image processing, explain the intensity slicing technique of the same. | CO2 | (08) |
| 4. | a) | Analyse i) Image negative ii) Histogram sliding iii) Histogram operation on the image segment f(x,y). f(x,y)=1105010012150160250 | CO2 | (08) |
|  | b) | Discuss on log transformation with respect to f(x,y). | CO2 | (04) |
|  | c) | Analyse power law transform operation on the segment f(x,y). | CO2 | (08) |
|  |  | UNIT- III |  |  |
| 5. | a) | Write the steps involved in frequency domain smoothing filtering with the help of block diagram. | CO3 | (06) |
|  | b) | Apply spatial domain filtering for the pixel (4,4) using 3 X 3 mask . andrepeat for 5 X 5 mask except laplacian filter. 0; 1; 1; 0; 0; 1; 1; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 7; 7; 2; 1; 0. 0; 1; 2; 6; 6; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 1; 1; 0; 0; 1; 1; 0. | CO3 | (14) |
| 6. | a) | With the help of block diagram, explain the Homomorphic filtering approach for image enhancement. What are the advantages of this filters. | CO3 | (10) |
|  | b) | Apply spatial domain filtering for the pixel value 120 using 3X 3 mask. 0; 10; 5; 100. 5; 95; 150; 20. 120; 50; 190; 210. 175; 210; 255; 110. | CO3 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain image segmentation techniques in details . | CO4 | (10) |
|  | b) | Apply denoising using restoration method using 3X3 mask on the pixel value 12. 0; 1; 1; 0; 0; 1; 1; 0. 0; 1; 12; 3; 3; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 7; 7; 2; 1; 0. 0; 1; 2; 6; 6; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 1; 1; 0; 0; 1; 1; 0. | CO4 | (10) |
| 8. | a) | Write in details threshold based segmentation. | CO4 | (10) |
|  | b) | For the given 8×8 image given below, implement image segmentation using region merging technique based on Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value i) th= 4 , ii) th=2. 0; 1; 1; 0; 0; 1; 1; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 7; 7; 2; 1; 0. 0; 1; 2; 6; 6; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 1; 1; 0; 0; 1; 1; 0. | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | U=ones(4,4); find forward and inverse hadmard transform. | CO5 | (10) |
|  | b) | For the input image u=ones(8,8), find the transformed image V using Haar transform. | CO5 | (10) |
| 10. | a) | Given A=sqrt(1/2)*[1 1;1 -1] and u=[1 2; 3 4], find the transformed image V. Calculate Basis images. | CO5 | (06) |
|  | b) | Find the transform matrix & KL transform of u=[ 4 -2;-1 3]; | CO5 | (14) |

****************