RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKE UP EXAMINATIONS –  APRIL 2021

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Digital Image Processing | Max. Marks | : | 100 |
| Course Code | : | EEE24 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Explain the fundamental steps involved in Digital image processing. | CO1 | (10) |
|  | b) | A medical image has a size of 6×8 cms. The sampling resolution is 2 cycles/ inches . How many pixel are required? Will an image of size 256×256 be enough? | CO1 | (06) |
|  | c) | Given A= zeros(4,4); A(2,3)=1; Find different distances from the given point to all the pixel present in A. | CO1 | (04) |
| 2. | a) | What is image quantization? How it is achieved under grey level reduction and spatial reduction. | CO1 | (10) |
|  | b) | For v={1,2,3}, compute the lengths of shortest 4, 8 , m paths between p(7,1) and q(0,7) in the following image. And find distances between the two points using three different methods. 0; 1; 1; 0; 0; 1; 1; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 7; 7; 2; 1; 0. 0; 1; 2; 6; 6; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 1; 1; 0; 0; 1; 1; 0. | CO1 | (06) |
|  | c) | An image in 1800 pixels wide and 2000 pixels high. The image was scanned at 250 DPI . What is the physical size of the image? | CO1 | (04) |
|  |  | UNIT – II |  |  |
| 3. | a) | Explain in details any two type of image enhancement techniques. | CO2 | (08) |
|  | b) | Consider the following image shown in image 3b), having grey value between 2 to 15. Stretch the grey values uniformly and linearly in the entire range. 10; 11; 12; 11; 10. 12; 12; 13; 5; 4. 13; 12; 5; 3; 5. 13; 12; 4; 3; 5. 12; 4; 3; 4; 4. fig: image 3b) | CO2 | (06) |
|  | c) | Write a note on colour enhancement method. | CO2 | (06) |
| 4. | a) | Perform law transformation on the given input value shown in image 4b), for γ=2.5 and γ=0.4 and comment on compression and expansion observation. | CO2 | (08) |
|  | b) | For the given image segment find the following : Fig: 4(b) image segment =im 200; 10; 50; 100. 5; 95; 150; 200. 110; 15; 190; 10. 175; 210; 255; 100. - Imdivide(Immultiply(im,5),6)+50 - Imcomplement(im) | CO2 | (08) |
|  | c) | Distinguish between enhancement and restoration method. | CO2 | (04) |
|  |  | UNIT – III |  |  |
| 5. | a) | Explain different frequency domain image enhancement approaches. | CO3 | (06) |
|  | b) | For the image segment shown in Fig. 5(b) apply all spatial domain filtering techniques for the pixel (3,4) using 3x3 masking. 0; 110; 12; 15; 10. 12; 12; 135; 50; 4. 50; 70; 50; 30; 5. 13; 12; 45; 35; 55. 100; 40; 3; 140; 45. | CO3 | (14) |
|  |  | Fig.5(b) |  |  |
| 6. | a) | For the image segment shown in Fig. 5(b) apply all smoothing filtering techniques for the pixel (3,2) using 5x5 masking. | CO3 | (10) |
|  | b) | Explain all sharpening filtering techniques used for frequency domain image enhancement technique. | CO3 | (10) |
|  |  | UNIT – IV |  |  |
| 7. | a) | Write the application of image segmentation. | CO4 | (10) |
|  | b) | For the given 8×8 image in Fig. 7(b), implement image segmentation using region growing technique based on Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value i) th= 3 , ii) th=4 and Seed points are pixel P(4,6) and pixel q(5,4). 5; 6; 6; 6; 7; 7; 6; 6. 6; 7; 6; 7; 5; 5; 4; 7. 6; 6; 4; 4; 3; 3; 5; 6. 5; 4; 3; 2; 7; 5; 4; 3. 5; 4; 4; 3; 2; 1; 0; 2. 1; 2; 2; 3; 3; 4; 4; 5. 1; 0; 0; 4; 4; 5; 5; 6. 7; 7; 6; 6; 5; 5; 4; 4. Fig: 7(b) | CO4 | (10) |
| 8. | a) | Write in details edge based segmentation. | CO4 | (10) |
|  | b) | For the given 8×8 image in fig 7b), implement image segmentation using region merging technique based on Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value i) th= 3 , ii) th=2. | CO4 | (10) |
|  |  | UNIT – V |  |  |
| 9. | a) | u=ones(4,4), find forward and inverse DCT transform and DST transform. | CO5 | (12) |
|  | b) | Find 4×4 Hadamard matrix H4. Then find the forward and inverse DFT transform for u=ones(4,4). | CO5 | (08) |
| 10. | a) | Write the properties of Haar, DFT& DST. | CO5 | (06) |
|  | b) | Find the transform matrix & KL transform of u=[ 5 -3 -2 4]. | CO5 | (14) |

****************