RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

SEMESTER END EXAMINATIONS –  JANUARY 2017

| Course & Branch | : | Electrical & Electronics & Engineering. | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Digital Image Processing | Max. Marks | : | 100 |
| Subject Code | : | EEPE49 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Transmission is accomplished in a packets consisting of a start bit , a byte of information and a stop bit. Find out - How many minutes would it take to transmit a 2048×2048 image with 256 intensity levels using a 33.6 k baud modem? - What would be the time be at 3000k baud. | CO1 | (04) |
|  | b) | Explain the basic classes of image processing applications. | CO1 | (04) |
|  | c) | Explain the following term as applicable in image processing with necessary graph, i) cones and rods ii) Brightness adaptation iii) Isopreference. | CO1 | (06) |
|  | d) | For v={2,3,4}, compute the lengths of shortest 4, 8 , m paths between p and q in the following image. 3; 4; 1; 2; 0. 0; 1; 0; 4; 2 q. 2; 2; 3; 1; 4. P 3; 0; 4; 2; 1. 1; 2; 0; 3; 4. | CO1 | (06) |
| 2. | a) | Explain with necessary equations i) Adjacency , ii) Distance measure which exhibits basic relationship between the pixels in an image. | CO1 | (06) |
|  | b) | Find the time required in seconds for transmitting a monochrome image of size 2.5” × 2” scanned at 150 DPI and to be sent at 28 KBPS speed. | CO1 | (04) |
|  | c) | Explain briefly the elements of digital image processing system with a diagram. | CO1 | (10) |
|  |  | UNIT- II |  |  |
| 3. | a) | Consider the following 8- level grey scale image of size 8x8.Compute histogram of the image. Compare equalized histogram and display graphically. 0; 1; 1; 0; 0; 1; 1; 6. 0; 1; 2; 3; 3; 2; 1; 7. 0; 4; 5; 3; 3; 5; 4; 6. 0; 1; 2; 7; 7; 2; 1; 5. 0; 1; 2; 6; 6; 2; 1; 6. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 1; 1; 0; 0; 1; 1; 0. | CO2 | (10) |
|  | b) | Explain the concept of histogram matching ,development of the method and the corresponding implementation with a suitable example. | CO2 | (10) |
| 4. | a) | Specify the objective of image enhancement techniques. | CO2 | (10) |
|  | b) | Suppose for a 8 level image, frequency table for input grey level are Rk; 0; 1; 2; 3; 4; 5; 6; 7. Nk; 0; 0; 3; 5; 2; 4; 10; 0. Perform a linear contrast stretching on this image. | CO2 | (10) |
|  |  | UNIT- III |  |  |
| 5. | a) | Discuss characteristics of high boost filter in frequency domain. Explain how high boost filtering increases the enhancement of the images. | CO3 | (10) |
|  | b) | Suggest typical derivative for image enhancement i) Roberts ii) Prewitt iii) sobel. | CO3 | (10) |
| 6. | a) | For the image segment f(x,y),given below. Apply 0; 1; 0; 6; 5. 2; 3; 1; 2; 5. 1; 2; 4; 5; 4. 1; 0; 6; 5; 2. 2; 3; 5; 7; 6. - Smoothing filter - Weighted average filter - Median filter iv) Min filter - Max filter - Laplacian filter (4 connectivity and 8 connectivity only for 3x3) Of size 3x3 and 5x5 on the pixel 7 | CO3 | (14) |
|  | b) | Explain smoothing frequency domain filters. What is ringing effect. | CO3 | (06) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Define thresholding and explain the various methods of thresholding in detail. | CO4 | (10) |
|  | b) | Consider an image f= [1 2 5 ; 5 5 5; 5 3 2 ] . Show the output of any two edge detection algorithm. | CO4 | (10) |
| 8. | a) | For an optimal threshold for the image given where the probability distribution of the graylevel z is given as follows: P1(z)= {0 if z<1; 0.5z+1 if 1<=z<=3 ;0 when z>3 P2(z)= {0 if z<1; -0.5z+5 if 1<=z<=2; 0 when z>2 Assume P1=P2. Write a program to get the segmented image. | CO4 | (10) |
|  | b) | What is edge ? List the properties of second derivative around an edge. | CO4 | (10) |
|  |  | UNIT-V |  |  |
| 9. | a) | Find the Hadamard Transform of the following image: 5 6 8 10 6 6 5 7 4 5 3 6 8 7 5 5 | CO5 | (10) |
|  | b) | Explain K-L transform in detail. | CO5 | (10) |
| 10. | a) | Specify the properties of 2D discrete fourier transform. Explain symmetric property in 2D discrete fourier transform | CO5 | (10) |
|  | b) | Obtain the Haar transform matrix for N=4? | CO5 | (10) |

****************