RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS –  JANUARY 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Digital Image Processing | Max. Marks | : | 100 |
| Subject Code | : | EEPE49 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Draw the block diagram of an image processing system and describe the roles of its components. | CO1 | (10) |
|  | b) | Suppose that a camera is focused a lamp post of height 5 metres situated at a distance of 20 metres. If the focal length of the camera is 10 mm, what will be the height of the image? | CO1 | (04) |
|  | c) | List the applications of image processing. | CO1 | (06) |
| 2. | a) | Describe the process of conversion of an analog image into a digital image. | CO1 | (08) |
|  | b) | For the pixels p(88,44) and q(35,42), compute their distance in different techniques. | CO1 | (06) |
|  | c) | What are the different types of pixel connectivity that may be present in a digital image? Illustrate them using suitable diagrams. | CO1 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | Consider the following 8- level grey scale image of size 8x8.Compute histogram of the image.Compare equalized histogram and display graphically. 0; 1; 1; 0; 0; 1; 1; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 7; 7; 2; 1; 0. 0; 1; 2; 6; 6; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 1; 1; 0; 0; 1; 1; 0. | CO2 | (06) |
|  | b) | Find the following: - image negative - g1(v)=5v -20 - g2(v)=v/2+50 for the given segment below. 1; 2; 3. 4; 5; 3. 1; 2; 7. | CO2 | (06) |
|  | c) | Explain the concept of histogram matching ,development of the method and the corresponding implementation with a suitable example. | CO2 | (08) |
| 4. | a) | Specify the objective of image enhancement techniques. | CO2 | (06) |
|  | b) | Suppose for a 8 level image, frequency table for input grey level are: Rk; 0; 1; 2; 3; 4; 5; 6; 7. Nk; 0; 0; 3; 5; 2; 4; 10; 0. | CO2 | (06) |
|  |  | Perform a linear contrast stretching on this image. |  |  |
|  | c) | Perform the following point processing techniques in the given segment - power law transformation - log transformation. 1; 2; 3. 4; 5; 3. 1; 2; 7. | CO2 | (08) |
|  |  | UNIT-III |  |  |
| 5. | a) | Bring out the differences between spatial domain and frequency domain image enhancement approaches. | CO3 | (06) |
|  | b) | For the image segment shown in image 3b) apply i) smoothing filter, ii) weighted average filter, iii) median filter, iv) min filter, v) max filter, vi) midpoint filter, vii) laplacian filter only for 3×3 (4 connectivity and 8 connectivity) of size 3×3 and 5×5 on the pixel(3,3) | CO3 | (14) |
| 6. | a) | Discuss characteristics of high boost filter in frequency domain. Explain how high boost filtering increases the enhancement of the images. | CO3 | (10) |
|  | b) | Suggest typical derivative for image enhancement: i) Roberts ii) Prewitt iii) sobel. | CO3 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | With the basic formulation, explain edge detection of discontinuity of an image. | CO4 | (10) |
|  | b) | What is segmentation? Discuss the applications of segmentation. | CO4 | (10) |
| 8. | a) | Explain the local processing technique of edge linking and boundary detection | CO4 | (10) |
|  | b) | Discuss the region splitting and merging algorithm of region oriented segmentation. | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Find forward and inverse hadmard transform. | CO5 | (10) |
|  | b) | Find 2×2 Haar matrix H2. Then find the forward Haar transform for | CO5 | (10) |
| 10. | a) | Write the properties of DFT, DCT and DST. | CO5 | (06) |
|  | b) | Find the transform matrix and KL transform of: | CO5 | (14) |

****************