RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS – JANUARY 2020

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Digital Image Processing | Max. Marks | : | 100 |
| Course Code | : | EEE24 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Describe the functions of elements of digital image processing system with a diagram. | CO1 | (10) |
|  | b) | A medical image has a size of 8×8 inches. The sampling resolution is 5 cycles/ mm. How many pixel are required? Will an image of size 256×256 be enough? | CO1 | (06) |
|  | c) | Suppose a camera is focused at a pillar of height 2 meters and situated at a distance of 12 meters. If the focal length of the camera is 10 mm. What will be the height of the image produced in camera. | CO1 | (04) |
| 2. | a) | What is image quantization? How it is achieved under grey level reduction and spatial reduction. | CO1 | (10) |
|  | b) | For V={0,2,3} compute the length of shortest 4,8,m path between p and q in the following image shown in image 2(b). Repeat the same for V={1,2,3} 3; 4; 1; 2; 0. 0; 1; 0; 4; 2q. 2; 2; 3; 1; 4. P3; 0; 4; 2; 1. 1; 2; 0; 3; 4. image 2(b) | CO1 | (06) |
|  | c) | An image is 2400 pixels wide and 2400 pixels high. The image was scanned at 300 dpi . What is the physical size of the image? | CO1 | (04) |
|  |  | UNIT- II |  |  |
| 3. | a) | Explain in details any two type of image enhancement techniques. | CO2 | (08) |
|  | b) | consider the following image shown in image 3(b), having grey value between 0 to 15. Stretch the grey values uniformly and linearly in the entire range. 5; 11; 12; 4; 10. 10; 12; 4; 5; 4. 11; 13; 5; 3; 5. 13; 12; 4; 13; 5. 12; 4; 3; 4; 4. image 3(b) | CO2 | (06) |
|  | c) | Write a note on colour enhancement method. | CO2 | (06) |
| 4. | a) | Perform law transformation on the given input value shown in image 4c) ,for γ=2 and γ=0.5 and comment on compression and expansion observation. | CO2 | (08) |
|  | b) | What is the importance of image enhancement in image processing? Explain in brief any one point processing techniques implemented in image processing. | CO2 | (06) |
|  | c) | Do the transformation g1(v)=v+1, g2(v)=v2 and g3(v) = negative transformation of the image f(x,y) shown in image 4(c). 0; 10; 50; 100. 5; 95; 150; 200. 110; 150; 190; 210. 175; 210; 255; 100. image 4(c) | CO2 | (06) |
|  |  | UNIT- III |  |  |
| 5. | a) | Define smoothing filters, what are the uses? Explain low pass spatial filtering with suitable example. | CO3 | (10) |
|  | b) | Explain the different spatial domain filters for sharpening of an image. | CO3 | (10) |
| 6. | a) | Examine the basic steps for filtering in the frequency domain. | CO3 | (06) |
|  | b) | Discuss the various frequency domain filters for sharpening of an image . | CO3 | (08) |
|  | c) | Define homomorphic filtering. How is it used for image enhancement in frequency domain? Explain. | CO3 | (06) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Define segmentation. Explain the three basic types of discontinuities in a digital image and the technique implemented for detection. | CO4 | (10) |
|  | b) | Explain the basic global and adaptive thresholding used for image segmentation. | CO4 | (10) |
| 8. | a) | With an illustration, explain splitting and merging algorithm of region based segmentation. | CO4 | (08) |
|  | b) | Explain edged based segmentation. | CO4 | (06) |
|  | c) | Define Image Restoration? Compare image enhancement and image restoration processes. | CO4 | (06) |
|  |  | UNIT- V |  |  |
| 9. | a) | U=ones(4,4); find forward and inverse DCT. | CO5 | (08) |
|  | b) | Find 8×8 Haar transform matrix. | CO5 | (12) |
| 10. | a) | Prove that it needs O(Nlog2N) operations for Hadamard transform. | CO5 | (10) |
|  | b) | Compute i) 2D DFT and ii) 2D DST of given 4×4 grey scale image U=ones(4,4); | CO5 | (10) |

****************