RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS  JANUARY – FEBRUARY 2021

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Digital Image Processing | Max. Marks | : | 100 |
| Course Code | : | EEE24 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Describe image formation in the human eye. Explain with examples that the perceived brightness is not a simple function of intensity. | CO1 | (10) |
|  | b) | A medical image has a size of 6×8 inches. The sampling resolution is 5 cycles/ mm. How many pixel are required? Will an image of size 256×256 be enough? | CO1 | (06) |
|  | c) | Given A= zeros(4,4); A(2,3)=1; Find different distances from the given point to all the pixel present in A. | CO1 | (04) |
| 2. | a) | What is image quantization? How it is achieved under grey level reduction and spatial reduction. | CO1 | (10) |
|  | b) | For v={1,2,3}, compute the lengths of shortest 4, 8 , m paths between p(7,1) and q(0,7) in the following image. And find distances between the two points using three different methods. 0; 1; 1; 0; 0; 1; 1; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 7; 7; 2; 1; 0. 0; 1; 2; 6; 6; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 1; 1; 0; 0; 1; 1; 0. | CO1 | (06) |
|  | c) | An image in 1800 pixels wide and 2000 pixels high. The image was scanned at 250 DPI . What is the physical size of the image? | CO1 | (04) |
|  |  | UNIT – II |  |  |
| 3. | a) | Explain with example histogram specification and histogram equalization. | CO2 | (08) |
|  | b) | The output of A= imhist(graylevel_image) is given as shown in Fig. 3(b). Perform histogram stretching such that the grey level is spread between 2 to 5 .The image is 64 X 64, 8 levels. Fig. 3(b) | CO2 | (06) |
|  | c) | Write a note on pseudo colour processing method. | CO2 | (06) |
| 4. | a) | Perform bit plane slicing on the given input value shown in Fig. 4(b), and find enhanced grey level image by adding most significant 4 planes. | CO2 | (08) |
|  | b) | For the given image segment find the following: Fig: 4(b) image segment =im Find im1=Imcomplement(im) and perform the following i) imb & im1b ii) imb ¦ im1b iii) xor(imb, im1b). where imb and im1b are the black and white images considering threshold as mid point of the given image grey levels. 200; 10; 50; 100. 160; 95; 150; 200. 110; 15; 190; 10. 175; 210; 255; 100. | CO2 | (08) |
|  |  | Fig.4(b) |  |  |
|  | c) | Write a note on point processing techniques. | CO2 | (04) |
|  |  | UNIT – III |  |  |
| 5. | a) | Bring out the differences between spatial domain and frequency domain image enhancement approaches. | CO3 | (06) |
|  | b) | For the image segment shown in Fib. 5(b) apply all spatial domain filtering techniques for the pixel (0,3) using 3x3 masking. 100; 110; 12; 115; 10. 12; 12; 135; 5; 4. 150; 70; 50; 30; 5. 13; 12; 45; 3; 55. 100; 40; 3; 140; 45. | CO3 | (14) |
|  |  | Fib. 5(b) |  |  |
| 6. | a) | For the image segment shown in image 5b) apply all restoration filtering techniques for the pixel (2,2) using 5x5 masking. | CO3 | (10) |
|  | b) | Explain Homomorphic filtering technique. | CO3 | (10) |
|  |  | UNIT – IV |  |  |
| 7. | a) | Write the application of image segmentation. | CO4 | (10) |
|  | b) | For the given 8×8 image in Fig. 7(b), implement image segmentation using region growing technique based on Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value i) th= 3, ii) th=5 and Seed points are pixel P(6,2) and pixel q(3,4). 5; 6; 6; 6; 7; 7; 6; 6. 6; 7; 6; 7; 5; 5; 4; 7. 6; 6; 4; 4; 3; 3; 5; 6. 5; 7; 3; 2; 7; 7; 4; 3. 5; 4; 4; 3; 2; 1; 0; 2. 1; 2; 2; 3; 3; 4; 4; 5. 1; 0; 0; 4; 4; 5; 5; 6. 7; 7; 6; 6; 5; 5; 4; 4. | CO4 | (10) |
|  |  | Fig. 7(b) |  |  |
| 8. | a) | Write in details edge based segmentation. | CO4 | (10) |
|  | b) | For the given 8×8 image in fig 7b), implement image segmentation using region splitting technique based on Max{f(x,y)}-Min{f(x,y)}≤ th, here threshold value i) th= 3 , ii) th=4. | CO4 | (10) |
|  |  | UNIT – V |  |  |
| 9. | a) | u=ones(4,4), find forward and inverse hadmard transform. | CO5 | (08) |
|  | b) | Find 4×4 Haar matrix H4. Then find the forward Haar transform for u=ones(4,4). | CO5 | (12) |
| 10. | a) | Write the properties of DFT, DCT & DST | CO5 | (06) |
|  | b) | Find the transform matrix & KL transform of u=[ 4 -2 -1 3] | CO5 | (14) |

****************