| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)

Accredited by NBA & NAAC with ‘A+’ Grade

SEMESTER END EXAMINATIONS – MARCH 2022

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Digital Image Processing | Max. Marks | : | 100 |
| Course Code | : | EEE24(O) | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Describe image formation in the human eye. Explain with examples that the perceived brightness is not a simple function of intensity. | CO1 | (10) |
|  | b) | A medical image has a size of 8×8 inches. The sampling resolution is 5 cycles/ mm. How many pixel are required? Will an image of size 256×256 be enough? | CO1 | (06) |
|  | c) | Suppose a camera is focused at a pillar of height 5 meters and situated at a distance of 15 meters. If the focal length of the camera is 15 mm. What will be the height of the image produced in camera? | CO1 | (04) |
| 2. | a) | What is image quantization? How it is achieved under grey level reduction and spatial reduction. | CO1 | (10) |
|  | b) | For V={0,2,3} compute the length of shortest 4,8,m path between p & q in the following image shown in Fig.2(b). Repeat the same for V={1,2,3}. 3; 4; 1; 2; 0. 0; 1; 0; 4; 2q. 2; 2; 3; 1; 4. P3; 0; 4; 2; 1. 1; 2; 0; 3; 4. | CO1 | (06) |
|  |  | Fig.2(b) |  |  |
|  | c) | An image in 2400 pixels wide and 2400 pixels high. The image was scanned at 300 dpi . What is the physical size of the image? | CO1 | (04) |
|  |  | UNIT – II |  |  |
| 3. | a) | Explain in details any two type of image enhancement techniques. | CO2 | (08) |
|  | b) | Consider the following image shown in Fig.3(b), having grey value between 0 to 15. Stretch the grey values uniformly and linearly in the entire range. 10; 11; 12; 11; 10. 12; 12; 13; 5; 4. 13; 12; 5; 3; 5. 13; 12; 4; 3; 5. 12; 4; 3; 4; 4. | CO2 | (06) |
|  |  | Fig 3(b) |  |  |
|  | c) | Write a note on colour enhancement method. | CO2 | (06) |
| 4. | a) | Perform law transformation on the given input value shown in Fig.4(c), for γ=2.5 and γ=0.4 and comment on compression and expansion observation. | CO2 | (08) |
|  | b) | What is the importance of image enhancement in image processing? Explain in brief any one point processing techniques implemented in image processing. | CO2 | (06) |
|  | c) | Do the transformation g1(v)=v+1, g2(v)=v2 and g3(v) = negative transformation of the image f(x,y) shown in Fig. 4(c). 0; 10; 50; 100. 5; 95; 150; 200. 110; 150; 190; 210. 175; 210; 255; 100. Fig.4(c) | CO2 | (06) |
|  |  | UNIT – III |  |  |
| 5. | a) | Bring out the differences between spatial domain and frequency domain image enhancement approaches. | CO3 | (06) |
|  | b) | For the image segment shown in Fig. 3(b) apply i) smoothing filter, ii) weighted average filter, iii) median filter, iv) min filter, v) max filter, vi) midpoint filter, vii) laplacian filter only for 3×3( 4 connectivity & 8 connectivity ) of size 3×3 and 5×5 on the pixel(3,3) | CO3 | (14) |
| 6. | a) | Discuss characteristics of high boost filter in frequency domain. Explain how high boost filtering increases the enhancement of the images. | CO3 | (10) |
|  | b) | Suggest typical derivative for image enhancement: i) Roberts ii) Prewitt iii) sobel. | CO3 | (10) |
|  |  | UNIT – IV |  |  |
| 7. | a) | Write the application of image segmentation. | CO4 | (10) |
|  | b) | For the given 8×8 image in Fig. 7(b), implement image segmentation using region growing technique based on Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value i) th= 3,R R ii) th=4 andSeed points are pixel P(4,6) and pixel q(5,4). 5; 6; 6; 6; 7; 7; 6; 6. 6; 7; 6; 7; 5; 5; 4; 7. 6; 6; 4; 4; 3; 3; 5; 6. 5; 4; 3; 2; 7; 5; 4; 3. 5; 4; 4; 3; 2; 1; 0; 2. 1; 2; 2; 3; 3; 4; 4; 5. 1; 0; 0; 4; 4; 5; 5; 6. 7; 7; 6; 6; 5; 5; 4; 4. Fig. 7(b) | CO4 | (10) |
| 8. | a) | Write in details edge based segmentation. | CO4 | (10) |
|  | b) | For the given 8×8 image in Fig. 7(b), implement image segmentation using region merging technique based on Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value i) th= 3 , ii) th=2.R R | CO4 | (10) |
|  |  | UNIT – V |  |  |
| 9. | a) | Find forward and inverse hadmard transform. U=1111111111111111 | CO5 | (10) |
|  | b) | Find 2×2 Haar matrix H2. Then find the forward Haar transform for u=1111 | CO5 | (10) |
| 10. | a) | Write the properties of DFT, DCT and DST. | CO5 | (06) |
|  | b) | Find the transform matrix and KL transform of u=4−2−13 | CO5 | (14) |

****************