| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)

Accredited by NBA & NAAC with ‘A+’ Grade

SEMESTER END EXAMINATIONS – MARCH 2022

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Digital Image Processing | Max. Marks | : | 100 |
| Course Code | : | EEE751 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Briefly explain the components of a general purpose image processing system. | CO1 | (06) |
|  | b) | For the image segment shown on Fig. 1(b), compute the lengths of the shortest 4,8 and m path between p and q for V={28,30}, Explain the reason if a particular path does not exist, 40 28 30 28 (q) 30 30 10 28 28 30 28 28 (p) 28 10 28 30 Fig. 1(b) | CO1 | (07) |
|  | c) | Define: i) spatial resolution ii) intensity resolution iii) adjacency iv)false contouring. | CO1 | (07) |
| 2. | a) | Explain the fundamental steps in Digital Image Processing. | CO1 | (06) |
|  | b) | Considering a scan line in a continuous image function, explain the concepts in sampling and Quantization. | CO1 | (07) |
|  | c) | Define the following: i) 4 adjacency ii)8 adjacency iii) m adjacency iv) connectivity. | CO1 | (07) |
|  |  | UNIT – II |  |  |
| 3. | a) | What is the principle of spatial domain techniques of image enhancement? Explain the following methods of image enhancement i) Bitplane slicing ii) Gray level slicing. | CO2 | (10) |
|  | b) | What is meant by image histogram? Derive an equation to implement histogram equalization of an image. | CO2 | (10) |
| 4. | a) | For the given 4*4 image, perform histogram equalization and draw the histogram of image before and after equalization. | CO2 | (10) |
|  | b) | Discuss on following point processing operations performed on an image Image Addition (ii) Image Subtraction. | CO2 | (10) |
|  |  | UNIT – III |  |  |
| 5. | a) | Explain the characteristics of smoothing linear filters. Discuss the advantages of median filtering over averaging filter. | CO3 | (05) |
|  | b) | Obtain frequency domain formulation of unsharp masking, high boost filtering and high frequency emphasis filters in Image sharpening techniques. | CO3 | (07) |
|  | c) | Explain the transfer function of Gaussian LPF. Plot i) perspective plot ii) Filter displayed as an image iii) Filter radial cross section. | CO3 | (08) |
| 6. | a) | Derive the equation to implement 3 x 3 laplacian mask of two variables. | CO3 | (05) |
|  | b) | Explain image sharpening using ideal highpass filter and explain the ringing property of ideal filters. | CO3 | (07) |
|  | c) | Discuss the concept of constrained least square filtering. | CO3 | (08) |
|  |  | UNIT – IV |  |  |
| 7. | a) | Define segmentation, explain three basic types of discontinuities in a digital image. | CO4 | (10) |
|  | b) | Obtain an optimal thresholding value T for an image in which the object and the background pixels are Gaussian distributed with a different values of variance and mean. | CO4 | (10) |
| 8. | a) | With an illustration, explain splitting and merging algorithm of region based segmentation. | CO4 | (10) |
|  | b) | Illustrate the following morphological algorithms: (i) Boundary extraction (ii) Hole filling (iii) Thinning. | CO4 | (10) |
|  |  | UNIT – V |  |  |
| 9. | a) | Define orthogonality, Unitary with respect to signal. | CO5 | (04) |
|  | b) | U=[ 1 1 1 1; 1 1 1 1; 1 1 1 1; 1 1 1 1], Find forward Hadamard transform. | CO5 | (06) |
|  | c) | Given x(m,n)=[1 2 3; 4 5 6] and h(m,n)=[1 1 1]. Find the convolution between two matrices. | CO5 | (10) |
| 10. | a) | Recall the properties of the Walsh transform. | CO5 | (04) |
|  | b) | Find Walsh basis for N=4. | CO5 | (06) |
|  | c) | Compute the DCT matrix for N=4 and also mention the uses of DCT. | CO5 | (10) |

****************