RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

EXAMINATIONS SEPTEMBER /OCTOBER 2020

SUPPLEMENTARY SEMESTER / GRADE IMPROVEMENT/ RE –REGISTERED CANDIDATES

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Digital Image Processing | Max. Marks | : | 100 |
| Course Code | : | EEE24 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Write short note on perception of human eye. | CO1 | (10) |
|  | b) | Transmission is occupied in packets consisting of a start bit, a byte of information and a stop bit, - How many minutes would it take to transmit a 1048×1048 image with 256 intensity levels using 25.6K baud modem? - What would the time be at 2500K baud? | CO1 | (06) |
|  | c) | Let p and q be the pixels at coordinates as shown in image segment shown in Fig. 2(b). Find out which distance measure gives the minimum distance between pixels. | CO1 | (04) |
| 2. | a) | Explain the fundamental steps involved in digital image processing. | CO1 | (10) |
|  | b) | For V={1,2} compute the length of shortest 4,8,m path between p & q in the following image shown in Fig.2(b). Repeat the same for V={0,1} 5; 4; 3; 1; 1q. 5; 4; 0; 2; 0. 3; 2; 0; 2; 4. 2; 1; 1; 3; 5. P1; 3; 5; 1; 3. Fig:2(b) | CO1 | (06) |
|  | c) | Find the time required in seconds for transmitting a monochrome image of size 2.5”×2” scanned at 150DPI and to be sent at 28 kBPS. | CO1 | (04) |
|  |  | UNIT- II |  |  |
| 3. | a) | Write a note on Multi-spectral image enhancement. | CO2 | (05) |
|  | b) | Compute 2 bit planes starting from MSB of the following 8 bit image 0; 10; 5; 100. 5; 95; 150; 20. 110; 50; 190; 210. 175; 210; 255; 110. Fig: 3(b) | CO2 | (05) |
|  | c) | Perform histogram specification on 8×8 image shown in table1 and target histogram is shown in table 2. Table1: Histogram of original image Grey level; 0; 1; 2; 3; 4; 5; 6; 7. No. of pixels; 8; 10; 10; 2; 12; 16; 4; 2. Table2: Histogram of target image Grey level; 0; 1; 2; 3; 4; 5; 6; 7. No. of pixels; 0; 0; 0; 0; 20; 20; 16; 8. | CO2 | (10) |
| 4. | a) | Write a note on false colour and Pseudo-colour. | CO2 | (08) |
|  | b) | Perform histogram stretching on 8×8 image , 8 levels grey image shown in table below. Table: image grey level distribution Grey level; 0; 1; 2; 3; 4; 5; 6; 7. No. of pixels; 0; 0; 5; 20; 20; 19; 0; 0. | CO2 | (06) |
|  | c) | Draw the histogram and equalised histogram of the following 8×8 image shown in Fig. 4(c) 0; 1; 1; 0; 0; 1; 1; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 7; 7; 2; 1; 0. 0; 1; 2; 6; 6; 2; 1; 0. 0; 4; 5; 3; 3; 5; 4; 0. 0; 1; 2; 3; 3; 2; 1; 0. 0; 1; 1; 0; 0; 1; 1; 0. | CO2 | (06) |
|  |  | Fig: 4(c) |  |  |
|  |  | UNIT- III |  |  |
| 5. | a) | Write a note on frequency domain sharpening image enhancement approaches. | CO3 | (08) |
|  | b) | Apply spatial domain filtering for the pixel (2,1) using 3 X 3 mask. 5; 11; 12; 4; 10. 10; 12; 4; 5; 4. 11; 13; 5; 3; 5. 13; 12; 4; 13; 5. 12; 4; 3; 4; 4. | CO3 | (12) |
| 6. | a) | Discuss characteristics of high boost filter in frequency domain. Explain how high boost filtering increases the enhancement of the images. | CO3 | (10) |
|  | b) | Explain with example , 1st order derivative of image enhancement and also derive the mask for Laplacian filter. | CO3 | (10) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Write the application of image segmentation. | CO4 | (10) |
|  | b) | For the given 8×8 image in Fig. 7(b), implement image segmentation using region growing technique based on Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value i) th= 3 , ii) th=4 and Seed points are pixel P(2,4) and pixel q(3,5). 5; 6; 6; 6; 7; 7; 6; 6. 6; 7; 6; 7; 5; 5; 4; 7. 6; 6; 4; 4; 3; 3; 5; 6. 5; 4; 3; 2; 7; 5; 4; 3. 5; 4; 4; 3; 2; 1; 0; 2. 1; 2; 2; 3; 3; 4; 4; 5. 1; 0; 0; 4; 4; 5; 5; 6. 7; 7; 6; 6; 5; 5; 4; 4. | CO4 | (10) |
|  |  | fig: 7(b) |  |  |
| 8. | a) | Find edge lines by Hough transform of the image having pixel values(1,0),(1,3),(2,2),(1,2),(3,3,(4,3),(3,4), for 4 different Ѳ value range -900< Ѳ≤ 900. | CO4 | (10) |
|  | b) | For the given 8×8 image in fig 7b), implement image segmentation using region merging technique based on Max{f(x,y)}-min{f(x,y)}≤ th, here threshold value i) th= 3 , ii) th=4. | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Explain the following properties of 2-D Fourier transform: (i) Separability (ii) Rotation (iii) Average value. | CO5 | (12) |
|  | b) | Discuss the Discrete cosine transform and its properties with examples. | CO5 | (08) |
| 10. | a) | Recall the properties of the Hadamard transform. | CO5 | (06) |
|  | b) | State and explain Haar transform. Obtain the Haar transform matrix for N=4 with basic equations. | CO5 | (10) |
|  | c) | List the applications and disadvantages of KL transform. | CO5 | (04) |

****************