RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

Bangalore – 560 054

MAKEUP EXAMINATIONS –  MARCH 2017

| Course & Branch | : | Electrical & Electronics Engineering. | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Electric Drives | Max. Marks | : | 100 |
| Subject Code | : | EE706 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Missing data, IF ANY, can be suitably assumed - Present the answers in a neat and legible manner.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | A weight of 500kg is being lifted up at a uniform speed of 1.5m/s by a winch driven by a motor running at a speed of 1000rpm. The moment of inertia of the motor and winch are 0.5 and 0.3 kg-m2 respectively. Calculate the motor and equivalent moment of inertia referred to the motor shaft. In the absence of weight, motor develops a torque of 100N-m when running at 1000rpm. | CO1 | (06) |
|  | b) | Briefly explain about all the components of load torque. Also draw their speed torque characteristics. | CO1 | (08) |
|  | c) | Discuss about the different types of power modulators used in electric drive systems. | CO1 | (06) |
| 2 | a) | Mention few advantages of an electric drive system. | CO1 | (04) |
|  | b) | Analyze the multiquadrant operation of a crane hoist drive system using four quadrant diagrams. State the conventions used and assumptions made, if any. | CO1 | (10) |
|  | c) | How do you define active and passive load torque? What are the differences between the two? | CO1 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | A 400V, 15kW, dc shunt motor takes 42A and runs at a speed of 1200rpm. The shunt field resistance is 200Ω. Assume that the load torque varies as the square of the speed. Neglect iron and friction losses, and also the armature reaction. Find the resistance to be connected in series with the armature to reduce the speed to 1000rpm. | CO2 | (07) |
|  | b) | Discuss about the operation of a dc motor fed from a dual converter in simultaneous mode. | CO2 | (08) |
|  | c) | A 220 V, 10A, 2000rpm, shunt motor has armature and field resistances of 0.5ohm and 400 ohm respectively. It drives a load whose torque is constant at rated motor torque. Calculate motor speed if the source voltage drops to 175V. | CO2 | (05) |
| 4. | a) | A 220V, 1200rpm, 8A separately excited motor with armature resistance of 0.6Ω is fed from a 1Ф fully controlled rectifier. The ac source voltage is 230V, 50Hz. Determine the converter firing angle when the motor is operating in , a) motoring quadrant at rated torque and 900rpm, b) braking quadrant at rated torque and 900rpm. | CO2 | (06) |
|  | b) | With relevant aid, explain the effect of varying field circuit voltage in a dc motor. | CO2 | (07) |
|  | c) | A dc motor needs to be operated in 1st and 4th quadrant. If the available source is dc in nature, what topology of converter will you suggest to meet the above mentioned requirement? Justify your suggestion. | CO2 | (07) |
|  |  | UNIT- III |  |  |
| 5. | a) | A 1800V, 2000kW, 750 rpm, 50Hz, 8-pole, 3-phase squirrel cage induction motor has the following parameters referred to the stator: Rs=0.075Ω, Rr’=0.1Ω, Xs=0.45Ω, Xr’=0.55Ω. Stator winding in delta connected and consists of two sections connected in parallel i)Calculate starting torque and maximum torque as a ratio of rated torque, if the motor is started by star-delta switching. What is the maximum value of line current during starting? ii) Calculate the transformation ratio of an auto-transformer used to limit the maximum starting current to twice the rated value. Also find the value of the starting torque. iii) What will be the value of maximum line current and torque during starting, if the part winding method is employed for starting? | CO3 | (10) |
|  | b) | Discuss about dc dynamic braking of induction motors. | CO3 | (07) |
|  | c) | Mention why rotor resistance method is preferred for crane hoist applications. | CO3 | (03) |
| 6. | a) | What are the different types of starters available for starting an induction motors? Explain the working of any type of starter with diagrams. | CO3 | (07) |
|  | b) | A 400V, star connected, 3-phase, 6-pole, 50Hz induction motor has the following parameters referred to the stator: Rs=Rr’=1 ohm, Xs=Xr’=2 ohm. This motor is to be braked by plugging from its initial full load speed of 950rpm. Stator to rotor turns ratio is 2.3. Calculate the initial braking current and torque as a ratio of their full load values | CO3 | (06) |
|  | c) | Explain V/f method of speed control in induction motors. | CO3 | (07) |
|  |  | UNIT-IV |  |  |
| 7. | a) | Illustrate the construction features of a BLDC motor. | CO4 | (07) |
|  | b) | Briefly discuss about BLDC controllers. | CO4 | (06) |
|  | c) | Draw the speed-torque characteristics of a synchronous motors and write short notes on it. | CO4 | (05) |
|  | d) | What is hall effect? | CO4 | (02) |
| 8. | a) | How is true synchronous mode operation different from self-controlled operation? Explain. | CO4 | (06) |
|  | b) | Compare BLDC motors with induction motors. | CO4 | (02) |
|  | c) | Discuss the braking operation of synchronous motors. | CO4 | (06) |
|  | d) | Explain any one type of rotor position measurement techniques of dc brushless motor. | CO4 | (06) |
|  |  | UNIT-V |  |  |
| 9. | a) | Using the heat balance equation, derive an expression for the temperature rise in a motor. | CO5 | (07) |
|  | b) | A motor operates on a periodic duty cycle in which it is clutched to its load for 10 minutes and declutched to run on no-load for 20 minutes. Minimum temperature rise is 40°C. Heating and cooling time constants are equal and have a value of 60 minutes. When load is declutched continuously the temperature rise is 15°C. Find, - Maximum temperature during the duty cycle - Temperature when the load is clutched continuously | CO5 | (06) |
|  | c) | Explain the operation of a bipolar stepper motor drive. | CO5 | (07) |
| 10. | a) | A 4-phase, 4/2-pole variable reluctance stepper motor is to be controlled for full-step operation. Find out the switching sequence and draw the switching table, to rotate the rotor continuously. Also find the step-angle that can be obtained in full-step operation. | CO5 | (07) |
|  | b) | Describe the constructional features of a stepper motor with suitable diagram. | CO5 | (06) |
|  | c) | An electric drive has the following specifications. (i) The system should be capable of being operated on a 1-phase, 230 VAC, 50 Hz supply (ii) The drive need to have variable speed operation with speeds varying from 0 to 35% of rated speed (iii) The frequency may be varied, when required, from zero to rated (iv) At low speeds, the speed variation should be smooth (v) The distortion caused by power converters on the power sources should be minimum. Suggest a suitable motor and power electronic converter to meet the above requirement. Also draw the abstract block diagram of the system layout as per your suggestion. | CO5 | (07) |

****************