RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS –  FEBRUARY 2018

| Course & Branch | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Electric Drives | Max. Marks | : | 100 |
| Subject Code | : | EE706 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Make necessary assumptions wherever necessary.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Write the fundamental torque equation of a drive system and write short notes on each of the parameters. | CO1 | (07) |
|  | b) | A horizontal conveyor belt moving at a uniform speed of 1.2m/s transports material at the rate of 100 tonnes/hr. Belt is 200m long and driven by a motor at 1200 rpm. - Determine the load inertia referred to the motor shaft. - Calculate the torque that motor should develop to accelerate the belt from standstill to full speed in 8 sec. moment of inertia of the motor is 0.1 Kg-m2 | CO1 | (06) |
|  | c) | Using four quadrant diagrams, analyze the multiquadrant operation of an Electric car. State the speed-torque conventions used and assumptions made. | CO1 | (07) |
| 2. | a) | Draw the speed-torque characteristics of the following loads: - Traction loads (ii)Constant power loads. | CO1 | (04) |
|  | b) | How do you define active and passive load torque? What are the differences between the two? | CO1 | (06) |
|  | c) | The motor is required to drive the take-up roll on a plastic strip line. The mandrel on which the strip is wound is 15cm in diameter and strip builds up to a roll 25 cm in diameter. Strip tension is maintained constant at 1000 N. The strip moves at a uniform speed of 25 m/s. The motor is coupled to a mandrel by a reduction gear of ratio 0.5. The gears have an approximate efficiency of 87% at all speeds. Determine the speed and power rating of the motor required for this application. | CO1 | (06) |
|  | d) | Mention any four advantages of the electric drive systems over mechanical drive systems. | CO1 | (04) |
|  |  | UNIT- II |  |  |
| 3. | a) | A dc shunt motor need to be braked. If the cost of the braking system need to be kept low and implementation should be simple, then what kind of braking technique would you suggest? Justify your answer. | CO2 | (06) |
|  | b) | A 200 V, 10.5A, 2000rpm, shunt motor has armature and field resistances of 0.5ohm and 400 ohm respectively. It drives a load whose torque is constant at rated motor torque. Calculate motor speed if the source voltage drops to 175V. | CO2 | (06) |
|  | c) | Discuss the operation of a single phase fully controlled converter fed separately excited dc motor with necessary diagrams. | CO2 | (08) |
| 4. | a) | A 220V, 500A, 600rpm separately excited motor has armature and field resistance of 0.02ohm and 10ohm respectively. The load torque is given by the expression TL = 2000 – 2N, N-m where N is the speed of the motor. Speeds below the rated are obtained by armature voltage control and speeds above the rated are obtained by field control. - Calculate motor terminal voltage and armature current when the speed is 450rpm. - Calculate field winding voltage and armature current when the speed is 750rpm. | CO2 | (08) |
|  | b) | A dc separately excited motor is running at 800 rpm driving a load whose torque is constant. Motor armature current is 500A. The armature resistance drop and rotational losses are negligible. Magnetic circuit can be assumed to be linear. Calculate motor speed and armature current if terminal voltage is reduced to 50% and field current is reduced to 80%. | CO2 | (06) |
|  | c) | Draw the speed torque characteristics of a separately excited motor when the motor is controlled by: - Armature voltage control method - Field control method. | CO2 | (06) |
|  |  | UNIT-III |  |  |
| 5. | a) | Explain stator frequency control method used for controlling the speed of induction motors. Also draw the speed-torque characteristics of the motor while stator frequency control is employed. | CO3 | (07) |
|  | b) | A 2.8kW, 400V, 50Hz, 4-pole, 1370rpm, delta connected squirrel-cage induction motor has following parameters referred to the stator: Rs=2Ω, Rr’=5 Ω, Xs=Xr’=5 Ω, Xm=80 Ω. Motor speed is controlled by stator voltage control. When driving a fan load it runs at rated speed at rated voltage. Calculate : - Motor terminal voltage, current and torque at 1200rpm, and - Motor speed, current and torque for the terminal voltage of 300V. | CO3 | (07) |
|  | c) | Plot the speed-torque characteristics of a 3-phase induction motor. Also derive a relation between speed and torque for this motor by considering a simplified per-phase equivalent circuit. | CO3 | (06) |
| 6. | a) | A 400V, star connected, 3-phase, 6-pole, 50Hz induction motor has the following parameters referred to the stator: Rs=Rr’=1 Ω, Xs=Xr’=2 Ω. For regenerative braking operation of this motor determine: - Maximum overhauling torque it can hold and range of speed for safe operation - Speed at which it will hold an overhauling load with a torque of 100 N-m | CO3 | (06) |
|  | b) | Discuss about induction motor control using ac voltage controllers. | CO3 | (07) |
|  | c) | Explain the cycloconverter control for variable frequency control of induction motor drives. | CO3 | (07) |
|  |  | UNIT- IV |  |  |
| 7. | a) | What are the different ways through which a synchronous motor can be braked? Explain about the method which is widely used. | CO4 | (07) |
|  | b) | Draw the circuit diagram of a voltage source inverter fed BLDC motor drive and explain its operation. | CO4 | (07) |
|  | c) | Discuss about the different modes of variable frequency control of synchronous motors. | CO4 | (06) |
| 8. | a) | Derive an expression for the torque produced by a cylindrical rotor wound field motor and draw the speed-torque characteristics. | CO4 | (08) |
|  | b) | What are the different methods by which rotor position of a BLDC motor can be determined? Briefly explain. | CO4 | (06) |
|  | c) | Explain pull-in process in synchronous motors. | CO4 | (06) |
|  |  | UNIT- V |  |  |
| 9. | a) | Explain the working of bipolar stepper motor driving circuits with appropriate sketches. | CO4 | (08) |
|  | b) | An electric drive has the following specifications. - The system should be capable of being operated on a 220 V DC source. - The drive should have the facility to be braked to a set speed as quickly as possible. - The drive need to have constant speed operation under normal operating conditions. - After the user has SET the speed of the motor, it should run at the same speed regardless of load variations. - The dominant harmonics of the power converter should be shifted to higher frequency region. Select appropriate motor, power electronic converter, control strategy to be used for this application. Also draw the abstract block diagram of the system layout you would suggest for this application. | CO5 | (08) |
|  | c) | Identify the class of duty for the drive systems in the following applications: - Air-conditioner - Refrigerator - Ventilation duct air blower - Electric car. | CO5 | (04) |
| 10. | a) | Explain the different classes of motor duty with appropriate diagrams. | CO5 | (07) |
|  | b) | A 4-phase, 4/2-pole variable reluctance stepper motor is to be controlled for half-step operation. Find out the switching sequence and draw the switching table, to rotate the rotor continuously. Also find the step-angle that can be obtained in half-step operation. | CO4 | (07) |
|  | c) | Draw the torque-stepping rate characteristics of a stepper motor and write short notes about it. | CO4 | (06) |

****************