RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Electric Drives | Max. Marks | : | 100 |
| Subject Code | : | EEE11/EE706 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Discuss the advantages of an electric drive? Mention the factors on which choice of an electric drive depends. | CO1 | (08) |
|  | b) | A motor drives two loads, One has rotational motion. It is coupled to the motor through a reduction gear ratio of 0.1 and efficiency of 90%.The load has a moment of inertia of 10kg-m2 and torque of 10N-m. Other load has translational motion and consists of 1000kg weight to be lifted up at an uniform speed of 1.5m/s. Coupling between this load and the motor has an efficiency of 85%. Motor has inertia of 0.2kg-m2 and runs at a constant speed of 1420 rpm. Determine the equivalent inertia referred to the motor shaft and equivalent torque developed by the motor. | CO1 | (06) |
|  | c) | Draw the speed – torque characteristics of the following type of loads: (i) Traction loads (ii) Constant power loads (iii) Air compressor type loads. | CO1 | (06) |
| 2. | a) | With a neat block diagram, state the essential parts of an electric drive system. Explain them briefly. | CO1 | (07) |
|  | b) | Explain the speed-torque conventions and multi-quadrant operation of a motor driving a hoist load. | CO1 | (10) |
|  | c) | Can a motor-load system with a passive load torque have an equilibrium speed in quadrant II? What will be your answer if the load is active? Justify. | CO1 | (03) |
|  |  | UNIT- II |  |  |
| 3. | a) | Explain how field flux control method can be used to control the speed of a dc motor. | CO2 | (06) |
|  | b) | A 230V, 960 rpm, 200A separately excited DC motor has an armature resistance of 0.02Ω. The motor is fed from a chopper with source voltage of 230V. When this motor is operated in dynamic braking mode with RB=2Ω, calculate: - Calculate duty ratio of chopper for motor speed of 600 rpm and braking torque of twice the rated value - What will be the motor speed for a duty ratio of 0.6 and motor torque equal to twice its rated value. | CO2 | (07) |
|  | c) | Discuss about the operation of a half-controlled rectifier fed dc motor with circuit diagram and relevant waveforms. | CO2 | (07) |
| 4. | a) | A 220V, 500A, 600rpm separately excited DC motor has armature and field resistance of 0.02Ω and 10Ω respectively. Field winding is excited by a variable voltage of rated value 220V. The load torque is given by the expression TL = 2000-2N where N is the speed in rpm. Speeds below the rated speed are obtained by armature voltage control and speeds above the rated are obtained by field control. - Calculate motor terminal voltage and armature current when the speed is 450rpm - Calculate field winding voltage and armature current when speed is 750rpm. | CO2 | (07) |
|  | b) | Which kind of chopper would you suggest to operate a dc motor in forward motoring and forward braking quadrant? Justify your suggestion. | CO2 | (07) |
|  | c) | Briefly explain about the armature voltage control method of a dc motor. | CO2 | (06) |
|  |  | UNIT-III |  |  |
| 5. | a) | A 3-phase, 50kW,400V, 960rpm, 50Hz, 6 pole squirrel cage induction motor has the following parameters referred to the stator: Rs=0.08Ω, Rr’=0.1Ω, Xs= Xr’=0.3Ω The stator winding is delta connected and consists of two sections connected in parallel. (i) Calculate starting torque as the ratio of maximum torque, if motor is started by star-delta starter. What is the maximum value of line current during starting? (ii) If the motor is started by connecting series reactors in the line, What should be the value of reactors so as to limit the line current to twice the rated value. | CO3 | (08) |
|  | b) | With neat diagrams, explain any one type of dynamic braking methods in induction motor. | CO3 | (06) |
|  | c) | Discuss about the operation of cyclo converter fed induction motors. |  | (06) |
| 6. | a) | With circuit diagram and waveforms, explain the operation of voltage source inverter fed Induction motor. | CO3 | (08) |
|  | b) | For variable frequency control of Induction motor, explain the following points: (i) For speeds below base speed v/f ratio is maintained constant, Why? (ii) For speeds above speed, the terminal voltage is maintained Constant, Why? | CO3 | (06) |
|  | c) | Explain that the rotor resistance starter allows fast start with less heating of Induction motor. | CO3 | (06) |
|  |  | UNIT- IV |  |  |
| 7. | a) | For a cylindrical rotor wound field motor, derive an expression for torque angle. | CO4 | (07) |
|  | b) | Describe the construction of a BLDC motor. | CO4 | (05) |
|  | c) | What are the different methods by which rotor position of a BLDC motor can be determined? Briefly explain. | CO4 | (06) |
|  | d) | What is the primary difference between out-runner type BLDC motors and in-runner type? | CO4 | (02) |
| 8. | a) | What are the different ways through which a synchronous motor can be braked? Explain about the method which is widely used. | CO4 | (06) |
|  | b) | Draw the circuit diagram of a voltage source inverter fed BLDC motor drive and explain its operation. | CO4 | (07) |
|  | c) | Discuss about the different modes of variable frequency control of synchronous motors. | CO4 | (07) |
|  |  | UNIT- V |  |  |
| 9. | a) | With a circuit diagram, explain the operation of a unipolar stepper motor drive. | CO5 | (07) |
|  | b) | Select the motor for driving an equipment which has the load torque curve as describe below: - For the first 10 seconds, the torque is constant and equal to 41 kg-m; - For the next 30 seconds, the torque drops linearly with time from 38 kg-m to 17 kg-m; - For the last 46 seconds, the torque is constant and equal to 8 kg-m. | CO5 | (06) |
|  | c) | Write short notes on the different classes of motor duty. | CO5 | (07) |
| 10. | a) | A motor has a thermal heating time constant of 45 minutes. When the motor runs continuously on full-load, its final temperature rise is 80⁰C. - What would be the temperature rise after one hour, if the motor runs continuously on full load? - If the temperature rise on 1 hour rating is 80⁰C, find the maximum steady-state temperature at this rating. - How long will the motor take for its temperature to rise from 50⁰C to 80⁰C, if it is working at its one hour ratting? | CO5 | (06) |
|  | b) | Explain half-step operation. Determine the stepping sequence for a 4/2 stepper motor, if the motor needs to run continuously. | CO5 | (07) |
|  | c) | Select appropriate type of motor, power modulator and suitable control techniques for a drive system with the following specifications: - The drive system need to handle active load torque - High torque at low speed is required - Control technique should be as simple as possible - Drive system should have the capability to braked electrically when required - The source available for powering the drive system is a 3-phase, 440V, 50Hz source. Also, draw an abstract block diagram of the drive system layout. | CO5 | (07) |

****************