RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKEUP EXAMINATIONS – FEBRUARY 2020

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Electric Drives | Max. Marks | : | 100 |
| Course Code | : | EEE11 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | By referring to the functional capabilities of a power modulator in an electric drive explain the different types of power modulators. | CO1 | (06) |
|  | b) | Provide the basic speed – torque sign conventions of an electric drive and hence, explain the four quadrant operation of motor driving a hoist load. | CO1 | (08) |
|  | c) | Provide relevant characteristics and explain the nature and classification of load torque on various applications. | CO1 | (06) |
| 2. | a) | Arrive at the fundamental torque equation and Derive an expression for the equivalent load torque and equivalent moment of inertia for loads with translational motion. | CO1 | (06) |
|  | b) | Define “Load Equalization”? Determine the expression required for the calculation of moment of inertia of the fly wheel required for load equalization. | CO1 | (08) |
|  | c) | A 3-phase, 150kW, 6-pole, 1500rpm wound rotor induction motor drives a load whose torque varies such that a torque of 4000N-m of 20sec duration is followed by a torque of 800 N-m of duration long enough for the motor to attain steady – state speed. Calculate the moment of inertia of motor – load fly wheel, if motor torque should not exceed twice the rated torque value. The moment of inertia of motor is 10kg-m2. Motor has a linear torque speed characteristics. Also calculate the moment of inertia of the flywheel. | CO1 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | Write a note on armature voltage control method of a dc motor. What are the different schemes to implement this for a dc motor drive? | CO2 | (08) |
|  | b) | Draw the diagram of a class E chopper fed DC motor. Figure out how the switches of the chopper need to be operated to obtain the operation in : i) Forward motoring and forward braking ii) Forward motoring and reverse motoring. | CO2 | (05) |
|  | c) | A 220 V, 1400 rpm, 50 A separately excited dc motor has an armature resistance of 0.4 Ω. The motor is fed from a circulating current dual converter bridge having the ac-input of 1-phase 230 V, 50 Hz. Calculate the firing angles of the converters for motor operation at: i) motoring operation at rated motor torque for 900 rpm ii) braking operation at rated motor torque for (-900 rpm) | CO2 | (07) |
| 4. | a) | A 500V, 45kW, 600 rpm, DC shunt motor has full-load efficiency of 90%. The field resistance is 200Ω and armature is 0.2Ω. The field current is maintained constant. Armature reaction and brush drop may be neglected. Calculate the rated armature current and hence find the speed under each of the condition in which the machine develops an electromagnetic torque equal to the rated value: i) Regenerative braking ii) Plugging with an external resistance of 5.5Ω inserted in the armature circuit iii) dynamic braking with external resistance of 2.6 Ω inserted | CO2 | (08) |
|  | b) | With a circuit diagram and waveforms, explain the working of a chopper controlled dynamic braking for a dc motor. Derive an expression for the effective value of the braking resistance. | CO2 | (06) |
|  | c) | With a relevant circuit diagram, discuss about the operation of a dc motor fed from a dual converter in circulating current mode. | CO2 | (06) |
|  |  | UNIT- III |  |  |
| 5. | a) | Explain the operation of star-delta starter with a neat diagram. | CO2 | (07) |
|  | b) | A 400V, star connected, 3-phase, 6-pole, 50Hz induction motor has the following parameters referred to the stator: Rs=Rr’=1 ohm, Xs=Xr’=2 ohm. For regenerative braking operation of this motor determine the maximum overhauling torque it can hold and range of speed for safe operation. | CO2 | (06) |
|  | c) | Explain the operation of ac voltage controller fed induction motor drive. Also explain the soft starting procedure. | CO2 | (07) |
| 6. | a) | Write short notes on VSI fed induction motor drives | CO2 | (07) |
|  | b) | With a neat diagram, explain the operation of part winding starter. | CO2 | (06) |
|  | c) | Derive speed-torque relation for an induction motor. Also, plot the characteristics. | CO2 | (07) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain the operation of a synchronous motor when fed form a fixed frequency supply. | CO4 | (10) |
|  | b) | With a neat circuit, explain the operation of a self-controlled synchronous motor drive employing load commutated thyristor inverter. | CO4 | (10) |
| 8. | a) | Describe the operation of a BLDC drive fed from a voltage source inverter. | CO4 | (10) |
|  | b) | Differentiate between true synchronous mode and self-control mode for variable frequency control of synchronous motor. Why a self-controlled synchronous motor is free from hunting oscillations. | CO4 | (10) |
|  |  | UNIT- V |  |  |
| 9. | a) | Draw the constructional diagram of a four-phase, 8/6-pole variable reluctance stepper motor and explain its operation. | CO3 | (07) |
|  | b) | An electric drive has the following specifications. i) The drive need to operate on 1-phase, 230 VAC, 50 Hz supply. ii) Speed range should be 10 % to 100 % of the rated speed. iii) Control algorithm should be simple. iv) The motor is should run at a constant speed regardless of torque variations. v) Feedback loop should enable the motor to run at the SET speed. Select appropriate motor, power electronic converter, control strategy to be used for this application. Also draw the abstract block diagram of the system layout you would suggest for this application. | CO4 | (07) |
|  | c) | A motor has a thermal heating time constant of 45 minutes. When the motor runs continuously on full-load, its final temperature rise is 80°C. i) What would be the temperature rise after one hour, if the motor runs continuously on full-load? ii)If the temperature ruse on one hour rating is 80°C, find the maximum steady-state temperature at this rating. iii) How long will the motor take for its temperature to rise from 50°C to 80°C, if it is working at its one hour rating? | CO4 | (06) |
| 10. | a) | Derive an expression for the mean temperature rise in a motor, after making necessary assumptions. | CO4 | (07) |
|  | b) | Mention three advantages and disadvantages of stepper motors. | CO3 | (06) |
|  | c) | A constant speed drive has the following duty cycle: (i) Load rising from 0 to 500kW for 4 minutes (ii)Uniform load of 400kW for 5 minutes (iii)Regenerative power of 300kW returned to the supply for 3 minutes (iv)Idle operation for 3 minutes Estimate the power rating for the motor. Assume losses to be proportional to the square of power. | CO4 | (05) |
|  | d) | Define ‘slew range’ in the context of stepper motors. | CO3 | (02) |

****************