RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

MAKE UP EXAMINATIONS –  APRIL 2021

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Electric Drives | Max. Marks | : | 100 |
| Course Code | : | EEE11 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Missing data, if any, may be suitably assumed.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | With help of neat block diagram, describe the elements and their functionalities of an electric drive. | CO1 | (08) |
|  | b) | Derive an expression for equivalent moment of inertia and equivalent torque. | CO1 | (08) |
|  | c) | An unsaturated DC shunt motor runs at its rated speed, when the rated voltage is applied on it, if the supply is reduced to 25%, what happens to the speed of the motor? | CO1 | (04) |
| 2. | a) | Write short notes on various components of load torque? Also explain the nature and classification of load torque on various applications. | CO1 | (08) |
|  | b) | With the help of a neat diagram and necessary notations, explain the multi quadrant operation of a crane hoist drive system. | CO1 | (08) |
|  | c) | Mention any four advantages of electric drives systems. | CO1 | (04) |
|  |  | UNIT – II |  |  |
| 3. | a) | Derive a relation between speed and torque of a separately excited dc motor and thereby plot its speed-torque characteristics. Write the description of the symbols used for deriving the expression. | CO2 | (07) |
|  | b) | An electric vehicle uses a dc motor. What type of electric braking technique (in addition to mechanical braking) will you suggest in this case? Explain your answer with reasons. | CO2 | (07) |
|  | c) | A 220V DC shunt motor takes 4.5A and runs at 970 rpm on no-load. The armature and field resistances are 0.2Ω and 220Ω, respectively. Calculate the speed, developed torque and shaft torque, when the motor input is 10 kW. | CO2 | (06) |
| 4. | a) | A 400V, 15kW, dc shunt motor takes 42A and runs at a speed of 1200rpm. The shunt field resistance is 200Ω. Assume that the load torque varies as the square of the speed. Neglect iron and friction losses, and also the armature reaction. Find the resistance to be connected in series with the armature to reduce the speed to 950rpm. | CO2 | (07) |
|  | b) | With appropriate diagrams, explain the operation of a single phase fully controlled rectifier fed dc motor. | CO2 | (07) |
|  | c) | Explain how regenerative braking, for separately excited dc motor, can be implemented using a chopper circuit. | CO2 | (06) |
|  |  | UNIT – III |  |  |
| 5. | a) | With circuit diagram and waveforms, explain the operation of voltage source inverter fed Induction motor drives. | CO3 | (07) |
|  | b) | Mention types of starters used for starting a 3phase induction motor. With a neat diagram, explain the operation of any one starter. | CO3 | (06) |
|  | c) | A 2.9kW, 400V, 50Hz, 4-pole, 1350rpm, delta connected squirrel-cage induction motor has following parameters referred to the stator: Rs=2Ω, Rr’=5 Ω, Xs=Xr’=5 Ω, Xm=80 Ω. Motor speed is controlled by stator voltage control. When driving a fan load it runs at rated speed at rated voltage. Calculate motor terminal voltage, current and torque at 1200rpm. | CO3 | (07) |
| 6. | a) | Discuss about induction motor stator voltage control method. Also, draw the speed torque characteristics of the induction motor during this speed control method. | CO3 | (06) |
|  | b) | Derive an equivalent circuit for the dc dynamic braking of an induction motor and explain why it is necessary to account for the saturation in the magnetic circuit. | CO3 | (06) |
|  | c) | A 1800V, 2000kW, 950 rpm, 50Hz, 6-pole, 3-phase squirrel cage induction motor has the following parameters referred to the stator: Rs=0.075Ω, Rr’=0.1Ω, Xs=0.45Ω, Xr’=0.55Ω. Stator winding in delta connected and consists of two sections connected in parallel. - Calculate starting torque and maximum torque as a ratio of rated torque, if the motor is started by star-delta switching. What is the maximum value of line current during starting? - Calculate the transformation ratio of an auto-transformer used to limit the maximum starting current to twice the rated value. Also find the value of the starting torque. | CO3 | (08) |
|  |  | UNIT – IV |  |  |
| 7. | a) | With relevance to brushless dc motors, discuss any one type of rotor position measurement techniques. | CO4 | (06) |
|  | b) | Discuss the starting operation of a synchronous motor when it is fed from a fixed frequency supply. | CO4 | (06) |
|  | c) | A 3phase, 5kW, 440V, 0.8 rated pf(lagging), 50Hz, 6pole, star connected synchronous motor has negligible stator winding resistance and synchronous reactance of 6Ω. Calculate: - Torque angle at full load - Pull out torque - Armature current and power factor at half the rated torque - Torque when operating at unity power factor and 150% of rated field current. | CO4 | (08) |
| 8. | a) | Explain the operation of a self-controlled synchronous motor drive employing load commutated thyristor inverter. | CO4 | (10) |
|  | b) | Mention three advantages of BLDC motors over conventional dc motors. | CO4 | (03) |
|  | c) | A 500kW, 3-phase, 3.3kV, 50Hz, 0.8 pf (lagging), 4-pole, star connected synchronous motor has the following parameters: Xs=15, Rs=0, rated field current=10A. Calculate: - Armature current and power factor at half the rated torque and rated field current. - Field current to get unit power factor at the rated torque. | CO4 | (07) |
|  |  | UNIT – V |  |  |
| 9. | a) | Explain the operation of single stack VR stepper motor with a neat diagram. | CO3 | (08) |
|  | b) | Calculate the stepping angle for a 3 phase 24 pole permanent magnet type stepper motor. | CO3 | (05) |
|  | c) | What are the assumptions made while performing heating & cooling calculation of an electric motor? | CO4 | (03) |
|  | d) | A constant speed drive has the following duty cycle. - Load rising from 0 to 400 KW ; 5 min - Uniform load of 500 KW; 5 min - Regenerative power of 400 KW returned to the supply; 4 min - Remains idle for ; 2 min Estimate power rating of the motor. Assume losses to be proportional to (power)2. | CO4 | (04) |
| 10. | a) | Write short notes on different classes of motor duty. | CO3 | (05) |
|  | b) | Explain the principle of operation of bipolar drives for stepper motors. | CO3 | (07) |
|  | c) | A rolling mill driven by thyristor converter-fed dc motor operates on a speed reversing duty cycle. Motor field current is maintained constant at the rated value. Moment of inertia referred to the motor shaft is 10,000 kg-m2. Duty cycle consists of the following intervals: - Rolling at full speed( 200 rpm) and at a constant torque of 25,000N-m for 10 sec. - No load operation for 1 sec at full speed. - Speed reversal from 200 to – 200 rpm in 5 sec. - No load operation for 1 sec at full speed. - Rolling at full speed and at a torque of 20,000 N-m for 15 sec. - No load operation at full speed for 1 sec. - Speed reversal from - 200 to 200 rpm in 5 sec. - No load operation at full speed for 1 sec. Determine the torque and power of the motor. | CO4 | (08) |

****************