RAMAIAH

INSTITUTE OF TECHNOLOGY

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS – JANUARY 2018

| Course & Branch | : | B.E. : Electrical & Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Electrical Drives | Max. Marks | : | 100 |
| Subject Code | : | EE706 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Make necessary assumptions wherever necessary.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Using four quadrant diagrams, analyze the multiquadrant operation of Washing machine in the wash cycle. State the speed-torque conventions used and assumptions made. | CO1 | (07) |
|  | b) | A weight of 500 kg is being lifted up at a uniform speed of 1.5m/s by a winch driven by a motor running at a speed of 1000rpm. The moment of inertia of the motor and winch are 0.5 and 0.3 kg-m2 respectively. Calculate the motor and equivalent moment of inertia referred to the motor shaft. In the absence of weight, motor develops a torque of 100N-m when running at 1000rpm. | CO1 | (05) |
|  | c) | Discuss about the different components of load torques. Also draw the speed-torque characteristics of each component. | CO1 | (08) |
| 2. | a) | List out the steady state and transient state operation requirements of an electric drive. | CO1 | (04) |
|  | b) | Derive an expression for equivalent moment of inertia and equivalent torque for drive systems with loads having rotational motion. | CO1 | (06) |
|  | c) | A motor drives two loads. One has rotational motion. It is coupled to the motor through a reduction gear with gear ratio of 0.1 and efficiency of 90%. The load has a moment of inertia of 10 kg-m2 and torque of 10 N-m. Other load has translational motion and consists of 1000 kg weight to be lifted up at a uniform speed of 1.5m/s. Coupling between this load and the motor has an efficiency of 85%. Motor has inertia of 0.2 kg-m2 and runs at a constant speed of 1420 rpm. Determine the equivalent inertia referred to the motor shaft and power developed by the motor. | CO1 | (06) |
|  | d) | Draw the speed-torque characteristics of the following loads: - High speed hoist - Air compressor type loads. | CO1 | (04) |
|  |  | UNIT- II |  |  |
| 3. | a) | Which kind of chopper would you suggest to operate a dc motor in forward motoring and forward braking quadrant? Explain the working of this chopper with relevant waveforms. | CO2 | (08) |
|  | b) | A 230V, 870 rpm, 100A separately excited DC motor has an armature resistance of 0.05Ω. It is driving an overhauling load whose torque is 400 Nm. Determine the speed at which motor can hold the load by regenerative braking. | CO2 | (06) |
|  | c) | With a relevant circuit diagram, discuss about the operation of a dc motor fed from a dual converter in circulating current mode. | CO2 | (06) |
| 4. | a) | With appropriate diagrams, discuss about plugging method of braking a dc motors. | CO2 | (07) |
|  | b) | A DC shunt motor is connected to constant voltage mains and drives a load torque which is independent of speed. Prove that if Eb>V/2, increasing the air-gap flux per pole decreases the speed of the motor. | CO2 | (06) |
|  | c) | A 220V, 1500rpm, 50A separately excited motor has armature resistance of 0.5Ω. It is fed from a circulating current dual converter with AC source voltage of 230V, 50Hz. Determine converter firing angles for the following operating points: - motoring operation at rated motor torque and 1000 rpm - braking operation at rated motor torque and 1000 rpm. | CO2 | (07) |
|  |  | UNIT- III |  |  |
| 5. | a) | With a neat circuit diagram, explain the principle of operation of a star-delta starter. | CO3 | (07) |
|  | b) | A 2.8kW, 400V, 50Hz, 4-pole, 1370rpm, delta connected squirrel-cage induction motor has following parameters referred to the stator: Rs=2ohm, Rr’=5ohm, Xs=Xr’=5ohm, Xm=80ohm. Motor speed is controlled by stator voltage control. When driving a fan load it runs at rated speed at rated voltage. Calculate motor speed, current and torque for the terminal voltage of 300V. | CO3 | (08) |
|  | c) | Compare the dynamic braking method of controlling a dc motor and an induction motor. | CO3 | (05) |
| 6. | a) | A 2.8kW, 400V, 50Hz, 4-pole, 1370rpm, delta connected squirrel-cage induction motor has following parameters referred to the stator: Rs=2ohm, Rr’=5ohm, Xs=Xr’=5ohm, Xm=80ohm. Motor speed is controlled by stator voltage control. When driving a fan load it runs at rated speed at rated voltage. Calculate motor terminal voltage, current and torque at 1200rpm. | CO3 | (06) |
|  | b) | Explain the principle of operation of a part-winding starter, with a neat circuit diagram. | CO3 | (07) |
|  | c) | Explain the operation of ac voltage controller fed induction motor. | CO3 | (07) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Explain the operation of self-controlled synchronous motor drive employing load commutated thyristor inverter by giving emphasis to the commutation process of thyristors, constant margin angle control and the pulsed mode operation of the inverter. | CO4 | (10) |
|  | b) | Discuss about the principles behind measurement of BLDC motor using hall effect sensor. | CO4 | (05) |
|  | c) | Why is a synchronous motor called a commutator less dc motor? Briefly explain. | CO4 | (05) |
| 8. | a) | Derive an expression for torque of a cylindrical rotor wound field synchronous motor. Also draw the speed torque characteristics when the supply frequency is held fixed. | CO4 | (07) |
|  | b) | With a neat diagram, explain the structure of a brushless dc motor. | CO4 | (07) |
|  | c) | Discuss about the starting operation of a synchronous motor when it is fed from a fixed frequency supply. | CO4 | (06) |
|  |  | UNIT- V |  |  |
| 9. | a) | Mention three advantages and disadvantages of stepper motors. | CO4 | (06) |
|  | b) | A motor operates on a periodic duty cycle in which it is clutched to its load for 20 minutes and declutched to run on no-load for 10 minutes. Maximum temperature rise is 60°C. Heating and cooling time constants are 50 and 70 minutes respectively. When load is declutched continuously the temperature rise is 10°C. Determine, (i) Maximum temperature during the duty cycle (ii) Temperature when the load is clutched continuously. | CO5 | (06) |
|  | c) | A constant speed drive has the following duty cycle: - Load rising from 0 to 400kW : 5 minutes - Uniform load of 500kW : 5 minutes - Regenerative power of 400kW returned to the supply : 4 minutes - Remains idle for :2 minutes Estimate the power rating of the motor required. Assume losses to be proportional to (power)2 | CO5 | (05) |
|  | d) | Identify the class of duty for the drive systems in the following applications: - Washing machine - CPU Cooling fan - Battery operated wall-clock. | CO5 | (03) |
| 10. | a) | Explain the half-step operation of a stepper motor with suitable aid. | CO4 | (06) |
|  | b) | Derive an expression for the mean temperature rise in a motor, after making necessary assumptions. | CO5 | (07) |
|  | c) | Draw the constructional diagram of a four-phase, 8/6-pole variable reluctance stepper motor and explain its operation. | CO4 | (07) |

****************