RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS –  JANUARY 2019

| Course & Branch | : | B.E: Electrical & Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Electric Drives | Max. Marks | : | 100 |
| Subject Code | : | EEE11/EE706 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT-I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | A motor drives two loads. One has rotational motion. It is coupled to the motor through a reduction gear with gear ratio of 0.1 and efficiency of 90%. The load has a moment of inertia of 10 kg-m2 and torque of 10 N-m. Other load has translational motion and consists of 1000 kg weight to be lifted up at an uniform speed of 1.5 m/s. Coupling between this load and the motor has an efficiency of 85%. Motor has an inertia of 0.2 kg-m2 and runs at a constant speed of 1420 rpm. Determine the equivalent inertia referred to the motor shaft and power developed by the motor. | CO1 | (06) |
|  | b) | Using four quadrant diagram, analyze the multiquadrant operation of the drive system in an electric scoter. State the conventions used for analysis. | CO1 | (07) |
|  | c) | Discuss elaborately about the different types of power modulators used in an electric drive system. | CO1 | (07) |
| 2. | a) | Mention the different types of friction torque and write short notes on them. | CO1 | (06) |
|  | b) | Differentiate between active load torque and passive load torque. | CO1 | (04) |
|  | c) | Write the fundamental torque equation for a motor-load system. Also, mention how the dynamic components in the torque equation affect the performance of the motor. | CO1 | (05) |
|  | d) | Discuss the status of dc and ac drives to the present day context. | CO1 | (05) |
|  |  | UNIT-II |  |  |
| 3. | a) | A 230V, 1000 rpm, 30A separately excited motor has an armature resistance of 0.7Ω. Motor is controlled in regenerative braking by a chopper operating at 800Hz from a dc source of 230V. Assuming continuous conduction, - Calculate duty ratio of chopper for rated torque and speed of 800 rpm - What will be the motor speed for duty ratio of 0.6 and rated motor torque? | CO2 | (07) |
|  | b) | With a relevant circuit diagram, discuss about the operation of a dc motor fed from a dual converter in circulating current mode. | CO2 | (06) |
|  | c) | Briefly explain the various braking methods in a DC motor. | CO2 | (07) |
| 4. | a) | Draw the circuit diagram of a class E chopper fed DC motor. Figure out how the switches of the chopper need to be operated to obtain the operation in Forward motoring and Reverse braking.Sketch the voltage and current waveforms. | CO2 | (07) |
|  | b) | Which type of braking method is considered to improve the overall efficiency of a drive system? Explain your answer with reasons. | CO2 | (06) |
|  | c) | A 220V, 1500rpm, 11.6A separately excited dc motor is controlled by a single phase fully controlled rectifier with an AC source voltage of 230V, 50Hz. Enough filter inductance is added to ensure continuous conduction for any torque greater than 25% of rated torque. Armature resistance is 2Ω. - What should be the value of firing angle to get the rated torque at 1000 rpm? - Calculate the motor speed at the rated torque and firing angle of 160° for regenerative braking in the second quadrant. | CO2 | (07) |
|  |  | UNIT-III |  |  |
| 5. | a) | “V/f method of induction motor speed control method is better that stator voltage control and stator frequency control method” Mention whether the above statement is TRUE or FALSE. Give reasons for your choice, in detail, with appropriate facts. | CO3 | (07) |
|  | b) | A star connected squirrel cage induction motor has the following ratings and parameters: 400V, 50Hz, 4-pole, 1370rpm, Rs=2Ω, Rr’=3ohm, Xs=Xr’=3.5Ω . Motor is controlled by a voltage source inverter. Calculate: - Speed for a frequency of 30Hz and 80% of full-load torque - Frequency for a speed of 1000rpm and full-load torque - Torque for a frequency of 40Hz and speed of 1100rpm | CO3 | (07) |
|  | c) | Discuss about the operation of cycloconverter fed induction motors. | CO3 | (06) |
| 6. | a) | A 400V, star connected, 3-phase, 6-pole, 50Hz induction motor has the following parameters referred to the stator: Rs=Rr’=1 Ω, Xs=Xr’=2 Ω. For regenerative braking operation of this motor determine: - Maximum overhauling torque it can hold and range of speed for safe operation - Speed at which it will hold an overhauling load with a torque of 100 N-m | CO3 | (07) |
|  | b) | Explain stator frequency control method used for controlling the speed of induction motors. Also draw the speed-torque characteristics of the motor while stator frequency control is employed. | CO3 | (07) |
|  | c) | With a neat diagram, explain the operation of part winding starter used for starting three phase induction motor. | CO3 | (06) |
|  |  | UNIT-IV |  |  |
| 7. | a) | Explain the operation of self-controlled synchronous motor drive employing load commutated thyristor inverter. | CO4 | (10) |
|  | b) | Explain the constructional features of brushless motors with suitable aid. | CO4 | (06) |
|  | c) | Discuss the advantages of BLDC motors over conventional dc motors | CO4 | (04) |
| 8. | a) | Describe the operation of a brushless dc drive fed from a voltage source inverter. | CO4 | (08) |
|  | b) | Differentiate between true synchronous mode and self-control mode for variable frequency control of synchronous motor. Why a self-controlled synchronous motor is free from hunting oscillations? | CO4 | (08) |
|  | c) | Briefly discuss about the pull-in process of synchronous motor | CO4 | (04) |
|  |  | UNIT-V |  |  |
| 9. | a) | A 4-phase, 4/2-pole variable reluctance stepper motor is to be controlled for half-step operation. Find out the switching sequence and draw the switching table, to rotate the rotor continuously. Also find the step-angle that can be obtained in half-step operation. | CO4 | (07) |
|  | b) | Write a note on drive circuits for stepper motors. Draw a unipolar drive circuit and bipolar drive circuit. | CO4 | (08) |
|  | c) | A constant speed drive has the following duty cycle: - Load rising from 0 to 400kW : 5 minutes - Uniform load of 500kW : 5 minutes - Regenerative power of 400kW returned to the supply : 4 minutes - Remains idle for :2 minutes Estimate the power rating of the motor required. Assume losses to be proportional to (power)2 | CO5 | (05) |
| 10. | a) | Identify the class of duty for the drive systems in the following applications: - Washing machine - Air-conditioner - Electric car - Battery operated wall-clock - Mixer grinder | CO5 | (05) |
|  | b) | An electric drive has the following specifications. - The system should be capable of being operated on a 220 V DC source - The drive should have the facility to be braked to a set speed as quickly as possible - The drive need to have constant speed operation under normal operating conditions - After the user has SET the speed of the motor, it should run at the same speed regardless of load variations Select appropriate motor, power electronic converter, control strategy to be used for this application. Also draw the abstract block diagram of the system layout you would suggest for this application. | CO5 | (07) |
|  | c) | Draw the constructional diagram of a four-phase, 8/6-pole variable reluctance stepper motor and explain its operation. | CO4 | (08) |

****************