RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS –  JANUARY 2020

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Electric Drives | Max. Marks | : | 100 |
| Course Code | : | EEE11 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Write the fundamental torque equation of a drive system and write short notes on each of the parameters. | CO1 | (07) |
|  | b) | Using the four-quadrant diagram, analyze the multi-quadrant operation of a drive used for a railway gate. State the speed-torque conventions used and assumptions made. | CO1 | (07) |
|  | c) | Write a short note on the power modulators used in electric drives. | CO1 | (06) |
| 2. | a) | Discuss about the different components of load torques. Draw the speed-torque characteristics of the components of friction torque. | CO1 | (06) |
|  | b) | A horizontal conveyor belt moving at a uniform velocity of 1m/s, transports load at the rate of 50,000 kg/hour. The belt is 180m long and is driven by a 960 rpm motor. i) Determine the equivalent rotational inertia at the motor shaft. ii) Calculate the required braking torque at motor torque to stop the belt at uniform rate in 10 sec? | CO1 | (06) |
|  | c) | Draw the block diagram of an electrical drive system and briefly discuss the functions of each block. | CO1 | (08) |
|  |  | UNIT- II |  |  |
| 3. | a) | A 110V, 6A, 1500 rpm separately excited dc motor has the armature and field resistances of 0.5Ω and 400Ω respectively. It drives a load whose torque is constant at rated motor torque. Calculate motor speed if the source voltage drops to 90V. | CO2 | (06) |
|  | b) | Discuss about the performance of a dc separately excited dc motor when its armature voltage is varied. | CO2 | (06) |
|  | c) | Suggest a suitable chopper circuit to control the armature of a separately excited dc motor, so that the dc drive system can be operated in forward braking and forward motoring quadrants. Also explain the operation of the chopper during forward braking and forward motoring modes. | CO4 | (08) |
| 4. | a) | A 230V, 500rpm, 100A, separately excited dc motor has an armature resistance of 0.1Ω. The motor is driving, under rated conditions, a load whose torque is constant and independent of speed. The speeds below the rated speed are obtained with armature voltage control and the speeds above the rated speed are obtained by field control. - Calculate the motor terminal voltage when the sped is 400rpm. - By what amount should the flux be reduced to get a motor speed of 800 rpm? | CO2 | (08) |
|  | b) | Discuss about dynamic braking in dc motors. | CO2 | (06) |
|  | c) | A 200V, 875rpm, 150A, separately excited motor is controlled by a 1-phase fully controlled rectifier with an ac source voltage of 220V, 50Hz. The rectifier operates in continuous current conduction mode. The armature resistance is 0.06Ω. Find, - The firing angle for rated motor torque and 750rpm. - The firing angle for rated motor torque and -500rpm. | CO2 | (06) |
|  |  | UNIT- III |  |  |
| 5. | a) | Explain the following starting schemes of a 3-phase Induction Motor: - Reactor Starter - Rotor resistance Starting. | CO3 | (08) |
|  | b) | Explain the operation of a 3- phase induction motor when supplied with non-sinusoidal supply voltage. Comment on how the current harmonics can be minimized. | CO3 | (06) |
|  | c) | A 400V, 50Hz, 3-phase, 6 pole, and Y-connected squirrel cage induction motor has the following parameters referred to stator: Rs= Rr’= 1, Xs=Xr’=2. For regenerative braking operation of this motor, determine: i. Maximum overhauling torque it can hold and range of speed for safe operation. ii. Speed at which it can hold an overhauling load with a torque of 100N-m. | CO3 | (06) |
| 6. | a) | Explain the operation of a Voltage Source Inverter (VSI) controlled induction motor drive. Draw the voltage waveforms of stepped wave inverter and PWM inverter. | CO3 | (06) |
|  | b) | Explain in detail with relevant figures and waveforms how a static Scherbius drive is used to control speed of a wound rotor induction motor below synchronous speed. | CO3 | (08) |
|  | c) | With neat diagrams, explain self-excited braking and zero sequence braking. | CO3 | (06) |
|  |  | UNIT- IV |  |  |
| 7. | a) | For a cylindrical rotor wound field motor, derive an expression for torque. Also sketch the speed-torque characteristics. | CO3 | (06) |
|  | b) | A 500 kW, 3phase, 3.3 kV, 50 Hz, 0.8 lagging power factor, 4 pole star-connected synchronous motor has following parameters: Xs=15Ω, Rs=0, rated field current is 10A. Calculate the following for regenerative braking operation: i)Braking torque and field current for machine operation at the rated current and unity power factor. ii) Armature current and power factor for 500kW output at 15 A field current. | CO3 | (08) |
|  | c) | Briefly discuss about the pull-in process of synchronous motors. | CO3 | (06) |
| 8. | a) | With diagrams, describe the constructional feature of different types of rotors in BLDC motors. | CO3 | (06) |
|  | b) | A 3phase, 5kW, 440V, 0.8 rated pf(lagging), 50Hz, 6pole, star connected synchronous motor has negligible stator winding resistance and synchronous reactance of 6Ω. Calculate: i) Torque angle at full load ii)Pull out torque iii)Armature current and power factor at half the rated torque iv)Torque when operating at unity power factor and 150% of rated field current. | CO3 | (08) |
|  | c) | For a salient pole wound field synchronous motor, derive an expression for torque. Also draw the phasor diagram. | CO3 | (06) |
|  |  | UNIT- V |  |  |
| 9. | a) | Draw the circuit diagram of a bipolar stepper motor drive circuit and explain its operation. | CO3 | (06) |
|  | b) | Derive an expression for temperature rise in a motor. | CO1 | (07) |
|  | c) | A 4/2 stepper motor need to be rotated by 135° from its current position whenever required. Suggest a suitable stepper motor drive circuit, Also, suggest suitable stepping operation and decide its stepping sequence. Justify your suggestions. | CO4 | (07) |
| 10. | a) | A constant speed drive has the following duty cycle: - Load rising from 0 to 500kW for 4 minutes - Uniform load of 400kW for five minutes - Regenerative power of 300kW returned to the supply for three minutes - Idle operation for three minutes Assuming the losses to be proportional to the square of power, estimate the power rating for the motor. Also, suggest a suitable motor and power modulator for the drive system. | CO4 | (07) |
|  | b) | Draw the constructional diagram of a four phase, 4/2 pole, variable stepper motor. | CO3 | (07) |
|  | c) | Draw the torque-stepping rate characteristics of a stepper motor and write short notes about it. | CO3 | (06) |

****************