RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SEMESTER END EXAMINATIONS  JANUARY – FEBRUARY 2021

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Electric Drives | Max. Marks | : | 100 |
| Course Code | : | EEE11 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit. - Missing data, if any, may be suitably assumed.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Using four quadrant diagrams, analyze the multi-quadrant operation of an electric bike drive system. Mention the speed-torque conventions used. | CO1 | (08) |
|  | b) | List the components of load torque and briefly explain about each component. Also, depict the characteristic curve of each component. | CO1 | (08) |
|  | c) | Draw the load (speed-torque) characteristics in the following drive systems: - Electric Train ii) Air compressor. | CO1 | (04) |
| 2 | a) | A weight of 500kg is being lifted up at a uniform speed of 1.5m/s by a winch driven by a motor running at a speed of 1000rpm. The moment of inertia of the motor and winch are 0.5 and 0.3 kg-m2 respectively. Calculate the equivalent torque and equivalent moment of inertia referred to the motor shaft. In the absence of weight, motor develops a torque of 100N-m when running at 1000rpm. | CO1 | (06) |
|  | b) | Discuss about the choices, a drives design engineer would consider while designing an electric drive system. | CO1 | (06) |
|  | c) | Identify the type of load torque (with relation to electric drive system) in the following systems: - Crane-hoist - Elevator - Electric Car - Mixer grinder | CO1 | (04) |
|  | d) | Mention any four advantages of electric drives. | CO1 | (04) |
|  |  | UNIT – II |  |  |
| 3. | a) | Explain with example, the transition of the operating point of a DC drive system, when the speed-control setting is changed. | CO2 | (05) |
|  | b) | Describe the working of dual converter control of separately excited DC motor in circulating current mode. | CO2 | (07) |
|  | c) | A 200 V, 875 rpm, 150 A, separately excited DC motor as an armature resistance of 0.06 Ω , It is fed from single phase fully controlled converter with AC source voltage of 220 V, 50 Hz, Assuming continuous conduction calculate (i) Firing angle for rated torque and 750 rpm, (ii) Firing angle for rated torque and (-500) rpm, (iii) Motor speed for α=1600 and rated torque. | CO2 | (08) |
| 4. | a) | Explain the operation of three phase half controlled rectifier fed separately excited DC motor in continuous conduction mode. | CO2 | (08) |
|  | b) | With the help of circuit diagram, explain motoring, regenerative and dynamic braking operation of controlled rectifier fed dc motor. | CO2 | (12) |
|  |  | UNIT – III |  |  |
| 5. | a) | A 400V, star connected, 3-phase, 3-pole, 50Hz induction motor has the following parameters referred to the stator: Rs=Rr’=1Ω, Xs=Xr’=2Ω. For regenerative braking operation of this motor determine: - maximum overhauling torque it can hold and range of speed for safe operation - speed at which it will hold an overhauling load with a torque of 100 N-m | CO3 | (10) |
|  | b) | Mention the different types of induction motor starters. Explain the working of any type of starter with relevant diagrams. | CO3 | (10) |
| 6. | a) | What kind of speed control method will you suggest for a crane hoist drive system with induction motor? Justify your answer. | CO3 | (06) |
|  | b) | Using the simplified per-phase equivalent circuit, derive the speed-torque expression for an induction motor. Also, draw the speed-torque characteristics. | CO3 | (07) |
|  | c) | With appropriate diagrams, discuss about stator frequency control of induction motors. | CO3 | (07) |
|  |  | UNIT – IV |  |  |
| 7. | a) | Explain the operation of synchronous motor in fixed frequency supply. | CO4 | (10) |
|  | b) | Explain the operation of three phase brushless DC motor, discuss the converter configuration for the same, and Mention its merits and demerits. | CO4 | (10) |
| 8. | a) | With a neat circuit, explain the working of self-controlled synchronous motor drive employing load commutated thyristror. | CO4 | (10) |
|  | b) | Explain with the help of block diagram, the close loop BLDCM speed control in PWM feedback mode. | CO4 | (10) |
|  |  | UNIT – V |  |  |
| 9. | a) | Draw the speed torque characteristics of a stepper motor and explain the different operating regions. | CO4 | (07) |
|  | b) | Describe the constructional features of a stepper motor with suitable diagram. | CO4 | (05) |
|  | c) | A drive consisting of semiconductor converter fed dc motor, runs according to the following periodic duty cycle: - Acceleration from standstill to 1000rpm in 10sec at uniform acceleration. - Running at 1000rpm and 800Nm torque for 8sec. - Braking from 1000rpm to standstill in 10sec at uniform deceleration. - Determine torque and power rating of the machine. Assume forced cooling and constant field current. J=100kgm2. | CO5 | (08) |
| 10. | a) | Mention the different classes of motor duty. Briefly explain about any three classes. | CO5 | (08) |
|  | b) | Explain the operation of unipolar drive circuit used in stepper motor control using circuit diagram. | CO4 | (06) |
|  | c) | A motor operates on a periodic duty cycle in which it is clutched to its load for 20 minutes and declutched to run on no-load for 10 minutes. Maximum temperature rise is 60°C. Heating and cooling time constants are 50 and 70 minutes respectively. When load is declutched continuously the temperature rise is 10°C. Determine: - Maximum temperature during the duty cycle. - Temperature when the load is clutched continuously. | CO5 | (06) |

****************