| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)

Accredited by NBA & NAAC with ‘A+’ Grade

SEMESTER END EXAMINATIONS – MARCH 2022

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Electric Drives | Max. Marks | : | 100 |
| Course Code | : | EEE11(O) | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Using four quadrant diagram, analyze the multi-quadrant operation of an electric drive system in an electric truck. State the conventions used and assumptions made. | CO1 | (10) |
|  | b) | Obtain an expression for equivalent moment of inertia and equivalent torque for drive systems with loads having translational motion. | CO1 | (06) |
|  | c) | Draw the load (speed-torque) characteristics in the following loads: - Traction loads ii) Air compressor loads. | CO1 | (04) |
| 2. | a) | Using four quadrant diagrams, analyze the four quadrant operation of an crane hoist drive. State assumptions made and conventions used. | CO1 | (10) |
|  | b) | A motor drives two loads. One has rotational motion. It is coupled to the motor through a reduction gear with gear ratio of 0.1 and efficiency of 95%. The load has a moment of inertia of 10 kg-m2 and torque of 10 N-m. Other load has translational motion and consists of 1000 kg weight to be lifted up at an uniform speed of 1.5 m/s. Coupling between this load and the motor has an efficiency of 85%. Motor has an inertia of 0.2 kg-m2 and runs at a constant speed of 1500 rpm. Determine the equivalent inertia referred to the motor shaft and power developed by the motor. | CO1 | (06) |
|  | c) | Mention any four advantages of electric drives. | CO1 | (04) |
|  |  | UNIT – II |  |  |
| 3. | a) | A 400V, 15kW, dc shunt motor takes 42A and runs at a speed of 1200rpm. The shunt field resistance is 200Ω. Assume that the load torque varies as the square of the speed. Neglect iron and friction losses, and also the armature reaction. Find the resistance to be connected in series with the armature to reduce the speed to 950rpm. | CO2 | (07) |
|  | b) | Discuss the operation of a single phase half controlled converter controlled separately excited dc motor with necessary diagrams. | CO2 | (07) |
|  | c) | A dc shunt motor need to be braked. If the overall efficiency of the drive system needs to be improved, what kind of braking technique will you suggest? Justify your answer. | CO2 | (06) |
| 4. | a) | A 220V, 900rpm, 100A, separately excited motor is controlled by a 1-phase fully controlled rectifier with an ac source voltage of 230V, 50Hz. The rectifier operates in continuous current conduction mode. The armature resistance is 0.05Ω. Find the firing angle for rated motor torque and 750rpm. | CO2 | (05) |
|  | b) | Explain armature voltage control method of controlling dc motors with appropriate diagrams. | CO2 | (08) |
|  | c) | Explain the operation of a dc motor when it is braked by plugging braking method. | CO2 | (07) |
|  |  | UNIT – III |  |  |
| 5. | a) | A 2100V, 2500kW, 750rpm, 50Hz, 8-pole, 3-phase squirrel cage induction motor has the following parameters referred to the stator: Rs=0.075Ω, Rr’=0.1Ω, Xs=0.45Ω, Xr’=0.55Ω. Stator winding in delta connected and consists of two sections connected in parallel. What will be the value of maximum line current and torque during starting, if the part winding method is employed for starting? | CO3 | (06) |
|  | b) | Using the simplified per-phase equivalent circuit, derive the speed-torque expression for an induction motor. Also, draw the speed-torque characteristics. | CO3 | (07) |
|  | c) | If the maximum torque capability is to be retained at low speeds, which type of speed control method should be used? Elaborate your answer with reasons. | CO3 | (07) |
| 6. | a) | Explain the operation of ac voltage controller fed induction motor drive. | CO3 | (07) |
|  | b) | A 400V, star connected, 3-phase, 3-pole, 50Hz induction motor has the following parameters referred to the stator: Rs=Rr’=1Ω, Xs=Xr’=2Ω. For regenerative braking operation of this motor determine: - maximum overhauling torque it can hold and range of speed for safe operation. - speed at which it will hold an overhauling load with a torque of 100 N-m. | CO3 | (06) |
|  | c) | Explain the operation of star-delta starter with a diagram. | CO3 | (07) |
|  |  | UNIT – IV |  |  |
| 7. | a) | With a neat diagram, describe the constructional features of brushless DC motor. | CO4 | (08) |
|  | b) | With suitable aid, discuss how a synchronous motor can be used for correcting power factor. | CO4 | (06) |
|  | c) | Discuss about the starting operation of a synchronous motor when it is fed from a fixed frequency supply. | CO4 | (06) |
| 8. | a) | Discuss the operation of a self-controlled synchronous motor drive employing load commutated thyristor inverter. | CO4 | (10) |
|  | b) | Explain any one type of rotor position measurement techniques (with relevance to BLDC motors). | CO4 | (06) |
|  | c) | Why is a synchronous motor called a commutator less dc motor? Briefly explain. | CO4 | (04) |
|  |  | UNIT – V |  |  |
| 9. | a) | Discuss about the different classes of motor duty with relevant load waveforms. | CO5 | (08) |
|  | b) | Select the motor for driving an equipment which has the load torque curve as describe below: - For the first 10 seconds, the torque is constant and equal to 41 kg-m; - For the next 30 seconds, the torque drops linearly with time from 38 kg-m to 17 kg-m; - For the last 46 seconds, the torque is constant and equal to 8 kg-m. | CO4 | (06) |
|  | c) | Draw the torque-stepping rate characteristics of a stepper motor and write short notes about it. | CO5 | (06) |
| 10. | a) | An electric drive has the following specifications: - The system should be capable of being operated on a 3-phase, 440 VAC, 50 Hz supply. - The drive need to have variable speed operation with speeds varying from 25 % to 100 % of the rated speed. - The overall system cost should be low. - After the user has SET the speed, the motor should run at the same speed regardless of load variations. - The system should have soft-starting ability. Select appropriate motor, power electronic converter, control strategy to be used for this application. Also draw the abstract block diagram of the system layout you would suggest for this application. | CO4 | (06) |
|  | b) | By using the thermal model of motor, derive an equation for expressing mean temperature rise in a motor. | CO5 | (07) |
|  | c) | Explain the operation of unipolar drive circuit. | CO5 | (07) |

****************