| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)

Accredited by NBA & NAAC with ‘A+’ Grade

SEMESTER END EXAMINATIONS – MARCH 2022

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Electric Drives | Max. Marks | : | 100 |
| Course Code | : | EEE741 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | A drive has the following parameters: J=10kgm2, T=100-0.1N Nm, Passive load torque Tl=0.05N Nm, where N is the speed in rpm. Initially, the drive is operating in steady state. Now it is to be reversed. For this, motor characteristic is changed to T=-100-0.1N Nm. Calculate the time of reversal. | CO1 | (06) |
|  | b) | Mention the factors which decide the choice of electrical drive for a given application. | CO1 | (06) |
|  | c) | Using a block diagram, state and explain an essential parts of an electric drive system. | CO1 | (08) |
| 2. | a) | A horizontal conveyer belt moving at a uniform speed of 1.5m/s transports material at the rate of 100tonnes/hr. Belt is 180m long and driven by a motor at 1200rpm. - Determine the load inertia referred to the motor shaft. - Calculate the torque that motor should develop to accelerate the belt from standstill to full speed in 8sec. Moment of inertia of the motor is 0.1kgm2. | CO1 | (07) |
|  | b) | Arrive at the fundamental torque equation and Derive an expression for the equivalent load torque and equivalent moment of inertia for loads with rotational motion. | CO1 | (07) |
|  | c) | Write short notes on the different components of load torque and sketch its speed-torque characteristics. | CO1 | (06) |
|  |  | UNIT – II |  |  |
| 3. | a) | Explain dynamic braking in separately excited motor using its characteristic curves. | CO2 | (04) |
|  | b) | Describe motoring and regenerative braking of separately excited DC motor by chopper control. | CO2 | (08) |
|  | c) | A 220V, 1200rpm, 8A separately excited motor with armature resistance of 0.6Ω is fed from a 1Ф fully controlled rectifier. An ac source voltage is 230V, 50Hz. Determine the converter firing angle when the motor is operating in: - Motoring quadrant at rated torque and 900rpm. - Braking quadrant at rated torque and 900rpm. | CO2 | (08) |
| 4. | a) | Differentiate between circulating and non-circulating current control method for the dual converter. | CO2 | (06) |
|  | b) | Discuss the operation of a single phase fully controlled converter fed separately excited dc motor with necessary diagrams. | CO2 | (08) |
|  | c) | A 220V, 960rpm, 200A separately excited dc motor has an armature resistance of 0.02Ω. Motor is operated in dynamic braking with a chopper control with a braking resistance of 1.8Ω. The source has a voltage of 230V. - Calculate duty ratio of chopper for a motor speed of 650rpm and braking torque of twice the rated value. - What will be the motor speed for a duty ratio of 0.65 and motor torque equal to twice its rated torque. | CO2 | (06) |
|  |  | UNIT – III |  |  |
| 5. | a) | Write a short note on rotor resistance starter used for starting a three phase induction motor. | CO3 | (04) |
|  | b) | With circuit diagram and waveforms, explain the operation of voltage source inverter fed Induction motor. | CO3 | (08) |
|  | c) | A 2200V, 2600kW, 735rpm, 50Hz, 8pole, 3-phase squirrel-cage induction motor has the following parameters referred to the stator: Rs=0.075Ω, Rr’=0.1Ω, Xs=0.45Ω, Xr’=0.55Ω. Stator winding is delta connected and consists of two sections connected in parallel. - What will the value of maximum line current and torque during starting, if the part-winding method of starting is employed. - If motor is started by connecting series reactors in line, what should be the value of reactors so as to limit line current to twice the rated value. | CO3 | (08) |
| 6. | a) | Write a short note on zero sequence braking of an induction motor | CO3 | (04) |
|  | b) | For variable frequency control of induction motor, explain the following points. - For speeds below base speed, (V/f) ratio is maintained constant, why? - For speeds above base speed , the terminal voltage is maintained constant, why? | CO3 | (08) |
|  | c) | A 400V, star connected, 3phase, 6pole, 50Hz induction motor has the following parameters referred to the stator: Rs=Rr’=1Ω, Xs=Xr’=1.8Ω. For regenerative braking operation of this motor, determine: - Speed at which it will hold an overhauling load with a torque of 100Nm. - Maximum overhauling torque it can hold and range of speed for safe operation. | CO3 | (08) |
|  |  | UNIT – IV |  |  |
| 7. | a) | How is true synchronous mode operation different from self-controlled operation? Explain. | CO4 | (04) |
|  | b) | With diagrams, describe the constructional feature of different types of rotors in BLDC motors. | CO4 | (08) |
|  | c) | A 3phase, 6 MW, 11kV, 0.9 power factor (leading), 50Hz, 6pole, star connected synchronous motor has negligible stator winding resistance and synchronous reactance of 9Ω. Rated field current is 50A. Machine is controlled by variable frequency control at constant (V/f) ratio up to the base speed and at constant V above base speed. Determine: - Torque and field current for the rated armature current, 750rpm and 0.8 leading power factor. - Armature current and power factor for regenerative braking power output of 4.5MW at 750rpm and rated field current. | CO4 | (08) |
| 8. | a) | Discuss the advantages of BLDC motors over conventional dc motors. | CO4 | (04) |
|  | b) | Describe the operation of a brushless dc drive fed from a voltage source inverter. | CO4 | (08) |
|  | c) | A 3phase, 5kW, 440V, 0.8 rated pf(lagging), 50Hz, 6pole, star connected synchronous motor has negligible stator winding resistance and synchronous reactance of 6Ω. Calculate: - Torque angle at full load - Pull out torque - Armature current and power factor at half the rated torque. | CO4 | (08) |
|  |  | UNIT – V |  |  |
| 9. | a) | A Stepper motor is called as synchronous motor. Why? | CO4 | (02) |
|  | b) | Discuss the constructional features of single stack variable reluctance motor. | CO4 | (08) |
|  | c) | Explain the working of a bipolar driving circuit with relevant diagram. | CO5 | (10) |
| 10. | a) | Explain the working of unipolar stepper motor driving circuit with an appropriate sketch. | CO4 | (08) |
|  | b) | Mention any two advantages and disadvantages of stepper motor. | CO4 | (04) |
|  | c) | Explain the torque versus stepping rate characteristics of a stepper motor. | CO4 | (08) |

****************