RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER EXAMINATIONS –  JULY 2019

| Course & Branch | : | B.E: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Subject | : | Electric Drives | Max. Marks | : | 100 |
| Subject Code | : | EEE11/EE706 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | How do you define active and passive load torque? What are the differences between the two? | CO1 | (06) |
|  | b) | Using the quadrant diagrams, analyze the multiquadrant operation of an Escalator system. CLEARLY state the speed-torque conventions used and assumptions made. | CO1 | (07) |
|  | c) | The motor is required to drive the take-up roll on a plastic strip line. The mandrel on which the strip is wound is 15cm in diameter and strip builds up to a roll of 25 cm in diameter. Strip tension is maintained constant at 1000 N. The strip moves at a uniform speed of 25 m/s. The motor is coupled to a mandrel by a reduction gear of ratio 0.5. The gears have an approximate efficiency of 87% at all speeds. Determine the speed and power rating of the motor required for this application. | CO1 | (07) |
| 2. | a) | Discuss about the different components of load torques. Also draw the speed-torque characteristics of each component. | CO1 | (08) |
|  | b) | A horizontal conveyor belt moving at a uniform speed of 1.2m/s transports material at the rate of 100 tonnes/hr. Belt is 200m long and driven by a motor at 1200 rpm. - Determine the load inertia referred to the motor shaft. - Calculate the torque that motor should develop to accelerate the belt from standstill to full speed in 8 sec. moment of inertia of the motor is 0.1 Kg-m2 | CO1 | (06) |
|  | c) | Write short notes on the present status of AC and DC drives | CO1 | (06) |
|  |  | UNIT-II |  |  |
| 3. | a) | Explain the reverse current braking with diagrams of dc separately excited motor. Why is it considered as an inefficient method of braking? | CO2 | (07) |
|  | b) | A 220V, 1500rpm, 10A separately excited dc motor is fed from a single- phase fully-controlled rectifier with an ac source voltage of 230V, 50Hz. Ra=2Ω. Conduction can be assumed to be continuous. Calculate firing angles for (i) Half the rated motor torque and 500rpm. (ii) Rated motor torque and (-1000)rpm. | CO2 | (06) |
|  | c) | Which kind of chopper would you suggest to operate a dc motor in forward motoring and forward braking quadrant? Justify your suggestion. | CO2 | (07) |
| 4. | a) | Plot the speed - torque characteristics of a separately excited dc motor. Also derive a relation between speed and torque for this dc motor. | CO2 | (07) |
|  | b) | Describe the relative merits and demerits of four quadrant dc drives employing non circulating and circulating current dual converters. | CO2 | (06) |
|  | c) | A 230V, 1200rpm, 15A separately excited motor has an armature resistance of 1.2Ω. Motor is operated under dynamic braking with chopper control. Braking resistance has a value of 20Ω. Calculate (i) Duty ratio of chopper for speed of 1000rpm and braking torque equal to 1.5 times rated motor torque. (ii) Motor speed for duty ratio of 0.5 and motor torque equal to its rated torque. | CO2 | (07) |
|  |  | UNIT- III |  |  |
| 5. | a) | Discuss about induction motor stator voltage control method. Also draw the speed torque characteristics of the induction motor during this speed control method. | CO3 | (06) |
|  | b) | A 2.8kW, 400V, 50Hz, 4-pole, 1370rpm, delta connected squirrel-cage induction motor has following parameters referred to the stator: Rs=2Ω, Rr’=5 Ω, Xs=Xr’=5 Ω, Xm=80 Ω. Motor speed is controlled by stator voltage control. When driving a fan load it runs at rated speed at rated voltage. Calculate motor terminal voltage, current and torque at 1200rpm. | CO3 | (07) |
|  | c) | With a neat diagram, explain the operation of star-delta starter used for starting a three phase induction motor. | CO3 | (07) |
| 6. | a) | A 1800V, 2000kW, 750 rpm, 50Hz, 8-pole, 3-phase squirrel cage induction motor has the following parameters referred to the stator: Rs=0.075Ω, Rr’=0.1Ω, Xs=0.45Ω, Xr’=0.55Ω. Stator winding in delta connected and consists of two sections connected in parallel - Calculate starting torque and maximum torque as a ratio of rated torque, if the motor is started by star-delta switching. What is the maximum value of line current during starting? - Calculate the transformation ratio of an auto-transformer used to limit the maximum starting current to twice the rated value. Also find the value of the starting torque. | CO3 | (08) |
|  | b) | Draw the speed torque characteristics of an induction motor when it is controlled by rotor resistance control method. | CO3 | (07) |
|  | c) | Write a brief note on braking methods in three phase induction motors | CO3 | (05) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Elaborately discuss about the operation of self-controlled synchronous motor drive employing load commutated thyristor inverter. | CO4 | (14) |
|  | b) | Describe the construction of a brushless dc motor using a diagram. | CO4 | (06) |
| 8. | a) | With relevance to brushless dc motors, discuss about any type of rotor position measurement techniques. | CO4 | (05) |
|  | b) | A 500kW, 3-phase, 3.3kV, 50Hz, 0.8 pf (lagging), 4-pole, star connected synchronous motor has the following parameters: X2=15 ohm, Rs= 0, rated field current = 10A. Calculate, (i)Armature current and power factor at half the rated torque and rated field current (ii)Field current to get unit power factor at the rated torque. | CO4 | (07) |
|  | c) | With relevance to synchronous motors, explain true synchronous mode operation and self-controlled mode operation. | CO4 | (08) |
|  |  | UNIT- V |  |  |
| 9. | a) | Explain the working of bipolar stepper motor driving circuit with an appropriate sketch. | CO4 | (06) |
|  | b) | Mention the different classes of motor duty. Briefly explain about any three classes. | CO5 | (07) |
|  | c) | Define slew range. | CO4 | (02) |
|  | d)) | A constant speed drive has the following duty cycle: - Load rising from 0 to 500kW for 4 minutes - Uniform load of 400kW for 5 minutes - Regenerative power of 300kW returned to the supply for 3 minutes - Idle operation for 3 minutes Estimate the power rating for the motor. Assume losses to be proportional to the square of power. | CO5 | (05) |
| 10. | a) | Draw the speed torque characteristics of a stepper motor and explain the different operating regions. | CO4 | (07) |
|  | b) | An electric drive has the following specifications: - The system should be capable of being operated on a 1-phase, 230 VAC, 50 Hz supply - The drive needs to have variable speed operation with speeds varying from 0 to 35% of rated speed - The frequency may be varied, when required, from zero to rated - At low speeds, the speed variation should be smooth - The distortion caused by power converters on the power sources should be minimum. Suggest a suitable motor and power electronic converter to meet the above requirement. Also draw the abstract block diagram of the system layout as per your suggestion. | CO5 | (07) |
|  | c) | A motor operates on a periodic duty cycle in which it is clutched to its load for 20 minutes and declutched to run on no-load for 10 minutes. Maximum temperature rise is 60°C. Heating and cooling time constants are 50 and 70 minutes respectively. When load is declutched continuously the temperature rise is 10°C. Determine, - Maximum temperature during the duty cycle - Temperature when the load is clutched continuously. | CO5 | (06) |

****************