RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

EXAMINATIONS SEPTEMBER /OCTOBER 2020

SUPPLEMENTARY SEMESTER / GRADE IMPROVEMENT/ RE –REGISTERED CANDIDATES

| Program | : | B.E.: Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Electric Drives | Max. Marks | : | 100 |
| Course Code | : | EEE11 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer one full question from each unit.

|  |  | UNIT- I |  |  |
| --- | --- | --- | --- | --- |
| 1. | a) | Derive the expression for finding the equivalent moment of inertia and equivalent torque components of a motor-load system (loads with rotational motion), all referred to motor shaft. | CO1 | (06) |
|  | b) | Discuss about the structure of an electric drive system with suitable diagram. | CO1 | (08) |
|  | c) | What are the factors which decide the choice of electrical drive for a particular application? Briefly explain. | CO1 | (06) |
| 2. | a) | A motor drives two loads. One has rotational motion. It is coupled to the motor through a reduction gear with a=0.1 and efficiency of 90%. The load has a moment of inertia of 10kg-m2 and a torque of 10 N-m. Other load has translational motions and consists of 1000 kg weight to be lifted up at an uniform speed of 1.5m/s. Coupling between this load and the motor has an efficiency of 85%. Motor has an inertia of 0.2 kg-m2 and runs at a constant speed of 1420 rpm. Determine equivalent inertia referred to the motor shaft and power developed by the motor. | CO1 | (06) |
|  | b) | Discuss about the different components of load torques. Also draw the speed-torque characteristics of each component. | CO1 | (08) |
|  | c) | Draw the load torque characteristics for the following loads and also comment on the characteristics: - Traction loads. - Constant power loads. - Air compressor type loads. | CO1 | (06) |
|  |  | UNIT- II |  |  |
| 3. | a) | With neat circuit diagrams and speed – torque characteristics explain the dynamic braking and plugging operations of a dc separately excited motor. | CO2 | (07) |
|  | b) | For a separately excited dc motor controlled by a 1 – phase fully controlled rectifier explain the discontinuous conduction operation with relevant circuit and waveforms. Obtain the boundary conditions and draw the speed – torque indicating the CCM & DCM. | CO2 | (08) |
|  | c) | A 220 V, 750 rpm, 150 A, separately excited dc motor has an armature resistance of 0.1. It is fed from a single phase fully controlled rectifier with an AC voltage source of 230 V, 50 Hz. Assume continuous conduction and calculate: - Firing angle for rated motor torque & 500rpm - Motor speed for  = 140o and rated torque. | CO2 | (05) |
| 4. | a) | Define continuous conduction mode and discontinuous mode for a dc separately excited motor controlled by a half controlled rectifier. Obtain the expressions for the motor operation in both the modes of operation. | CO2 | (08) |
|  | b) | With a neat circuit explain the operation of a dc drive in all four quadrants when fed by a single phase dual converter and operated in non – circulating current principle. State how speed reversal is obtained. | CO2 | (06) |
|  | c) | With the help of neat diagram and waveforms, explain the principle of operation of regenerative braking for a dc separately excited motor using chopper control. Derive the necessary equations and plot the speed – torque characteristics. | CO2 | (06) |
|  |  | UNIT- III |  |  |
| 5. | a) | Explain the dc dynamic braking in a three phase induction motor with circuit diagrams and necessary waveforms. | CO2 | (07) |
|  | b) | A star connected squirrel cage induction motor has the following ratings and parameters: 400V, 50Hz, 4-pole, 1370rpm, Rs=2Ω, Rr’=3Ω, Xs=Xr’=3.5Ω. Motor is controlled by a voltage source inverter. For regenerative braking operation of this motor, determine approximate values of : i) Speed for the frequency of 30Hz and 80% of full-load torque ii) Frequency for a speed of 1000rpm and full load torque iii) Torque for a frequency of 40Hz and speed of 1300rpm. | CO2 | (07) |
|  | c) | What are the different types of starters available for starting an induction motors? Explain the working of any type of starter with diagrams. | CO2 | (06) |
| 6. | a) | A 2200V, 2600kW, 735 rpm, 50Hz, 8-pole, 3-phase squirrel cage induction motor has the following parameters referred to the stator: Rs=0.075Ω, Rr’=0.1Ω, Xs=0.45Ω, Xr’=0.55Ω Stator winding is delta connected and consists of two sections in parallel. i) Calculate the transformation ratio of an auto-transformer so as to limit the maximum starting current to twice the rated value. What is the value of starting torque? ii) If the motor is started by connecting series reactors in line, what should be the value of reactors so as to limit line current to twice the rated value? | CO2 | (08) |
|  | b) | Draw the circuit diagram of a voltage source inverter fed induction motor. Sketch the waveform for any line voltage when the inverter is operated in the stepped wave and PWM modes. | CO2 | (05) |
|  | c) | If the maximum torque capability is to be retained at low speeds, which type of speed control method should be used? Elaborate your answer with reasons. | CO2 | (07) |
|  |  | UNIT- IV |  |  |
| 7. | a) | Discuss about the operation of self-controlled synchronous motor drive employing load commutated thyristor inverter. | CO3 | (08) |
|  | b) | A 3phase, 5kW, 440V, 0.8 rated pf(lagging), 50Hz, 6pole, star connected synchronous motor has negligible stator winding resistance and synchronous reactance of 6Ω. The motor is operated under regenerative braking with its terminals connected to a bus having rated motor voltage. i) Field current is adjusted so that motor operated at rated current and unity power factor. Calculate braking torque, torque angle and field current. ii) Calculate p.f and armature current when the machine develops braking torque equal to rated torque and field current is 1.2A. | CO3 | (07) |
|  | c) | Briefly discuss about the important features and applications of BLDC motors. | CO3 | (05) |
| 8. | a) | A 500 kW, 3phase, 3.3 kV, 50 Hz, 0.8 lagging power factor, 4 pole star-connected synchronous motor has following parameters: Xs=15Ω, Rs=0, rated field current is 10A. Calculate: i) Armature current and power factor at half the rated torque and rated field current. ii) Field current to get unity power factor at the rated torque. | CO3 | (08) |
|  | b) | Suggest a suitable power electronic converter topology, in the form of a circuit diagram, for driving a 3-phase BLDC motor. Sketch the waveforms for induced voltage, phase current and torque. | CO3 | (08) |
|  | c) | Briefly discuss about the pull-in process of synchronous motors. | CO3 | (04) |
|  |  | UNIT- V |  |  |
| 9. | a) | With the help of circuit diagrams, explain the working of unipolar and bipolar drive circuits used for stepper motors. | CO3 | (07) |
|  | b) | A motor operates on a periodic duty cycle in which it is clutched to its load for 20 minutes and declutched to run on no-load for 10 minutes. Maximum temperature rise is 60°C. Heating and cooling time constants are 50 and 70 minutes respectively. When load is declutched continuously the temperature rise is 10°C. Determine, (i) Maximum temperature during the duty cycle (ii)Temperature when the load is clutched continuously. | CO4 | (07) |
|  | c) | An electric drive has the following specifications. i) The system should be capable of being operated on a 3-phase, 440 VAC, 50 Hz supply ii) The drive need to have variable speed operation with speeds varying from 0 to 40% of rated speed iii) The speed variation should be smooth at low speeds iv) The supply distortion caused by power converters have to be minimal Select appropriate motor and power electronic converter that can be used for this application. Also draw the abstract block diagram of the system layout you would suggest for this application. | CO4 | (06) |
| 10. | a) | A 4-phase, 4/2-pole variable reluctance stepper motor is to be controlled for full-step operation. Find out the switching sequence and draw the switching table, to rotate the rotor continuously. Also find the step-angle that can be obtained in full-step operation. | CO3 | (07) |
|  | b) | A constant speed drive has the following duty cycle: i) Load rising from 0 to 400kW : 5 minutes ii) Uniform load of 500kW : 5 minutes iii) Regenerative power of 400kW returned to the supply : 4 minutes iv) Remains idle for :2 minutes Estimate the power rating of the motor required. Assume losses to be proportional to (power)2 | CO4 | (05) |
|  | c) | Mention the different classes of motor duty. Briefly explain about any three classes. | CO4 | (08) |

****************