RAMAIAH

Institute of Technology

| USN | 1 | M | S |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

(Autonomous Institute, Affiliated to VTU)

(Approved by AICTE, New Delhi & Govt. of Karnataka)                       Accredited by NBA & NAAC with ‘A’ Grade

SUPPLEMENTARY SEMESTER  EXAMINATIONS – SEPTEMBER 2021

| Program | : | B.E. : Electrical and Electronics Engineering | Semester | : | VII |
| --- | --- | --- | --- | --- | --- |
| Course Name | : | Electric Drives | Max. Marks | : | 100 |
| Course Code | : | EEE11 | Duration | : | 3 Hrs |

Instructions to the Candidates:

- Answer any five full questions. - Missing data, if any, may be suitably assumed.

| 1. | a) | A motor drives two loads. One has rotational motion. It is coupled to the motor through a reduction gear with gear ratio of 0.1 and efficiency of 90%. The load has a moment of inertia of 10kg-m2 and torque of 15N-m. Other load has translational motion and consists of 1500kg weight to be lifted up at a uniform speed of 1.8m/s. Coupling between this load and the motor has an efficiency of87%. Motor has an inertia of 0.6kg-m2 and runs at a constant speed of 1450 rpm. Determine the equivalent inertia referred to the motor shaft and power developed by the motor. | CO1 | (06) |
| --- | --- | --- | --- | --- |
|  | b) | Using the four-quadrant diagram, analyze the multi-quadrant operation of a drive used for a railway gate. State the speed-torque conventions used and assumptions made. | CO1 | (07) |
|  | c) | Briefly discuss the various components of load torque. Draw the speed-torque characteristics of the following loads: - Traction Load - High speed Hoist - Coiler drive. | CO1 | (07) |
| 2. | a) | What are the advantages of an electric drive? Mention the factors on which choice of an electric drive depends. | CO1 | (06) |
|  | b) | A drive has following parameters: J=10kg-m2, T=15+0.05N, Nm and Tl=5+0.06N, Nm, where N is the speed in rpm. Initially the drive is working in steady-state. Now the drive is braked by electrical braking. Torque of the motor in braking is given by T=-10-0.04N, Nm. Calculate time taken by the drive to stop. | CO1 | (07) |
|  | c) | A motor is required to drive the take-up roll on a plastic strip line. The mandrel on which the strip is wound is 20cm in diameter and strips builds up to a roll 15cm in diameter. Strip tension is maintained constant at 500N. the strip moves at a uniform speed of 25m/s. the motor is coupled to a mandrel by a reduction gear with a=0.3. the gear has an approximate efficiency of 85% at all speeds. Determine the speed and power rating of the motor required for this application. | CO1 | (07) |
| 3. | a) | With a relevant circuit diagram, discuss about the operation of a dc motor fed from a dual converter in circulating current mode. | CO2 | (05) |
|  | b) | Explain the multi-quadrant operation of dc separately excited motor fed from chopper. | CO2 | (06) |
|  | c) | A 230V, 960rpm, 20A separately excited dc motor has armature resistance and inductance of 1.2 and 50mH respectively. Motor is controlled by a single–phase half controlled rectifier with source voltage of 230V, 50Hz. Calculate and plot no-load speeds and speeds and developed torques on the boundary between continuous and discontinuous conduction for  = 45 and 90. | CO2 | (09) |
| 4. | a) | Examine regenerative braking in separately excited motor using its characteristic curves. | CO2 | (05) |
|  | b) | A dc motor needs to be operated in 1st and 4th quadrant. If the available source is dc in nature, what topology of converter will you suggest to meet the above mentioned requirement? Explain the working of this converter with relevant waveforms. | CO2 | (08) |
|  | c) | A 230V, 1200rpm, 15A separately excited motor has an armature resistance of 1.2Ω. Motor is operated under dynamic braking with chopper control. Braking resistance has a value of 20Ω. Calculate: - Duty ratio of chopper for speed of 1000rpm and braking torque equal to 1.5 times rated motor torque. - Motor speed for duty ratio of 0.5 and motor torque equal to its rated torque. | CO2 | (07) |
| 5. | a) | Explain the induction motor operation when V/F ratio is held constant, Also derive an expression for Maximum Torque. | CO2 | (06) |
|  | b) | Explain the principle of speed control of induction motor by stator voltage control method. | CO2 | (06) |
|  | c) | Explain the operation of static Scherbius drive using cycloconverter. Also explain different modes of operation of static Scherbius drive. | CO2 | (08) |
| 6. | a) | Draw the torque speed characteristics of polyphase induction motor. Also explain the following regions: Motoring , generating, Braking. | CO2 | (07) |
|  | b) | With relevant diagrams, explain rotor resistance control of induction motor. | CO2 | (06) |
|  | c) | A three phase 11.2 KW, 1750 rpm, 460 V, 60 Hz, 4 pole, star connected induction motor has following parameters: Rs=0, Rr1=0.38Ω, Xs=1.14Ω, Xr1=1.71Ω, Xm=33.2Ω. The motor is controlled by varying the supply frequency if the brake down requirement is 35 Nm. Calculate - Supply frequency - Speed at maximum torque. | CO2 | (07) |
| 7. | a) | Explain the operation of a self-controlled synchronous motor drive employing load commutated thyristor inverter. | CO4 | (07) |
|  | b) | With the help of a diagram, describe the constructional features of a BLDC motor. | CO4 | (08) |
|  | c) | Explain any type of synchronous motor braking methods. | CO4 | (05) |
| 8. | a) | Discuss about pulsed mode operation in synchronous motors. | CO4 | (06) |
|  | b) | Explain the constructional features of brushless motors with suitable aid. | CO4 | (08) |
|  | c) | Explain any one type of rotor position measurement techniques (with relevance to BLDC motors). | CO4 | (06) |
| 9. | a) | Draw the circuit diagram of a unipolar stepper motor drive and explain its operation. | CO4 | (06) |
|  | b) | An electric drive that needs to be implemented has the following requirements: - The motor should be capable of being operated from a110VDC source. - Braking should be achieved at the minimum possible time. - Under steady state, the motor should run at near constant speed. - The variation between the SET speed and actual speed need to be as minimum as possible. - The filter used to reduce converter harmonics should be small in size. Select appropriate motor, power electronic converter, control strategy to be used for this application. Also draw the abstract block diagram of the system. | CO5 | (07) |
|  | c) | Write short notes on different classes of motor duty. | CO5 | (07) |
| 10. | a) | Draw the circuit diagram of a bipolar stepper motor drive and explain its operation. | CO4 | (07) |
|  | b) | Using the heat balance equation, derive an expression for the temperature rise in a motor. State the symbols used in the derivation. | CO5 | (06) |
|  | c) | An electric drive that needs to be implemented has the following requirements: - The system should be capable of being operated from a 1-phase, 230 V, 50 Hz AC supply - The drive need to have variable speed operation with speeds varying from 40% to 100% of the rated speed - The converter and the control strategy used should be as simple as possible to keep the overall system cost low - After the user has SET the speed, the motor should run at the same speed regardless of load variations - The system should have soft-starting ability Select appropriate motor, power electronic converter, control strategy to be used for this application. Also draw the abstract block diagram of the system. | CO5 | (07) |

****************